| # NSN Connector FRS (Functional Requirement Specifications) Auto-Import connector |  |
| --- | --- |
|  | Draft version |

Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft | Indrasan | 12-11-2024 |

Table of Contents

Introduction	4

1	EnodeB	5

1.1	Physical configuration	5

1.1.1	Cabinet	5

1.1.2	Shelf	5

1.1.3	Boards	6

1.1.3.1	Boards Slot Indexes:	7

1.2	Logical configuration:	8

1.1	Global Parameters	8

1.2	Networking Parameters	8

1.3	Radio Parameters	11

1.4	Cell Logical Parameters	14

2	BSC	20

2.1	Physical configuration	20

2.1.1	Cabinet	20

2.1.2	Shelf	21

2.1.3	Boards	21

3	BTS:	23

3.1	Physical configuration	23

3.1.1	Cabinet	23

3.1.2	Shelf	24

3.1.3	Boards	24

# Introduction

- This FRS is intended for NSN Nodes extraction.

- It describes how to parse the sites, nodes and their correspondent physical configuration.

- The dump files provided are just one .xml contains the configuration of all nodes.

- The folder location to be specified later.

# EnodeB

- Each class="HW" which includes MRBTS in attribute “distname” represents one eNodeB and in same class NEType will be LNBTS.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="HW" ,distName attribute - The distName attribute context is as follows: class="HW" version="HW1.0" distName="PLMN-PLMN/MRBTS-747370/HW-1" id="8329600"> - E.g distName="PLMN-PLMN/MRBTS-747370" NE ID = MRBTS-747370 |
| NE Name | [Mandatory] | - Same as NE ID |
| Site id | [Mandatory] | Follow the site matching sheet |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = eNodeB |
| Model |  | - = “systemTitle” attribute under the correspondent Class=” HW” - E.g. <p name="systemTitle">FlexiZoneMicro System</p> |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration for MRBTS (eNodeB)

### Cabinet

- Each class="HW" which includes MRBTS in attribute “distname” represents one eNodeB and in same class NEType will be LNBTS. Here HW-1 represents one cabinet.

<managedObject class="HW" version="HW1.0" distName="PLMN-PLMN/MRBTS-747370/HW-1" id="8329600">

<p name="NEType">LNBTS</p>

<p name="locationName">Jeddah</p>

<p name="operationalState">enabled</p>

<p name="systemTitle">FlexiZoneMicro System</p>

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = 1 to be assigned by default |
| Model | [Mandatory] | - = systemtitle <p name="systemTitle">FlexiZoneMicro System</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |

### Shelf

- The Shelf should be same as cabinet.

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = Cabinet id |
| Model | [Mandatory] | Same as Cabinet Model |
| RU Index | [Mandatory] | - =[(Shelf id -1)*2]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =3 |
| Description | [Optional] | - Null |

### Boards

- Each class="Module" and class=" SUBMODULE” represents one board

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of correspondent shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Start with 1. Increased with +1 for every new Board. |
| Board Type and Model | [Mandatory] | - = <p name="userLabel"> attribute under the same class - E.g. <p name="userLabel">FSM</p> |
| Part Number | [Optional] | - = “identificationCode” attribute - E.g. <p name="identificationCode">473161A</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class <p name="serialNumber">EB184810990</p> |
| Issue Number | [Optional] | - Version in same class - <p name="version">202</p> |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

# SRAN Physical configuration

A node is classified as SRAN in class=’HW’ under attributes distName="PLMN-PLMN/MRBTS-627397/HW-1" which does not contains NEType=WBTS Or LNBTS.

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - It is extracted from class=’HW’ under attributes below. - E.g class="HW" version="EQMHW19A_1904_002" distName="PLMN-PLMN/MRBTS-627397/HW-1" id="2104243"> NE ID= MRBTS-627397 |
| NE Name | [Mandatory] | - Same as NE ID |
| Site id | [Mandatory] | - Follow the site matching sheet |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = SRAN |
| Model |  | - = Nokia SRAN |
| Manufacturer | [Optional] | - = Nokia |

### Cabinet for class ‘HW’

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="HW" version="EQMHW19A_1904_002" distName="PLMN-PLMN/MRBTS-627397/HW-1" id="2104243">

- the node ID for this cabinet is: MRBTS-627397

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = 1 to be assigned by default |
| Model | [Mandatory] | - = Flexi Base Station (by default) |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |

### Shelf for class SBTS

- Each class="HW" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="HW" version="EQMHW19A_1904_002" distName="PLMN-PLMN/MRBTS-627397/HW-1"
 the node ID for this cabinet is: MRBTS-627397

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - =1 |
| Shelf index | [Mandatory] | - =from class= “HW” from the distName="PLMN-PLMN/MRBTS-627397/HW-1 - Shelf index= 1 - E.g distName="PLMN-PLMN/SBTS-627397/HW-1 - Shelf index =1 |
| Shelf Id | [Mandatory] | - = shelf index |
| Model | [Mandatory] | - = Flexi Base Station (by default) |
| RU Index | [Mandatory] | - =[(Shelf id -1)*3]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =4 |

### Boards for class “INVUNIT”

- Each class equal to “INVUNIT” represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="INVUNIT" version="EQMHW19A_1904_002" distName="PLMN-PLMN/MRBTS-627397/HW-1/INVUNIT-1"

- Node ID= MRBTS-627397

- e.g.

HW-1 indicates that Cabinet=1 & Shelf=1.

INVUNIT=11 indicates that slot no. 11

Index on slot is start with 0 & increment by 1 for same cabinet, shelf & slot.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Index | [Mandatory] | - the index of the related shelf |
| Index on slot | [Mandatory] | = Default index on slot = 0 & increment by 1 for same cabinet, shelf & slot. |
| Slot index | [Mandatory] | - To be taken from class=INVUNIT, numerical value in front of INVUNIT. |
| Board Type | [Mandatory] | - = “inventoryUnitType” attribute under the same class - E.g. |
| Part Number | [Optional] | - = " vendorUnitTypeNumber" attribute under the same class - E.g. |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. |
| Issue Number | [Optional] | - = “version" attribute under the same class - E.g. |

# BSC

- Each class="HW" under attributes distName="PLMN-PLMN/BSC-404822/HW-1" represent one BSC

- class="HW" version="1.0" distName="PLMN-PLMN/BSC-404822/HW-1"

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="HW", distName attribute distName="PLMN-PLMN/BSC-404822/HW-1" - The distName attribute context is as follows: "PLMN-PLMN/BSC-ID" - E.g distName="PLMN-PLMN/BSC-404822" ID = 404822 |
| NE Name | [Mandatory] | Need to take from attribute: <p name="systemTitle"> <p name="systemTitle"> |
| Site id | [Mandatory] | Follow the site matching sheet |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = BSC |
| Model | [Mandatory] | - Default “Nokia BSC” |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration for BSC

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/BSC-404822/HW-1/CABINET-11"
 the node ID for this cabinet is: BSC-404822.

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute take Cabinet ID and subtract 10 from it - E.g. distName="PLMN-PLMN/BSC-404822/HW-1/CABINET-11" cabinet ID= 1 |
| Model | [Mandatory] | - = "cabinetType" attribute - E.g. <p name="cabinetType">GCBV</p> |
| Manufacturer | [Mandatory] | - Nokia |

### Shelf

- Each Class="CARTRIDGE" in attributes represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g

- the node ID for this cabinet is: BSC-404822

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf. For above shelf ID=1 Note: We will consider 3rd digit in the value of Cartridge. |
| Model | [Mandatory] | - = “cartridgeType" attribute for the same class <p name="cartridgeType">BCNC</p> |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-404822/HW-1/CABINET-11/CARTRIDGE-11100/UNIT-BCNMB_B_1_1_SHM-1"

- Node ID= 404822

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - Class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-404822/HW-1/CABINET-11/CARTRIDGE-11100/UNIT-BCNMB_B_1_1_SHM-1"

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-404822/HW-1/CABINET-11/CARTRIDGE-11100/UNIT-BFAN_A_1_1_FAN-1"

  - these two boards belong to the same shelf (CARTRIDGE-11100)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">1</p> slot index =1 |
| Board Type and Model | [Mandatory] | - = “unitTypeExpected” attribute under the same class - E.g. <p name="unitTypeExpected">BCNMB_B</p> |
| Manufacturer |  | - = “vendorName” attribute under the same class - E.g - <p name="vendorName">Nokia</p> |
| Part Number | [Optional] | - = Null |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = Null |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

# RNC

RNC node will be find in class “HW” as below given in snap: RNC will be “RNC-752” under attributes distName="PLMN-PLMN/RNC-752/HW-1"

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - = distName attribute within each line containing class="HW" - E.g.: distName="PLMN-PLMN/RNC-752" Node ID = RNC-752 |
| NE Name | [Mandatory] | Same as NE ID |
| Site id | [Mandatory] | - Follow the site matching sheet |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = RNC |
| Model |  | - = Nokia RNC |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/RNC-752/HW-1/CABINET-cabinet-1"

the cabinet ID for this cabinet is: 1

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = Cabinet ID from “distName” attribute take Cabinet ID - E.g. distName distName="PLMN-PLMN/RNC-752/HW-1/CABINET-cabinet-1" cabinet ID= 1 |
| Model | [Mandatory] | - Default: Nokia RNC Cabinet |
| Manufacturer | [Mandatory] | Nokia |

### Shelf

- Each class="Rack" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="RACK" version="1.0" distName="PLMN-PLMN/RNC-752/HW-1/CABINET-cabinet-1/RACK-c1"
 the node ID for this shelf is: RNC-752

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - =Rack from “distName” attribute take Rack ID - E.g. PLMN-PLMN/RNC-752/HW-1/CABINET-cabinet-1/RACK-c1 Shelf ID=1 |
| Model | [Mandatory] | - Nokia RNC Shelf |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |
| Serial number | [Optional] | - NA |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/RNC-752/HW-1/CABINET-cabinet-1/RACK-c1/UNIT-AMC-1" id="3307609">

- <p name="identificationCode">MIC-5401-0000E</p>

- <p name="serialNumber">AT142400153</p>

- <p name="unitType">HDSAM-A</p>

- <p name="vendorName">ADVANTEC</p>

- <p name="version">C110598.B3A</p>

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/RNC-752/HW-1/CABINET-cabinet-1/RACK-c1/UNIT-AMC-1" id="3307609">  the board belongs to cabinet 1 and shelf 1.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 (For one shelf it will start from 0 & incremented by 1 till max. 9) |
| Slot index | [Mandatory] | - Slot index will be taken from the numeric value of Unit under attributes: |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class - E.g. |
| Part Number | [Optional] | - = “identificationCode” attribute under the same class |
| Serial Number | [Optional] | - = “serialNumber” attribute under the same class |

# BTS:

- Each class="HW" where BCF coming under attributes “distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1 “represents one BTS node.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="HW", distName attribute - = distName attribute - E.g distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1" ID= PLMN-PLMN/BSC-395041/BCF-158 |
| NE Name | [Mandatory] | - Same as NE ID |
| Site id | [Mandatory] | - Follow the site matching sheet |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = BTS |
| Model |  | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. <managedObject class="HW" version="1.0" distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1" id="305512"> <p name="operationalState">1</p> <p name="systemTitle">Flexi Multiradio</p> <p name="vendorName">Nokia</p> - The Model for “PLMN-PLMN/BSC-395041/BCF-158” = Flexi Multiradio - If no systemTitle found keep Default model as “Flexi Multiradio” |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration

### Cabinet

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute,
E.g 
<managedObject class="HW" version="1.0" distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1"
 the node ID for this cabinet is: BSC-395041/BCF-158

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute - E.g. distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1" cabinet ID= 1 |
| Model | [Mandatory] | - If the value of “unitType” attribute of one UNIT is equal to FCIA Model = FCIA - If the value of “unitType” attribute of one UNIT is equal to FCOA Model = FCOA - If “FCIA” and “FCOA” are not found, Model = Flexi Multiradio |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |

### Shelf

- Each class="SUBRACK" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute,
E.g 
<managedObject class="SUBRACK" version="1.0" distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1/SUBRACK-1"
 the node ID for this cabinet is: BSC-395041/BCF-158

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf (Subrack-1) is shelf id for below exp: class="SUBRACK" version="1.0" distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1/SUBRACK-1" |
| Model | [Mandatory] | - “Subrack NSN BTS” to add by default |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute,
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1/SUBRACK-1/UNIT-ESMB_1_1"
 Node ID= PLMN-PLMN/BSC-395041/BCF-158

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1/SUBRACK-1/UNIT-ESMB_1_1"

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1/SUBRACK-1/UNIT-ESMB_1_1"

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-395041/BCF-158/HW-1/SUBRACK-1/UNIT-FIQB_1_4"

  - These above two boards belong to the same shelf (SUBRACK-1)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf (SUBRACK-1 i.e. 1) |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">1</p> slot index =1 <p name="identificationCode">472109A</p> <p name="operationalState">1</p> <p name="position">1</p> <p name="serialNumber">TY134308837</p> <p name="unitId">1</p> <p name="unitType">ESMB</p> <p name="vendorName">Nokia</p> <p name="version">102</p> |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class - E.g. <p name="identificationCode">472109A</p> - <p name="operationalState">1</p> - <p name="position">1</p> - <p name="serialNumber">TY134308837</p> - <p name="unitId">1</p> - <p name="unitType">ESMB</p> - <p name="vendorName">Nokia</p> - <p name="version">102</p> |
| Part Number | [Optional] | - = “identificationCode” attribute - E.g. <p name="identificationCode">472109A</p> <p name="operationalState">1</p> <p name="position">1</p> <p name="serialNumber">TY134308837</p> <p name="unitId">1</p> <p name="unitType">ESMB</p> <p name="vendorName">Nokia</p> <p name="version">102</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = “serialNumber” attribute - E.g. <p name="identificationCode">472109A</p> <p name="operationalState">1</p> <p name="position">1</p> <p name="serialNumber">TY134308837</p> <p name="unitId">1</p> <p name="unitType">ESMB</p> <p name="vendorName">Nokia</p> <p name="version">102</p> |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

# NodeB:

- Each class="HW" which includes WBTS in attribute “distname” represents one NodeB.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - E.g distName="PLMN-PLMN/RNC-751/WBTS-4159 ID= RNC-751/WBTS-4159 distName="PLMN-PLMN/RNC-751/WBTS-4159/HW-1 |
| NE Name | [Mandatory] | Same as NE ID |
| Site id | [Mandatory] | - Follow the site matching sheet |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = NodeB where NE Type will be “WBTS” we will consider it as NodeB |
| Model |  | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. - <p name="systemTitle">Nokia Flexi WCDMA Base Station</p> |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration

### Cabinet

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute,
E.g. distName="PLMN-PLMN/RNC-751/WBTS-4159/HW-1”

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute - E.g. distName=" distName="PLMN-PLMN/RNC-751/WBTS-4159/HW-1" cabinet ID= 1 |
| Model | [Mandatory] | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. - <p name="systemTitle">Nokia Flexi WCDMA Base Station</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |

### Shelf

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - “userLabel” under class="Module" |
| RU Index | [Mandatory] | - =[(Shelf id -1)*2]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =3 |
| Description | [Optional] | - Null |

### Boards

- Each class="Module" and class=" SUBMODULE"  represents one board

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - Shelf ID to be deduced from distname. - E.g., Shelf ID to be deduced from distname. E.g., distName="PLMN-PLMN/RNC-402/WBTS-4159/HW-1/MODULE-173/SUBMODULE-225" → Shelf ID is the shelf deduced from ="PLMN-PLMN/RNC-402/WBTS-4159/HW-1/MODULE-173” |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Slot index of the first board on the shelf is 1 and to be incremented by 1 automatically for each new board. |
| Board Type | [Mandatory] | - <p name="unitType"> attribute under the same class SUBMODULE. - E.g. <p name="unitType">FTIB</p> |
| Part Number | [Optional] | - = “identificationCode” attribute - E.g. <p name="identificationCode">472182A</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = “serialNumber” attribute - E.g. <p name="serialNumber">K9153917272</p> |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

# Antenna:

## Antenna data

We need to parse antenna details in radioantenna csv sheet.

- The following attributes are defined at antenna node level in radioantenna csv & serial number will be parse under addinfo csv.

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - Concatenate of Parent Node & last integer value in attribute distName: Example:1 distName="PLMN-PLMN/MRBTS-417373/HW-1/INVUNIT-34" NE ID: MRBTS-417373_34 Example:2 distName="PLMN-PLMN/MRBTS-506262/HW-1/INVUNIT-28" NE ID: MRBTS-506262_28 Example:3 distName="PLMN-PLMN/MRBTS-417990/HW-1/INVUNIT-42" NE ID: MRBTS-417990_42 Example:4 distName="PLMN-PLMN/RNC-504/WBTS-3343/HW-1/MODULE-40192/SUBMODULE-34817" NE ID: RNC-504/WBTS-3343_34817 |
| NE Name | [Mandatory] | Same as NE ID |
| Site id | [Mandatory] | - Follow the site matching sheet & Rule |
| Site Name | [Optional] | - = Site id |
| Type | [Mandatory] | - = Antenna where attribute <p name="inventoryUnitType"> contains “SingleAntennaDevice” or “MultiAntennaDevice” if this will be “N/A” then consider the value of “<p name="vendorUnitFamilyType">”. And <p name="unitType"> (In case of TILT) <p name="unitType">TILT</p> And <p name="inventoryUnitType"> (In case of MDDA) <p name="inventoryUnitType">MDDA</p> |
| Model | [Optional] | Same as Type if require |
| Serial Number | [Mondatory] | Serial number will get from below attribute: <p name="serialNumber" within above mentioned type only. |
| Manufacturer | [Optional] | - = Nokia will need to take from below attribute: <p name="vendorName"> |