Huawei 3G Parsing

FRS (Functional Requirement Specifications)

Auto-Import

# Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft. | Indrasan Yadav | 06-05-2025 |

# Preamble

## Logical

- This document specifies how to parse the logical inventory attributes for the NodeB and its cells.

- It covers the parameters defined for NodeB RAN Logical attributes.

- The provided dump files are of “.xml”.

- For Ooredoo Tunisia, each NodeB logical dump file is separate.

- This is motivated by Oman-Tel/ZAIN-KSA Projects and can be used for other operators having same file structure.

## Interfaces

- This document specifies how to parse the logical inventory attributes of the Huawei 3G interfaces:

  - Iub connecting NodeB to RNC.

  - Iur connecting RNC to RNC.

  - Iu-CS connecting RNC to MGW or MSS.

  - Iu-PS connecting RNC to SGSN.

- Interfaces can be over IP or ATM.

- Iu-CS and Iu-Ps physical connection to be defined temporarily from RNC side only, Core Element physical connection to be added after introducing the core module.

- NodeB Dumps:

  - NodeB dump files are XML files structured similarly to Huawei eNodeB logical dumps.

- RNC Dump:

- RNC dump files are text files with MML script files as CFG files.

# Dump Structure

Dumps are provided in different structures for NodeB & RNC:

1st Structure type for NodeB:

Dumps are provided in 2 parts but in same structure 1st one will be physical in .xml & 2nd one will be logical in .xml file for each node of NodeB:

# One XML file per NodeB node.

- The dump file is an “.xml” file composed of set of Classes.

- This dump contains all the logical configuration for the nodes.

2nd Structure type for NodeB:

- The provided dumps are one or multiple XML files for multiple NodeBs

- This dump contains all the logical configuration for the nodes

- The dump consists of multiple <subsession> section

- Each subsession contains the configuration of one node (NE)

- In this document we are referring to each class by its type. E.g. “NE” class  the class that has the type “NE”

- Attributes for each class are found between < >, e.g. <IP> attribute under the “DEVIP” class.

- The attributes that are found under a specific “NE” class relate to this NE

One node configuration

Attributes

Class Type

One Class

2nd Structure type for RNC:

- The provided dumps for RNC physical are in .xml & logical are in MML script file (CFGMML file).

- This dump contains all the logical configuration for the nodes.

# NodeB logical parameters

The eNodeB logical inventory attributes are classified as per the below categories:

  - Global Parameters

  - Networking Parameters

Note: The internal mapping between the Physical and Logical parameters will be based on the Node Name.

## Global Parameters

The below global attributes defines at NodeB level are as follow:

Priority-1: The internal mapping between the Physical and Logical parameters will be based on the Node Name.

Priority-2: Try to match to NodeB under SRAN-BS.

Priority-3:

  - Match (CellID and parent RNC ID) of logical with (CellID and parent RNC ID) of physical following the below rule:

  - Fetch “CellID” attribute of concerned Logical NodeBName under “ADD UCELLSETUP” tag in RNC logical dump.

  - Fetch Parent RNC ID of concerned Logical NodeBName.

  - Fetch in the physical dumps under same RNC the physical site with “CellID” attribute under "CELL" table matching with the logical “CELLID”.

  - Matching Physical NodeBName=” NodeBName” attribute on the same line.

Priority-3: (For Ooredoo Tunisia)

- From Logical dump (CFGMML-RNCxxx-xxxx-xxxxx), read CELLID and the corresponding NODEBNAME under “ADD UCELLSETUP” and get the value of “SYSOBJECTID” under “SET SYS”.

- In physical dump (AIM_BSC6900UMTS_xxx-xxx.xml), Match “SYSOBJECTID” with NEName under NE tag.

- Under the matched NEName, search for a row under <TABLE attrname="UCELL"> that contains CellId=CELLID and NodeBName=NODEBNAME.

Priority-4:

    - Match logical NodeB name to NodeBFunctionName under ROWDATA under MBTS3GCell or under UCELL in SRAN-BS physical.

    - Or match logical NodeB name to NodeBFunctionName under ROWDATA under NodeBCell in NodeB physical.

    - If matched to SRAN-BS then match to the NodeB under SRAN-BS, if no NodeB under SRAN-BS then create a NodeB with same NodeB logical name.

| Attribute | Importance | Example |
| --- | --- | --- |
| Node ID | [Mandatory] | - =”NODEBID” attribute under “ADD UNODEB” tag of each line. (In RNC logical dump) - Note: If more than one Node ID was detected, only take the 1st detected one. - E.g.: ADD UNODEB:NODEBID=635, NODEBNAME="UM03640" → Node ID=635. |
| Node Name | [Mandatory] | - =”NODEBNAME” attribute under “ADD UNODEB” tag of each line. (In RNC logical dump) - Note: If more than one Node ID was detected, only take the 1st detected one. E.g.: ADD UNODEB:NODEBID=635, NODEBNAME="UM03640" → NODEBNAME=”UM03640” |
| Site Name | [Mandatory] | - It will be describing later once full dump/data will receive from customer. |
| Type | [Mandatory] | - =deduced from node configuration or the value of “NEType=” in physical dump. |
| OAM IP | [Optional] | - = “NBIPOAMIP“ or “NBATMOAMIP” value under “ADD UNODEBIP“ tag of each line. - The related NodeB Name is deduced after matching the "NODEBID" attribute under "ADD UNODEBIP" with the "NODEBID" under "ADD UNODEB" tag. Then NodeB Name will be the value of "NODEBNAME ". - E.g.: NBIPOAMIP=" 10.221.74.102". |
| Subnet Mask | [Optional] | - =”NBIPOAMMASK” or ”NBATMOAMMASK” value under “ADD UNODEBIP” tag of each line. - E.g.: NBIPOAMMASK=" 255.255.255.0". |
| 64-QAM support | [Optional] | - Deduced from cell level. (Which is described under Cell Logical Parameter) - It is sufficient that at least one CELLID belonging to NodeB Name has 64-QAM support=YES, so that the NODEB itself has 64-QAM supported. |
| HSDPA Support | [Optional] | - Deduced from cell level. (Which is described under Cell Logical Parameter) - It is sufficient that at least one CELLID belonging to NodeB Name has HSDPA support=YES, so that the NODEB itself has HSDPA support. |
| HSUPA Support | [Optional] | - Deduced from cell level. (Which is described under Cell Logical Parameter) - It is sufficient that at least one CELLID belonging to NodeB Name has HSUPA support=YES, so that the NODEB itself has HSUPA support. |
| MIMO support | [Optional] | - Deduced from cell level. (Which is described under Cell Logical Parameter) - It is sufficient that at least one CELLID belonging to NodeB Name has MIMO support=YES, so that the NODEB itself has MIMO support. |

## Networking Parameters

The networking attributes below defined at NodeB level are as follows:

| Attributes | Importance | Example |
| --- | --- | --- |
| MCC | [Mandatory] | - NEP Tool Calculated |
| MNC | [Mandatory] | - NEP Tool Calculated |
| TAC list | [Optional] | - NEP tool calculated, TAC list of children cells. |
| Operator | [Mandatory] | - =“ CNOPERATORNAME=” value under tag “ADD UCNOPERATOR” in RNC logical dump. - E.g.: ADD UCNOPERATOR:CNOPINDEX=0, CNOPERATORNAME="Omantel", |
| Parent RNC | [Mandatory] | - ="SYSOBJECTID" attribute under "SET SYS" tag in RNC logical dump. - E.g.: SYSOBJECTID= "Wattaya-RNC05" |

## Cell Logical Parameters (CFGMML)

| Attributes | Importance | Example |
| --- | --- | --- |
| Cell Name | [Optional] | - =” CELLNAME” attribute under “ADD UCELLSETUP” of each tag. - E.g.: ADD UCELLSETUP:CELLID=21821, CELLNAME="UM0246021F2A" |
| Cell ID | [Mandatory] | - The “Cell ID” is equal to the concatenation of “LAC”, “_” and “CELLID” - Get the” CELLID” attribute under “ADD UCELLSETUP” of each tag. - Get the “LAC” value attribute under “ADD UCELLSETUP” of each tag. - Note: if the “LAC” value is in hexadecimal (e.g. H’C3B6), a conversion into decimal value is required. |
| Cell Technology | [Mandatory] | - =” UMTS” (By default) The value of “SYSSERVICES=” under tag “SET SYS” |
| Node ID |  | [Mandatory] |  | - =Deduced from the “NODEBNAME” attribute under “ADD UCELLSETUP” tag - E.g.: NODEBNAME="UM02460" |  |
| Parent Site ID | [Mandatory] | - =Deduced from parent Node (Which will be described later once full dump will be received from customer) |
| Neighbor Cells 2G | [Optional] | - Get the value of” GSMCELLINDEX” attribute under “ADD U2GNCELL” tag and match with the same value of “GSMCELLINDEX “under the command line “ADD UEXT2GCELL”. - If the value is matched, get the correspondent “LAC” and “CID” value which is mentioned on the same line of the command line “ADD UEXT2GCELL”.Thus , the “Neighbour cells 2G” is deduced as follow: It is a concatenation of “LAC”, ”_” “CID” Note: if the value of “CID” and “LAC” are hexadecimal, hence they needs to be converted into decimal. Note: The “CELLID” attribute under “ADD U2GNCELL “represents the parent “CELLID” attribute, thus you must match the “CELLID” value with the same attribute name under “ADD UCELLSETUP” to get the “LAC” value to deduce the parent cell ID. - Note: if the “LAC” value is in hexadecimal (e.g. H’213), a conversion into decimal value is required. |
| Neighbor Cells 3G | [Optional] | - Get the value of” NCELLID” attribute under” ADD UINTERFREQNCELL” and “ADD UINTRAFREQNCELL” tags. - Match the obtained value with the “CELLID” attribute under “ADD UEXT3GCELL” once matched, get the value of “LAC” mentioned on the same line, where the Neighbor cells 3G will be a concatenation of “LAC”, “_” and “NCELLID”. - Else value not matched, match the value of “NCELLID” with the value of “CELLID” under “ADD UCELLSETUP”. - Once matched, get the “LAC” value mentioned on the same line of “ADD UCELLSETUP”. - Note: if the “LAC” value is in hexadecimal (e.g. H’213), a conversion into decimal value is required. - The parent Cell ID would be deduced as follows: - Get the value of “CELLID” mentioned under “ADD UINTERFREQNCELL” and “ADD UINTRAFREQNCELL”. Match the value of “CELLID” under “ADD UCELLSETUP” to get the “LAC” value. - Note: if the “LAC” value is in hexadecimal (e.g. H’213), a conversion into decimal value is required. - Thus, the parent cell ID will be concatenation of “LAC”, “_” and “CELLID”. |
| Neighbor Cells 4G | [Optional] | - Get” LTECELLINDEX” attribute under “ADD ULTENCELL” tag, where for each “CELLID” attribute there are a set of neighbour cells 4G. - Lookup for the same value of “LTECELLINDEX” under tag “ADD ULTECELL”. - Once matched, take the value of “LTECELLNAME” and lookup for the same value under CellattributesLocalCellIDCellName within the 4G dump files. - Once matched, the “neighbour 4G cell ID” will be the concatenation of the “eNodeB Name” with the “CellId” attribute. - Note: If once getting the “LTECELLNAME” the cell ID can be deduced at saving, no need then for the last two steps. |
| Last Audit Date | [Optional] | - =Null |
| Additional Information | [Optional] | - =Null |

Below are the specific attributes to being parsed in case the cell technology is UMTS

- Specific attributes for 3G cells

| Attributes | Importance | Example |
| --- | --- | --- |
| Parent RNC ID | [Optional] | - =To be deduced from parent node |
| Primary Scrambling code | [Optional] | - = “PSCRAMBCODE” attribute under “ADD UCELLSETUP” tag of each line. - The cell ID is got through “CELLID” attribute. |
| LAC | [Optional] | - = "LAC" value under "ADD UCELLSETUP" tag of each line, take only the value followed by " ‘“. E.g.: LAC=H'0385 0385 - The value is in hexadecimal, a conversion into decimal value is needed. - E.g.: 0385 901 - The obtained value refers to the cell ID value of "CELLID" attribute. |
| RAC | [Optional] | - = "RAC" value under "ADD UCELLSETUP" tag of each line, take only the value followed by " ‘“. - E.g.: RAC=H'6F - The value is hexadecimal, we need to change it to decimal - E.g.: 6F 111 - The obtained value refers to the cell ID value of "CELLID". |
| SAC | [Optional] | - = "SAC" value under "ADD UCELLSETUP" tag of each line, take only the value followed by "‘“. - E.g.: SAC= H'EB83 EB83 - The value is in hexadecimal, a conversion into decimal value is needed. - The obtained value refers to the cell ID value of "CELLID”. |
| URA | [Optional] | - = “URAID” attribute under “ADD UCELLURA” tag of each line. - The cell id is got through “CELLID” attribute. |
| DTX Enabled | [Optional] | - To be deduced through the attribute "HSPAPLUSSWITCH" Under "UCELLALGOSWITCH" tag as follows: - = "Yes" if the digit after the keyword " DTX_DRX- " is equal to “1” - =” No” if the digit after the keyword " DTX_DRX- " is equal to “0” - E.g.: HSPAPLUSSWITCH=64QAM-1&MIMO-0&E_FACH-0&DTX_DRX-0 - From each line of “UCELLALGOSWITCH” tag, get the “CELLID” attribute. |
| HSDPA Enabled | [Optional] | - A Cell has HSDPA Enabled if it’s CELL ID is matched with "CELLID" attribute under “ADD UCELLHSDPA" tag - Else not matched, Default value =NO |
| HSUPA Enabled | [Optional] | - A Cell has HSUPA Enabled if its CELL ID is matched with "CELLID" attribute under “ADD UCELLHSUPA" tag. - Else not matched, Default value =NO |
| 64-Qam Enabled | [Optional] | - To be deduced through the attribute " HSPAPLUSSWITCH" Under "UCELLALGOSWITCH" tag as follows: - = "Yes" if the digit after the keyword "64QAM-" is equal to “1” - =” No” if the digit after the keyword "64QAM-" is equal to “0” - E.g.: HSPAPLUSSWITCH=64QAM-1&MIMO-0 - From each line of “UCELLALGOSWITCH” tag, get the “CELLID” attribute |
| MIMO Enabled | [Optional] | - To be deduced through the attribute "HSPAPLUSSWITCH" Under "UCELLALGOSWITCH" tag as follows: - = "Yes" if the digit after the keyword "MIMO-" is equal to “1” - =” No” if the digit after the keyword "MIMO-" is equal to “0” - E.g.: HSPAPLUSSWITCH=64QAM-1&MIMO-0 - From each line of “UCELLALGOSWITCH” tag, get the “CELLID” attribute. |
| MIMO Configuration | [Optional] | - =Null (could be modified later) |
| E-Tilt | [Optional] | - =Null |
| Frequency band | [Optional] | - To be deduced through the attribute “BANDIND” Under “ADD UCELLSETUP” tag as follow: - Band1: 2100 - Band2: 1900 PCS - Band3: 1800 DCS - Band4: AWS - Band5: 850 - Band6: 850 Japan - Band7: 2600 - Band8: 900 - Band9: 1800 Japan |
| Cell status | [Optional] | - If the cell has a command ACT UCELL where CELLID is matching with CELLID under ADD UCELLSETUP then cells status is ACTIVE. - Else, the cell is in INACTIVE state. |

### Specific attributes for 4G cells (Carrier Logical Parameters)

- Under the cell, there will be a carrier folder. For each carrier, the following attributes should be defined.

| Attributes | Importance | Example |
| --- | --- | --- |
| Carrier ID | [Optional] | - The “Carrier ID” is equal to the concatenation of: “Cell ID”, “_” and “UARFCN Downlink” |
| UARFCN Downlink | [Optional] | - =” UARFCNDOWNLINK” under “ADD UCELLSETUP” tag of each line. - The cell ID is got through “CELLID” attribute. - E.g.: UARFCNDOWNLINK=10638 |
| UARFCN Uplink | [Optional] | - =” UARFCNUPLINK” under “ADD UCELLSETUP” tag of each line. - The cell ID is got through “CELLID” attribute. - E.g.: UARFCNUPLINK=9688 |
| Code Type | [Optional] | - HSDPA codes and Shared codes: Number of HSDPA and shared codes are deduced as per below: - If” AllocCodeMode” attribute under “ADD UCELLHSDPA” tag is equal to “Manual” then number of HSDPA codes =” HSPDSCHCODENUM” attribute on the same line and number of Shared codes =0. - If” AllocCodeMode” attribute under “ADD UCELLHSDPA” tag is equal to “Automatic” then number of HSDPA codes =” HSPDSCHMINCODENUM” attribute on the same line and number of shared codes = ”HSPDSCHMAXCODENUM” - ”HSPDSCHMINCODENUM” attributes on the same line. - Attributes refer to CELLID on the same line. - Number of Reserved Codes= 1. - UMTS Codes = 16-HSDPA codes-Shared Codes-Reserved codes). - E.g. HSDPA codes=5 - Shared codes=10 - Reserved codes=1. - UMTS codes=16-5-10-1=0. - Assign codes: codes are assigned to the carrier under the concerned cell as per below priorities: - Priority 1: Reserved Codes. - Priority 2: UMTS codes. - Priority 3: Shared codes. - Priority 4: HSDPA codes. - Example: - HSDPA codes=5 - Shared codes=10 - Reserved codes=1. - UMTS codes=0. - Assignment would be as per below: - Code Type 1=Reserved. - Code Type 2=Shared. - Code Type 3=Shared. - Code Type 4=Shared. - Code Type 5=Shared. - Code Type 6=Shared. - Code Type 7=Shared. - Code Type 8=Shared. - Code Type 9=Shared. - Code Type 10=Shared. - Code Type 11=Shared. - Code Type 12=HSDPA. - Code Type 13=HSDPA. - Code Type 14=HSDPA. - Code Type 15=HSDPA. - Code Type 16=HSDPA. |
| Supports HSDPA? | [Optional] | - =” True” if any of the Carrier codes is HSDPA or Shared. - Else, =” False”. |

# Addinfo

- Below listed additional info to be added on RNC.

- Attribute name is the concatenation of Category and value.

- The values in note field should be separated with “;”

- In note field, if there are multiple values for an attribute  separate them with “,”

- If OPC/DPC found in HEXADECIMAL that need to be converted to Decimal first, if in the value has H’ as prefix OR having alphabets then it is considered as HEX and need to be converted to DEC.

- Take 2nd Association in consideration only if LOCIP2 & PEERIP2 attribute contains valid nonzero values.

| Category | Value | Note |
| --- | --- | --- |
| IPRT | = the value of “NEXTHOP” attribute under each “ADD IPRT” tag | - NETWORK= the value of “DSTIP” attribute under “ADD IPRT” tag - MASK= the value of “DSTMASK” attribute under “ADD IPRT” tag - GATEWAY= the value of “NEXTHOP” attribute under “ADD IPRT” tag |
| OPC | = the value of “SPC” attribute under each “ADD OPC” tag | - OPC= the value of “SPC” under “ADD OPC” tag |
| ASSOCIATION | = the value of “ANI” attribute under each “ADD IPPATH” tag | - LOCAL_IP= the value of “IPADDR” attribute under each “ADD IPPATH” tag - REMOTE_IP= the value of “PEERIPADDR” attribute under each “ADD IPPATH” tag - ID= the value of “ANI” attribute under each “ADD IPPATH” tag |

| Category | Value | Note |
| --- | --- | --- |
| IPRT Every “ADD IPRT” represents one IPRT | the value of under “NEXTHOP” attribute under “ADD IPRT” | - NETWORK= the value of under “DSTIP” attribute under “ADD IPRT” - MASK= the value of under “DSTMASK” attribute under “ADD IPRT” - GATEWAY= the value of under “NEXTHOP” attribute under “ADD IPRT” |

| Category | Value | Note |
| --- | --- | --- |
| OPC | Take the value of “SPC” attribute under command “ADD OPC” | NI-3 |

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE Every “ADD SCTPLNK” represents one ADJNODE | The value of “SCTPLNKID” attribute under “ADD SCTPLNK” | - CONNECTION_TYPE=IUCS - CONNECTION_TYPE=IUPS - CONNECTION_TYPE=IUR - NODE_ID= IP_DERIVED Note: for below statements SCTPLNKN=X is not present in ADD SCTPLNK then take SCTPLNKID=X instead & match under ADD M3LNK and proceed - DPC: Take the attribute SCTPLNKN=X from ADD SCTPLNK command and match this under ADD M3LNK. Take the attribute SIGLKSX=X and match in ADD M3LKS. Take the attribute DENO=X and match in ADD M3DE. Take attribute DPX=X and match under ADD N7DPC, from this take the value of DPC. NOTE: X is any integer - OPC: Take the value of “SPC” attribute under command “ADD OPC”. NOTE: If found multiple ADD OPC statements then take the value of SPX attribute from above deduced N7DPC and match it with the SPX attribute value under ADD OPC, take corresponding value of SPC attribute as OPC. - LINKSET: Take the attribute SCTPLNKN=X from ADD SCTPLNK command and match this under ADD M3LNK. Take the attribute SIGLKSX=X and match in ADD M3LKS.Now take the value of attribute “NAME” as LINKSET. - CONNECTION_NAME: Take the attribute SCTPLNKN=X from ADD SCTPLNK command and match this under ADD M3LNK. Now take the value of attribute “NAME” as CONNECTION_NAME. |
| ASSOCIATION // 1st association // | The value of “SCTPLNKID” attribute under “ADD SCTPLNK” | - LOCAL_IP= the value of “LOCIP1” under “ADD SCTPLNK” - REMOTE_IP= the value of “PEERIP1” under “ADD SCTPLNK” |
| ASSOCIATION // 2nd association // | The value of “SCTPLNKID” attribute under “ADD SCTPLNK” | - LOCAL_IP= the value of “LOCIP2” under “ADD SCTPLNK” ( ignore this association if LOCIP2 or PEERIP2 is 0.0.0.0) - REMOTE_IP= the value of “PEERIP2” under “ADD SCTPLNK” |

  - IUB

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE //IUB // This category only for ADD ADJNODE tags where NODET=IUB | = the value of “ANI” attribute under each ADD ADJNODE tag | - CONNECTION_TYPE= the value of “NODET” attribute under “ADD ADJNODE” tag. This value should be matched with the Enum ran interfaces and to be null if not found - NODE_ID= - Take the value of “NODEBID” attribute under “ADD ADJNODE” tag - Match it with “NODEBID” attribute under “ADD UNODEB” tag - Take the value of “NODEBNAME” attribute - ID= the value of “ANI” attribute under “ADD ADJNODE” tag - DESCRIPTION=”Iub” |

  - IUR, IUCS and IUPS

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE //IUR, // This category is only for ADD ADJNODE tags where NODET=IUR, IUCS or IUPS | = the value of “ANI” attribute under each “ADD ADJNODE” tag | - CONNECTION_TYPE= the value of “NODET” attribute under “ADD ADJNODE” tag. This value should be matched with the Enum ran interfaces and to be null if not found - DPC= - Take the value of “DPX” attribute under “ADD ADJNODE” tag - Match it with “DPX” attribute under “ADD N7DPC” tag - Take the value of “DPC” attribute - NODE_ID= IP_DERIVED - ID= the value of “ANI” attribute under “ADD ADJNODE” tag - DESCRIPTION= Value of” NODET” |

# Networking Parameters

The following networking attributes defined at RNC level are as follows:

| Attributes | Importance | Example |
| --- | --- | --- |
| MCC | [Optional] | - = NEP tool calculated. |
| MNC | [Optional] | - = NEP tool calculated. |
| Connected RNCs | [Optional] | - Destination RNCs deduced from Iur Interfaces. |
| Serving MSSs | [Optional] | - If RNC has A-C interfaces with MGW destination, then: - Get Destination MGWs from IuCS-C interfaces. - Serving MSSs are destination MSSs connected to above MGWs via IuCS-C interfaces. - Else, Destination MSSs deduced from IuCS-C Interfaces. |
| Serving MGWs | [Optional] | - Destination MGWs deduced from IuCS-C and IuCS-U Interfaces. |
| Serving SGSNs |  | - Destination SGSNs deduced from IuPS Interfaces. |
| Operator | [Optional] | - =” CNOPERATORNAME” under “ADD UCNOPERATOR”. |

# IP, VLAN, GW Parsing

## Physical Port IP

- Deduced from “ADD ETHIP” tags.

- If more than one interface were found on one physical port,  consider them as sub-interfaces.

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Node Name_Shelf_Slot_Port_SubInterface. |
| Physical Sub-interface C/Sh/S/I/P | [Mandatory] | - Take the value of “SRN” under “ADD ETHIP” - Take the value of “SYSOBJECTID” under “SET SYS” - Priority 1: Match it in Logical_Physical_Matching file column A “Logical Name” and take column B “Physical Name” - Priority 2: If not found in the static file, take “SYSOBJECTID” value as it is. - Match it with “NEName” in physical dumps - For AIM dumps: - Match SRN with “SubrackNo” under “Subrack” class - Cabinet index= “CabinetNo” - Shelf index= “SRN” - For DIM dumps: - Match “SRN” with “FrameNo” - Cabinet index= “RackNo” - Shelf index= “InventoryUnitId” - Slot, Index on Slot and Port are equal to SN, 0 and PN under ADD ETHIP. - Sub-interface index = IPINDEX under ADD ETHIP. |
| IP address | [Optional] | - = “IP” under “ADD ETHIP” |
| Subnet Mask | [Optional] | - = “MASK” under “ADD ETHIP” |
| Description | [Optional] | - = “REMARK” attribute under “ADD ETHIP” |

## Device IP

- Deduced from “ADD DEVIP” tags.

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Node Name_SRN_SN - SRN and SN are under “ADD DEVIP” |
| IP address | [Optional] | - = “IPADDR” under “ADD DEVIP”. |
| Mask | [Optional] | - 255.255.255.248 |

## Ethernet Trunk IP

- Deduced from “ADD ETHTRKIP” tags.

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Type | [Mandatory] | - Ethernet Trunk |
| Interface Name | [Mandatory] | - =ETHTRKIP_SRN_SN_TRKN_IPINDEX. - SRN, SN, TRKN and IPINDEX under “ADD ETHTRKIP”. |
| IP address | [Optional] | - = IP under “ADD ETHTRKIP” |
| Subnet Mask | [Optional] | - = MASK under “ADD ETHTRKIP” |
| Physical C/Sh/S/I/P/Sub-Interface | [Mandatory] | - Shelf, Slot, Index on Slot and Port are equal to SRN, SN, 0 and TRKN under “ADD ETHIP”. - Sub-interface index = IPINDEX under “ADD ETHIP” |
| Gateway | [Mandatory] | - From IPADDR and MASK under all above “ADD ETHTRKIP” find the range of valid IPs for each “ADD ETHTRKIP”. - Find “ADD SRCIPRT” with “NEXTHOP” under “ADD SRCIPRT” is matching within the range of IPs of “ADD ETHTRKIP” and SRN, SN under “ADD SRCIPRT” are matching with SRN, SN under “ADD ETHTRKIP”. - Gateway=” NEXTHOP” under “ADD SRCIPRT”. |
| VLAN | [Mandatory] | - Match “NEXTHOP” under “ADD SRCIPADDR” with “IPADDR” under “ADD VLANID”. - VLAN = “VLANID” under “ADD VLANID”. |

## NodeB3812E IP Parsing

- Each “ETHIP” class under “ETHIPCLASS” represents one node interface

- If more than one interface were found on one physical port,  consider them as sub-interfaces

| Attributes | Importance | Parsing |
| --- | --- | --- |
| NodeB Name |  | - From NODEBSRCID class - E.g. <NODEBSRCID ID = "0x2400001d">UB00030</NODEBSRCID> NodeB Name= UB00030 |
| Interface Name | [Mandatory] | - =Node Name_Shelf_Slot_Port |
| Physical Sub-interface C/Sh/S/I/P | [Mandatory] | - Cabinet index= bCabinetNo under “ETHIP” class - Shelf index= bSubrack under “ETHIP” class - Slot index= bIubBoardNo under “ETHIP” class - Index on slot= bSubBoardNo under “ETHIP” class - Port index= bPortNo under “ETHIP” class |
| IP address | [Optional] | - Take the value of “dwIp” under “ETHIP” class - Convert it from decimal to binary - If the result has less than 32 bits - Add number of zeroes on the left to complete the result to 32 bits - Each 8 bits will represent an octet in the IP address - Convert each octet from binary to decimal - Separate the octets by “.” To get the IP address format - E.g. <dwIp ID = "0x48420004">182326536</dwIp> - In binary= 1010110111100001010100001000 - Extend to 32 bits= 00001010110111100001010100001000 - Separate to octets= 00001010 11011110 00010101 00001000 - Convert each octet to decimal and add “.”= 10. 222.21.8 |
| Subnet Mask | [Optional] | - Take the value of “dwMask” under “ETHIP” class - Repeat the same steps of calculating the IP address |
| Description | [Optional] | - =Null |

## SRAN IP Parsing

- From NE Backup folder  BTS3900 folder

- Deduced from <DEVIP> tags.

- If more than one IP were found on a physical interface,   consider them as sub-interfaces

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Node Name | [Mandatory] | - = NENAME under <NE> |
| Interface Name | [Mandatory] | - =Node Name_Shelf_Slot_Port_SubInterface. |
| Physical Sub-interface C/Sh/S/I/P | [Mandatory] | - Cabinet,Shelf, Slot, Index on Slot and Port are equal to CN, SRN, SN, SBT and PN under <DEVIP> - Sub-interface index = to be incremented from 0 |
| IP address | [Mandatory] | - = IP under <DEVIP> |
| VLAN | Optional | - = Null |
| Subnet Mask | [Mandatory] | - = MASK under <DEVIP> |
| Description | Optional | - = the value of USERLABEL attribute under <DEVIP> |

# Interfaces (Need to cross check once again when full dump received)

## “Iub” Over ATM Interface

- Each Iub over ATM interface is represented in logical dumps with one “ADD ADJNODE” tag where “NODET” equals “IUB” and “TRANST” equals “ATM” or “ATM_IP”.

- Iub over ATM interface has one or several AAL2 paths represented each with one “ADD AAL2PATH” tag where “ANI” value is matching with “ANI” under “ADD ADJNODE” tag.

- For VDC Only:

  - Other than logical dumps, please use “.txt” file that will be available under a folder named “nodebmml”.

  - This .txt file contains the whole NodeBs ATM configuration, ranging from the physical connection toward the allocated number of E1s per each NodeB.

  - The file would have the following structure:

  - The beginning of each node will begin with “MML Command-----LST IMAGRP:;”

  - The end of each node will be “ ---    END”

  - The nodeB name will be mentioned after the key-word “NE :” , Example NE: 3G_Nwanedi_LIM_New.

  - Note: Per each nodeB, we could have under the “List IMA Group configuration” more than one configuration (more than  “IMA Group No.” as highlighted in yellow:

  - Below are the attributes of an Iub over ATM interface considering RNC is the Participant2 and NodeB is the Participant1:

| Attributes | Importance | Example |
| --- | --- | --- |
| NodeB Name | [Mandatory] | - To be deducted from Logical and Physical dumps. - Get “NODEBID” attribute under each tag of “ADD ADJNODE” where “NODET” equals “IUB” and “TRANST” equals “ATM” from Logical dumps - Participant1 Name = Node Name of above deduced Node ID in physical configuration. |
| Interface Name | [Mandatory] | - =”NodeB Name”_”RNC Name”_”ATM” |
| Interface Type |  | - “Iub” |
| Interface Technology | [Mandatory] | - “ATM” |
| RNC Name | [Mandatory] | - To be deducted from Logical and Physical dumps. - Get “RNCID” attribute under tag of “ADD URNCBASIC” from Logical dump. - Participant1 Name=RNC Name of above deduced RNC ID in physical configuration. |
| RNC C/SH/S/I/P | [Optional] | - if matching to nodebmml is successful then: - To be deduced from both Logical and Physical Dumps. - Get “CARRYF” and “CARRYSN” attributes under “ADD AAL2PATH”. - Find Board with “FrameNo”= “CARRYF” and “SlotNo”= “CARRYSN” under “Board” table in physical dumps. - Participant 2 Cabinet, Shelf and Slot are equal to Cabinet, Shelf and Slot of obtained board. - Port = “CARRYNCOPTN” attribute under “ADD AAL2PATH”. - If multiple “ADD AAL2PATH” tags were found for the same interface then repeat the above for all of them, if the same port is found then take it only one time, if different ports were found then take each port one time only. - Else: - Get CARRYF, CARRYSN and CARRYIMAGRPN attributes under ADD AAL2PATH tag. - Find an ADD IMALNK tag with SRN, SN and IMAGRPN matching with above CARRYF, CARRYSN and CARRYIMAGRPN. - Subrack number = SRN and Slot number = SN. - PN=round down (IMALNKN/63), IMALNKN is the attribute under ADD IMALNK tag (e.g. IMALNKN=62→PN=0, IMALNKN=63→PN=1, IMALNKN=126→PN=2). |
| NodeB C/SH/S/P | [Optional] | - if matching to nodebmml is successful then: - To be deduced from “.txt” file that will be available under a folder named “nodebmml”. - Find Participant1 name in “.txt” file. - Get Subrack No and Slot No under the Participant1 name. - The allocated number of ports will be derived per each board from the “IMA Link No. in IMA Group”; it will be equal to the count of digits available under this attribute. - Example: if it is equal to 0,1,2,3 we have 4E1s. - If the detected board type contains WMPT Then port index will start with 2, 3, 4 and 5. - Else if the detected board type contains UTRP3 then port index will start with 0,1…7 - Else: - Number of ports is equal to number of ADD IMALNK tags found in Participant2 C/SH/S/P. - Find in physical dump the board with name containing “MPT or “NUTI” or “BBU” (e.g. “WD22WMPT”). - Cabinet, Shelf and Slot index is equal to Cabinet, Shelf and Slot of above found board. - The port index then will start with index 2 in case of “MPT” and with 0 in case of “NUTI” or “BBU”. |
| TX BW | [Optional] | - =Number of Participant1 allocated ports * 2048. - E.g. 4 ports are allocated on the Participant1 → TX BW = 8192. |
| RX BW | [Optional] | - =TX BW. |

## “Iub” Over IP Interface

- Each Iub over IP interface is represented in logical dumps with one “ADD ADJNODE” tag where “NODET” equals “IUB” and “TRANST” equals “IP” or “ATM_IP”.

- Iub over IP interface has one or several IP paths represented each with one “ADD IPPATH” tag where “ANI” value is matching with “ANI” under “ADD ADJNODE” tag.

- Below are the attributes of an Iub over IP interface:

- Each S1-MME interface is represented with one <S1interface> Class in logical dumps.

- S1-MME interface has one associated <CPBEARER> class where <CPBEARID> under <attributes> under <CPBEARER> class is matching with <S1CpBearerId> under <attributes> under <S1Interface> class.

- <CPBEARER> class has an associated <SCTPLNK> class where <SCTPNO> under <SCTPLNK> class is matching with <LINKNO> under <attributes> under <CPBEARER> class.

- Below are the attributes of an S1-MME interface:

| Attributes | Importance | Example |
| --- | --- | --- |
| NodeB Name | [Mandatory] | - To be deducted from Logical and Physical dumps. - Get “NODEBID” attribute under each tag of “ADD ADJNODE” where “NODET” equals “IUB” and “TRANST” equals “IP” from Logical dumps - NodeB Name = Node Name of above deduced Node ID in physical configuration. |
| Interface Name | [Mandatory] | - =“NAME” attribute under tag of “ADD ADJNODE”. - Value refers to “NODEBID” on the same line. |
| Interface Type | [Mandatory] | - “Iub” |
| Interface Technology | [Mandatory] | - “IP” |
| RNC Name | [Mandatory] | - =Node Name |
| TX BW | [Optional] | - TX BW=”TXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found, then consider the maximum” TXBW” value. |
| RX BW | [Optional] | - TX BW=”RXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found, then consider the maximum” RXBW” value. |
| NodeB C/SH/S/I/P | [Optional] | - Priority1: - Match NodeB name with NODEBFUNCTIONNAME under attributes under NODEBFUNCTION under class in NodeBCFG or CFGDATA files or Match NodeB name with “NAME” attribute under the class that has the type “NE” in 2nd Dump structure for NodeB - Match PEERIPADDR under ADD IPPATH with IP under attributes under DEVIP under class in NodeB dump - C/Sh/S/P are equal to CN, SRN, SN and PN attributes under above DEVIP class. - Index on Slot=0. - Priority2: - If “ADD UNODEBESN” with “NODEBID” attribute matching with “NODEBID” attribute under “ADD ADJNODE” tag is available in logical dumps: - Get “NBLB1” attribute under “ADD UNODEBESN”. - Find Board with “SerialNumber”= “NBLB1” under “Board” table in physical dumps. - Priority 3 for NodeB3812E - Take the value of NODEBID under ADD ADJNODE tag - Match it with NODEBID under ADD UNODEB and take the value of NODENAME - Take the value of PEERIPADDR under ADD IPPATH tag - Search within the logical dumps of NodeB3812E for a node that has the name of NODENAME under NODEBSRCID - Search in it for the IP of PEERIPAADDR in dwIp under ETHIP classes (the IP is in decimal in dumps and should be converted to IP dotted decimal format) - Cabinet index= bCabinetNo under ETHIP class - Shelf index= bSubrack under ETHIP class - Slot index= bIubBoardNo under ETHIP class - Index on slot= bSubBoardNo under ETHIP class - Port index= bPortNo under ETHIP class - Priority 4 - Find in NodeB physical dump the board with “BoardType” attribute contains “MPT” or “NUTI” or “BBU” or “WMBB”. - NodeB Cabinet, Shelf Slot and Index on Slot are equal to Cabinet, Shelf, Slot and index on Slot of obtained board. - The port index equals “0” in case of “MPT”, “8” in case of “NUTI” or “BBU” and 3 in case of “WMBB”. |
| NodeB IP | [Optional] | - =”PEERIPADDR” under “ADD IPPATH” |
| NodeB Subnet Mask | [Optional] | - If matching NodeBCFG or CFGDATA files is successful then: - = MASK under attributes under DEVIP under class in NodeB dump - Else if for NodeB3812E - From the ETHIP class that was used to deduce C/SH/S/I/P - Take the value of dwMask after converting it to IP dotted decimal format - Else: - If PEERMASK under ADD IPPATH is different than 255.255.255.255 then Mask equals PEERMASK under ADD IPPATH. - Else, equals 255.255.255.240. |
| NodeB Gateway | [Optional] | - Priority 1: - Match NodeB name with NODEBFUNCTIONNAME under attributes under NODEBFUNCTION under class in NodeBCFG or CFGDATA files. or Match NodeB name with “NAME” attribute under the class that has the type “NE” in 2nd Dump structure for NodeB - Find all IPRT classes. - From DSTIP and DSTMASK under attributes under IPRT deduce the range of IPS for each IPRT class. - Get IPADDR under ADD IPPATH in the RNC file. - Deduce IPRT classes with IPADDR is within its range of IPs. - If multiple IPRT classes were found: - Deduce from NodeB IP and NodeB mask the range of IPs. - Select IPRT class with NEXTHOP under attributes under IPRT is in the range of IPs deduced from NodeB IP and mask. - Gateway= NEXTHOP under attributes under IPRT. - Priority 2 for NodeB3812E - From NodeB3812E logical dump of the concerned NodeB - From IPROUTE classes - Deduce the range of IP for each class using dwDestIP and dwDestIPMask attribute - The values are in decimals and should be converted to IP dotted decimal format - Deduce a range with the IP of IPPADDR under ADD IP PATH included - Take the value of dwNextHopIP after converting to dotted decimal - Else: - Else= from the “IPRT” class that has the attribute “DSTIP” equals “0.0.0.0” take the value of “NEXTHOP” attribute |
| NodeB Vlan | [Optional] | - Case 1: - Deduced from NodeBCFG or CFGDATA file or 2nd NodeB dump Structure - Match NEXTHOPIP under attributes under VLANMAP class with above NEXTHOP under attributes under IPRT. - = VLANID under attributes under VLANMAP - If not found - Take the value of “VLANID” under “GTRANSPARA” class - Case 2: For the NodeB of type BTS3812E_UMTS: - From the ADD IPPATH that was deduced earlier in RNC dump - Take the value of IPADDR attribute and convert it to decimal form, ex: 10.3.13.137 is equal to 167972233 - Match it with dwRncIp attribute under IPPATH class in NodeB dump - Take the value of bTrafficType attribute - Match it with bTrafficType under VLANMAP class - Take the value of wVlanId attribute as the VLAN - Multiple VLANMAP classes could be found. Take the class with the smallest value of the attribute bPriority |
| RNC C/SH/S/P | [Optional] | - To be deduced from both Logical and Physical Dumps. - Find “ADD IPPATH” tag with “ANI” value matching with “ANI” value under “ADD ADJNODE” tag in logical dumps. - If IPADDR under ADD IPPATH is matching with IPADDR under any ADD ETHIP then: - Shelf, slot and port are equal to SRN, SN and PN under ADD ETHIP respectively. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD ETHIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD ETHIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Else: - Find “ADD DEVIP” with “IPADDR” attribute matching with “IPADDR” attribute under “ADD IPPATH” tag. - For AIM dumps: - Get “SRN” and “SN” attributes under “ADD ETHIP” - Slot= SN attribute - Subrack = SRN - take the value of SRN attribute under ADD ETHIP and match it with SubrackNo under Subrack in RNC physical dump and get the value of CabinetNo - Cabinet id= CabinetNo - For DIM dumps: - Take the value of SRN under ADD ETHIP - Take the value of SYSOBJECTID under SET SYS - Match it with NEName in physical dumps (frame dumps) - Match SRN with FrameNo - Cabinet index= RackNo - Shelf index= InventoryUnitId - Port Index Deduction: - Match the "SRN" and "SN" under "ADD DEVIP" tag with the "SRN" and "SN" under "ADD ETHIP". - From IPADDR and MASK attributes under ADD ETHIP find the range of valid Connected IPs for each "ADD ETHIP". - Get PEERIPADDR attribute under “ADD IPPATH”. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD DEVIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD DEVIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Select among above deduced ADD ETHIP the ones with NEXTHOP under ADD IPRT is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "ADD ETHIP". - Sub-Interface ID = IPINDEX under "ADD ETHIP". - Multiple ports might be found. |
| RNC IP | [Optional] | - =“IPADDR” under “ADD ETHIP”. |
| RNC Subnet Mask | [Optional] | - =”MASK” under “ADD ETHIP”. |
| RNC Gateway | [Optional] | - take value of “IPADDR=” under class “ADD DEVIP” - Then match value of “IPADDR” in dump with value of “PEERIPADDR=” under class “ADD IPPATH”. Where in ADD IPPATH, we are having “ITFT=IUB”. Except “ITFT=IUB”, please ignore . - Then match value of “PEERIPADDR=” with value of “DSTIP=” under class “ADD IPRT”. - Match available then take the value of “NEXTHOP=” against respective “ADD DEVIP” As per above rules, gateway should be “10.220.5.49” for ADD DEVIP “10.220.15.54”. |
| RNC Vlan | [Optional] | - Find ADD VLANID with IPADDR is matching with above deduced NEXTHOP under ADD IPRT. - VLANID under ADD VLANID. - Else, =Null. |

## “Iub” Over IP Interface (In Transmission Resource Pool Mode)

- Each Iub over IP interface is represented in logical dumps with one “ADD ADJNODE” tag where “NODET” equals “IUB”, “TRANST” equals “IP” and contains IPPOOLINDEX attribute.

- Each NodeB might have a CFG file deduced by matching NodeB name with NODEBFUNCTIONNAME under attributes under NODEBFUNCTION under class in NodeBCFG or CFGDATA files.

- Below are the attributes of an Iub over IP interface:

| Attributes | Importance | Example |
| --- | --- | --- |
| NodeB Name | [Mandatory] | - To be deduced from Logical and Physical dumps. - Get “NODEBID” attribute under each tag of “ADD ADJNODE” where “NODET” equals “IUB” and “TRANST” equals “IP” from Logical dumps - NodeB Name = Node Name of above deduced Node ID in physical configuration. |
| Interface Name | [Mandatory] | - =“NAME” attribute under tag of “ADD ADJNODE”. - Value refers to “NODEBID” on the same line. |
| Interface Type | [Mandatory] | - “Iub” |
| Interface Technology | [Mandatory] | - “IP” |
| RNC Name | [Mandatory] | - =Node Name |
| TX BW | [Optional] | - =”TXBW” attribute under “ADD ADJNODE” tag. |
| RX BW | [Optional] | - =”RXBW” attribute under “ADD ADJNODE” tag. |
| NodeB Gateway | [Optional] | - If NodeB has a CFG file: - Find all IPRT classes. - From DSTIP and DSTMASK under attributes under IPRT deduce the range of IPs for each IPRT class. - Get IPADDR under ADD IPPOOLIP in the RNC file deduced below. - Deduce IPRT classes with IPADDR is within its range of IPs. - If multiple IPRT classes were found: - Deduce from NodeB IP and NodeB mask the range of IPs. - Select IPRT class with NEXTHOP under attributes under IPRT is in the range of IPs deduced from NodeB IP and mask. - Gateway= NEXTHOP under attributes under IPRT. - Else=Null |
| NodeB IP | [Optional] | - If NodeB has a CFG file: - From IP and MASK under attributes under DEVIP deduce the range of IPs for each DEVIP class. - Deduce DEVIP classes with above deduced NodeB Gateway IPs is within its range of IPs. - NodeB IP = IP under attributes under DEVIP. - Else: - Find all ADD UNCP and ADD UCCP with NODEBID is matching with NODEBID under ADD UNODEB. - Find all ADD SCTPLNK with SCTPLNKID is matching with SCTPLNKID under ADD UNCP and ADD UCCP. - NodeB IP equals all ”PEERIP1” and ”PEERIP2” under all “ADD SCTPLNK”. - If value is 0.0.0.0 then ignore it. |
| NodeB Vlan | [Optional] | - If NodeB has a CFG file: - To be deduced from NodeB CFG file. - Match NEXTHOPIP under attributes under VLANMAP class with above NEXTHOP under attributes under IPRT. - = VLANID under attributes under VLANMAP - Else, =Null. |
| NodeB Subnet Mask | [Optional] | - If NodeB has a CFG file: - = MASK under DEVIP class deduced in NodeB IP. - Else: - = 255.255.255.240. |
| NodeB C/SH/S/I/P | [Optional] | - Priority1: - If NodeB has a CFG file. - C/Sh/S/P are equal to CN, SRN, SN and PN attributes under DEVIP class deduced in NodeB IP.. - Index on Slot=0. - Priority2: - If “ADD UNODEBESN” with “NODEBID”attribute matching with “NODEBID” attribute under “ADD ADJNODE” tag is available in logical dumps: - Get “NBLB1” attribute under “ADD UNODEBESN”. - Find Board with “SerialNumber”= “NBLB1” under “Board” table in physical dumps. - Priority3 - Find in NodeB physical dump the board with “BoardType” attribute contains “MPT” or “NUTI” or “BBU” or “WMBB”. - NodeB Cabinet, Shelf Slot and Index on Slot are equal to Cabinet, Shelf, Slot and index on Slot of obtained board. - The port index equals “0” in case of “MPT”, “8” in case of “NUTI” or “BBU” and 3 in case of “WMBB”. |
| RNC IP | [Optional] | - Match IPPOOLINDEX under ADD ADJNODE with IPPOOLINDEX under all ADD IPPOOLIP. - Match IPADDR under all ADD IPPOOLIP with SRCIPADDR under ADD SRCIPRT with IPTYPE=DEVIP. - Match SRN, SN and NEXTHOP under ADD SRCIPRT having IPTYPE=DEVIP with Shelf, Slot and Gateway of a node interface on the same RNC. - RNC IP= IP @ of above deduced node interface. |
| RNC Subnet Mask | [Optional] | - = Mask of above deduced node interface. |
| RNC Gateway | [Optional] | - = Gateway of above deduced node interface. |
| RNC Vlan | [Optional] | - = VLAN of above deduced node interface. |
| RNC C/SH/S/I/P | [Optional] | - Cabinet, Shelf, Slot, Index on Slot, Port and Sub-interface are deduced from above deduced node interface. |

## “Iub” Over IP Interface (CM Export)

- Each Iub over IP interface is represented in logical dumps with one “BSC6900UMTSADJNODE” class where the attributes of this class are as follows:

  - “NODET” equals “IUB”

  - “TRANST” equals “IP” or “ATM_IP”.

- Iub over IP interface has one or several IP paths represented each with one “BSC6900UMTSIPPATH” class where “ANI” attribute value is matching with “ANI” attribute value under “BSC6900UMTSADJNODE” class

- Below are the attributes of an Iub over IP interface:

| Attributes | Importance | Example |
| --- | --- | --- |
| NodeB Name | [Mandatory] | - Get “NODEBID” attribute under each “BSC6900UMTSADJNODE” where the attributes of this class are as follows: - “NODET” equals “IUB” - “TRANST” equals “IP” or “ATM_IP” from Logical dumps - Get “BSC6900UMTSNODEB” class with “NODEBID” attribute matching with “NODEBID” of “BSC6900UMTSADJNODE” class - NodeB Name = “NODEBNAME” attribute under the previous “BSC6900UMTSNODEB” class |
| Interface Name | [Mandatory] | - From “NAME” attribute under “BSC6900UMTSADJNODE” class. - take the value after “Label=” and before “,” - example <attr name="name">Label=U_N_0040AL_MM-MARAKIA, ANI=16</attr> interface name= U_N_0040AL_MM-MARAKIA |
| Interface Type | [Mandatory] | - “Iub” |
| Interface Technology | [Mandatory] | - “IP” |
| RNC Name | [Mandatory] | - =Node Name |
| TX BW | [Optional] | - TX BW=”TXBW” attribute under “BSC6900UMTSIPPATH” class - If multiple “BSC6900UMTSIPPATH” classes were found then consider the maximum ”TXBW” value. |
| RX BW | [Optional] | - TX BW=”RXBW” attribute under “BSC6900UMTSIPPATH” tag. - If multiple “BSC6900UMTSIPPATH” classes were found then consider the maximum ”RXBW” value. |
| NodeB C/SH/S/I/P | [Optional] | - Priority1: - Match NodeB name with “NAME” attribute under the class that contains “NE” keword in NodeBCFG or CFGDATA files. Or Match NodeB name with “NAME” attribute under the class that has the type “NE” in nodeB dumps - Match PEERIPADDR under BSC6900UMTSIPPATH with “IP” attribute under the classes that contain “DEVIP” keyword in NodeBCFG or CFGDATA file. - C/Sh/S/P are equal to CN, SRN, SN and PN attributes under above DEVIP class. - Index on Slot=0. - Priority2 - Find in NodeB physical dump the board with “BoardType” attribute contains “MPT” or “NUTI” or “BBU” or “WMBB”. - NodeB Cabinet, Shelf Slot and Index on Slot are equal to Cabinet, Shelf, Slot and index on Slot of obtained board. - The port index equals “0” in case of “MPT” , “8” in case of “NUTI” or “BBU” and 3 in case of “WMBB”. |
| NodeB IP | [Optional] | - =”PEERIPADDR” attribute under “BSC6900UMTSIPPATH” class |
| NodeB Subnet Mask | [Optional] | - If matching to NodeBCFG or CFGDATA files is successful then: - = MASK attribute under the above DEVIP class in NodeBCFG or CFGDATA file or 2nd dump structure - Else: - If PEERMASK under BSC6900UMTSIPPATH is different than 255.255.255.255 then Mask equals PEERMASK under BSC6900UMTSIPPATH. - Else, equals 255.255.255.248. |
| NodeB Gateway | [Optional] | - From NodeB dump - Find all classes with IPRT type - From DSTIP and DSTMASK attributes under IPRT deduce the range of IPs for each IPRT class. - Deduce IPRT classes with the RNC IP address is within its range of IPs. - If multiple IPRT classes were found: - Deduce from NodeB IP and NodeB mask the range of IPs. - Select IPRT class with NEXTHOP attribute under IPRT is in the range of IPs deduced from NodeB IP and mask. - Gateway= NEXTHOP attributes under IPRT. |
| NodeB Vlan | [Optional] | - from NodeB dump - Match NEXTHOPIP attribute under the class that has the type VLANMAP with above NEXTHOP attribute under IPRT. - = VLANID attribute under VLANMAP - Else, =Null |
| RNC C/SH/S/P | [Optional] | - To be deduced from both Logical and Physical Dumps. - Find “BSC6900UMTSIPPATH” class with “ANI” attribute value matching with “ANI” attribute value under “BSC6900UMTSADJNODE” class in logical dumps. - If “IPADDR” attribute under BSC6900UMTSIPPATH class is matching with IPADDR under any BSC6900UMTSETHIP class then: - Shelf, slot and port are equal to SRN, SN and PN attributes under BSC6900UMTSETHIP class respectively. - Find BSC6900UMTSIPRoute class: - Find BSC6900UMTSIPRoute tag with DSTIP attribute under BSC6900UMTSIPRoute is matching with PEERIPADDR under “BSC6900UMTSIPPATH” class and SRN, SN attributes under BSC6900UMTSIPRoute class are matching with SRN, SN attributes under BSC6900UMTSETHIP. - Else: - Find BSC6900UMTSIPRoute with SRN and SN are matching with BSC6900UMTSETHIP. - From DSTIP and DSTMASK under all above BSC6900UMTSIPRoute find the range of valid IPs for each BSC6900UMTSIPRoute and then get the BSC6900UMTSIPRoute with PEERIPADDR under “BSC6900UMTSIPPATH” is included in its range. - Else: - Find “BSC6900UMTSDEVIP” class with “IPADDR” attribute matching with “IPADDR” attribute under “BSC6900UMTSIPPATH” class. - Get “SRN” and “SN” attributes under “BSC6900UMTSDEVIP” - Find Board with “FrameNo”= “SRN” and “SlotNo”= “SN” under “Board” table in physical dumps. - RNC Cabinet, Shelf and Slot are equal to Cabinet, Shelf and Slot of obtained board. - Port Index Deduction: - Match the "SRN" and "SN" under all "BSC6900UMTSDEVIP " classes with the "SRN" and "SN" under "BSC6900UMTSETHIP". - From IPADDR and MASK attributes under BSC6900UMTSETHIP find the range of valid Connected IPs for each "BSC6900UMTSETHIP ". - Get PEERIPADDR attribute under “BSC6900UMTSIPPATH”. - Find BSC6900UMTSIPRoute class: - Find BSC6900UMTSIPRoute class with DSTIP attribute value is matching with PEERIPADDR under “BSC6900UMTSIPPATH” and SRN, SN under BSC6900UMTSIPRoute are matching with SRN, SN under BSC6900UMTSDEVIP. - Else: - Find BSC6900UMTSIPRoute with SRN and SN are matching with BSC6900UMTSDEVIP. - From DSTIP and DSTMASK under all above BSC6900UMTSIPRoute classes find the range of valid IPs for each BSC6900UMTSIPRoute and then get the BSC6900UMTSIPRoute with PEERIPADDR under “BSC6900UMTSIPPATH” is included in its range. - Select among above deduced BSC6900UMTSETHIP the ones with NEXTHOP attribute is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "BSC6900UMTSETHIP". - Sub-Interface ID = IPINDEX under "BSC6900UMTSETHIP". - Multiple ports might be found. |
| RNC IP | [Optional] | - =“IPADDR” under “BSC6900UMTSETHIP”. |
| RNC Subnet Mask | [Optional] | - =”MASK” under “BSC6900UMTSETHIP”. |
| RNC Gateway | [Optional] | - = NEXTHOP attribute under above BSC6900UMTSIPRoute class |
| RNC Vlan | [Optional] | - =null |

## “Iur” Over ATM Interface

- Multiple Iur interfaces might be configured under one RNC.

- Each Iur over ATM interface is represented in logical dumps with one “ADD ADJNODE” tag where “NODET” equals “IUR” and “TRANST” equals “ATM”.

- Iur over ATM interface has one or several AAL2 paths represented each with one “ADD AAL2PATH” tag where “ANI” value is matching with “ANI” under “ADD ADJNODE” tag.

- Below are the attributes of an Iur over ATM interface:

| Attributes | Importance | Example |
| --- | --- | --- |
| Participant2 Name | [Mandatory] | - To be deduced from Logical and Physical dumps. - Get “RNCID” attribute under tag of “ADD URNCBASIC” from Logical dump. - Participant1 Name=RNC Name of above deduced RNC ID in Physical configuration. |
| Participant2 Node Type | [Optional] | - =RNC |
| Interface Name | [Mandatory] | - =“NAME” attribute under tag of “ADD ADJNODE” where “NODET” = “IUR” and “TRANST”= “ATM”. |
| Interface Type | [Mandatory] | - “Iur”. |
| Interface Technology | [Mandatory] | - “ATM” |
| Participant1 Name | [Mandatory] | - To be deduced from Logical and Physical dumps. - Participant1 RNC ID equals “NRNCID” under “ADD UNRNC” tag where “DPX” attribute under “ADD UNRNC” is matching with “DPX” attribute under “ADD ADJNODE”. - Participant1 RNC Name=RNC Name of above deduced RNC ID in physical configuration. |
| Participant1 Node Type | [Optional] | - =RNC |
| TX BW | [Optional] | - Find “ADD ATMTRF” tag with “TRFX” attribute matching with “TXTRFX” attribute under “ADD AAL2PATH”. - If “UT” attribute under “ADD ATMTRF” tag is equal to “KBIT/S” then TX Peak Rate is equal to “PCR” attribute under “ADD ATMTRF”. - Else, if “UT” attribute under “ADD ATMTRF” tag is equal to “CELL/S” then TX Peak Rate is equal to (“PCR”*53*8/1000). - If multiple “ADD AAL2PATH” tags are found then take the maximum “PCR” value. |
| RX BW | [Optional] | - Find “ADD ATMTRF” tag with “TRFX” attribute matching with “RXTRFX” attribute under “ADD AAL2PATH”. - If “UT” attribute under “ADD ATMTRF” tag is equal to “KBIT/S” then RX Peak Rate is equal to “PCR” attribute under “ADD ATMTRF”. - Else, if “UT” attribute under “ADD ATMTRF” tag is equal to “CELL/S” then RX Peak Rate is equal to (“PCR”*53*8/1000). - If multiple “ADD AAL2PATH” tags are found then take the maximum “PCR” value. |
| Participant2 C/SH/S/P | [Optional] | - To be deduced from both Logical and Physical Dumps. - Get “CARRYF” and “CARRYSN” attributes under “ADD AAL2PATH”. - Find Board with “FrameNo”= “CARRYF” and “SlotNo”= “CARRYSN” under “Board” table in physical dumps. - Participant2 Cabinet, Shelf and Slot are equal to Cabinet, Shelf and Slot of obtained board. - Port = “CARRYNCOPTN” attribute under “ADD AAL2PATH”. - If multiple “ADD AAL2PATH” tags are found then repeat the above for all of them, if the same port is found then take it only one time, if different ports are found then take each port one time only. |
| Participant1 C/SH/S/P | [Optional] | - Find RNC logical file with “RNCID” attribute under “ADD URNCBASIC” is matching with “NRNCID” attribute under “ADD UNRNC” in Participant2 RNC. - Find Add “ADD UNRNC” in Participant1 file where “NRNCID” attribute is matching with Participant2 RNC ID. - Find “ADD ADJNODE” with “DPX” attribute matching with “DPX” attribute under “ADD UNRNC”. - Find “ADD AAL2PATH” with “ANI” attribute matching with “ANI” attribute under “ADD ADJNODE”. - Get “CARRYF” and “CARRYSN” under “ADD AAL2PATH”, find correspondent board in physical dumps and deduce Cabinet, Shelf and Slot. - Port = “CARRYNCOPTN” attribute under “ADD AAL2PATH” in Participant1 file. - If multiple “ADD AAL2PATH” tags are found then repeat the above for all of them, if the same port is found then take it only one time, if different ports are found then take each port one time only. |

## “Iur” Over IP Interface

- Multiple Iur interfaces might be configured under one RNC.

- Each Iur over IP interface is represented in logical dumps with one “ADD ADJNODE” tag where “NODET” equals “IUR” and “TRANST” equals “IP”.

- Iur over IP interface has one or several IP paths represented each with one “ADD IPPATH” tag where “ANI” value is matching with “ANI” under “ADD ADJNODE” tag.

- Below are the attributes of an Iur over IP interface:

| Attributes | Importance | Example |
| --- | --- | --- |
| Source RNC Name | [Mandatory] | - = SYSOBJECTID under SET SYS |
| Source RNC Type | [Optional] | - =RNC |
| Interface Name | [Mandatory] | - “Source RNC Name” &”_” & “Destination RNC Name” |
| Interface Type | [Mandatory] | - “Iur”. |
| Interface Technology | [Mandatory] | - “IP” |
| Destination RNC Name | [Mandatory] | - Get “NRNCID” under “ADD UNRNC” tag where “DPX” attribute under “ADD UNRNC” is matching with “DPX” attribute under “ADD ADJNODE”. - Find RNC with RNCID under ADD URNCBASIC is matching with above “NRNCID” - Destination RNC Name=RNC Name of above deduced RNC. |
| Destination RNC Type | [Optional] | - =RNC |
| Source RNC C/SH/S/P | [Optional] | - To be deduced from both Logical and Physical Dumps. - Get “IPADDR” value under “ADD IPPATH”. - If IPADDR under ADD IPPATH is matching with IPADDR under any ADD ETHIP then: - Shelf, slot and port are equal to SRN, SN and PN under ADD ETHIP respectively. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD ETHIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD ETHIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Else: - Find “ADD DEVIP” where “IPADDR” attribute is matching with “IPADDR” value under “ADD IPPATH”. - Get “SRN” and “SN” attributes under “ADD DEVIP” - Find Board with “FrameNo”= “SRN” and “SlotNo”= “SN” under “Board” table in physical dumps. - Participant1 Cabinet, Shelf, Slot and equal to Cabinet, Shelf and Slot of obtained board. - Index on Slot=0 - Port Index Deduction: - Match the "SRN" and "SN" under "ADD DEVIP" tag with the "SRN" and "SN" under "ADD ETHIP". - From IPADDR and MASK attributes under ADD ETHIP find the range of valid Connected IPs for each "ADD ETHIP". - Get PEERIPADDR attribute under “ADD IPPATH”. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD DEVIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD DEVIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Get NEXTHOP attribute under ADD IPRT tag. - Select among above deduced ADD ETHIP the ones with NEXTHOP under ADD IPRT is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "ADD ETHIP". - Sub-Interface ID = IPINDEX under "ADD ETHIP". - Multiple ports might be found. |
| Destination RNC C/SH/S/P | [Optional] | - Find “ADD UNRNC” in Destination RNC Dump where “NRNCID” attribute is matching with RNCID under ADD URNCBASIC in source RNC. - Find “ADD ADJNODE” with “DPX” attribute matching with “DPX” attribute under “ADD UNRNC”. - Find “ADD IPPATH” with “ANI” attribute matching with “ANI” attribute under “ADD ADJNODE”. - Same procedure done for source RNC. |
| Source RNC IP Address | [Optional] | - =”IPADDR” value under “ADD ETHIP” tag in source RNC dump. |
| Source RNC Subnet Mask | [Optional] | - =”MASK” value under “ADD ETHIP” tag in source RNC dump. |
| Destination RNC IP Address | [Optional] | - =”IPADDR” value under “ADD ETHIP” tag in destination RNC dump. |
| Destination RNC Subnet Mask | [Optional] | - =”MASK” value under “ADD ETHIP” tag in destination RNC dump. |
| TX BW | [Optional] | - =”TXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found then consider the maximum TXBW value. |
| RX BW | [Optional] | - =”RXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found then consider the maximum RXBW value. |
| Source RNC Gateway | [Optional] | - = NEXTHOP attribute under above ADD IPRT tag. |
| Source RNC Vlan |  | - Find ADD VLANID with IPADDR is matching with above deduced Source RNC NEXTHOP under ADD IPRT. - VLANID under ADD VLANID. - Else, =Null. |
| Destination RNC Gateway | [Optional] | - = NEXTHOP attribute under above ADD IPRT tag. |
| Destination RNC Vlan | [Optional] | - Find ADD VLANID with IPADDR is matching with above deduced Destination RNC NEXTHOP under ADD IPRT. - VLANID under ADD VLANID. - Else, =Null. |

## “IuCS-U” Over IP Interface (Huawei MGW)

- “IuCS-U” over IP interface is represented in logical dumps with one ADD ADJNODE tag where NODET=IUCS and TRANST=IP.

- ADD ADJNODE has one or several ADD IPPATH where ANI under ADD ADJNODE and under ADD IPPATH are matching.

- Below are the attributes of an “IuCS-U” over IP interface:

| Attributes | Importance | Parsing |
| --- | --- | --- |
| RNC Name | [Mandatory] | - To be deduced from Logical and Physical dumps. - Get “RNCID” attribute under tag of “ADD URNCBASIC” from Logical dump. - Participant1 Name=RNC Name of above deduced RNC ID in Physical configuration. |
| MGW Name | [Mandatory] | - Get PEERIPADDR attribute under ADD IPPATH in RNC. - PEERIPADDR is constituted of 4 numbers separated with dots. - Get the first 3 numbers from the left (e.g. PEERIPADDR="10.192.92.0" → “10.192.92”). - Find MGW with IPADDR attribute under ADD IPADDR starting with same 3 numbers deduced above(e.g. IPADDR="10.192.92.18") - MGW Name is the name of above found MGW. |
| Interface Name | [Mandatory] | - =NAME attribute under “ADD ADJNODE”. |
| Interface Type | [Mandatory] | - “IuCS-U”. |
| Interface Technology | [Mandatory] | - “IP” |
| RNC C/SH/S/I/P | [Optional] | - To be deduced from both Logical and Physical Dumps. - Get “IPADDR” value under “ADD IPPATH”. - If IPADDR under ADD IPPATH is matching with IPADDR under any ADD ETHIP then: - Shelf, slot and port are equal to SRN, SN and PN under ADD ETHIP respectively. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD ETHIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD ETHIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Else: - Find “ADD DEVIP” where “IPADDR” attribute is matching with “IPADDR” value under “ADD IPPATH”. - Get “SRN” and “SN” attributes under “ADD DEVIP” - Find Board with “FrameNo”= “SRN” and “SlotNo”= “SN” under “Board” table in physical dumps. - Participant1 Cabinet, Shelf, Slot and equal to Cabinet, Shelf and Slot of obtained board. - Index on Slot=0 - Port Index Deduction: - Match the "SRN" and "SN" under "ADD DEVIP" tag with the "SRN" and "SN" under "ADD ETHIP". - From IPADDR and MASK attributes under ADD ETHIP find the range of valid Connected IPs for each "ADD ETHIP". - Get PEERIPADDR attribute under “ADD IPPATH”. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD DEVIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD DEVIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Get NEXTHOP attribute under ADD IPRT tag. - Select among above deduced ADD ETHIP the ones with NEXTHOP under ADD IPRT is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "ADD ETHIP". - Sub-Interface ID = IPINDEX under "ADD ETHIP". - Multiple ports might be found. |
| RNC IP Address | [Optional] | - =”IPADDR” value under “ADD ETHIP” tag. |
| RNC Subnet Mask | [Optional] | - =”MASK” value under “ADD ETHIP” tag. |
| MGW C/SH/S/I/P | [Optional] | - To be deduced from MGW. |
| MGW IP Address | [Optional] | - To be deduced from MGW. |
| MGW Subnet Mask | [Optional] | - To be deduced from MGW. |
| TX BW | [Optional] | - =”TXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found then consider the maximum TXBW value. |
| RX BW | [Optional] | - =”RXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found then consider the maximum RXBW value. |
| RNC Gateway | [Optional] | - = NEXTHOP attribute under above ADD IPRT tag. |
| RNC Vlan |  | - Find ADD VLANID with IPADDR is matching with above deduced Destination RNC NEXTHOP under ADD IPRT. - VLANID under ADD VLANID. - Else, =Null. |
| MGW Gateway | [Optional] | - To be deduced from MGW. |
| MGW Vlan | [Optional] | - To be deduced from MGW. |

## “IuCS-C” Over IP Interface (Huawei MGW)

- ADD N7DPC tag where DPCT=IUCS represents one or multiple “IUCS-C” interface.

- ADD N7DPC has one or several ADD M3DE where DPX under ADD N7DPC and under ADD M3DE are matching.

- ADD M3DE has one or several ADD M3LKS where DENO under ADD M3DE and under ADD M3LKS are matching.

- ADD M3LKS has one or several ADD M3LNK where SIGLKSX under ADD M3LKS and under ADD M3LNK are matching.

- ADD M3LNK has one or several ADD SCTPLNK where SRN, SN, SCTPLNKN under ADD M3LNK and under ADD SCTPLNK are matching.

- Below are the attributes of an “IUCS-C” over IP interface:

| Attributes | Importance | Parsing |
| --- | --- | --- |
| RNC Name | [Mandatory] | - =Node Name |
| MGW Name | [Optional] | - Find ADD N7DPC in RNC where DPCT=IUCS. - Get DPC under above deduced ADD N7DPC and convert it to Hexadecimal. - Find ADD OPC in RNC where SPX under ADD OPC and under ADD N7DPC are matching. - Get SPC under above deduced ADD OPC. - Convert above SPC from decimal to Hexadecimal. - Connected MGW should fulfil below 2 conditions: - MGW should have ADD M3LE where OPC (ignore H’) under ADD M3LE in MGW is matching with DPC (in Hex) under ADD N7DPC (DPCT=IUCS) in RNC (e.g. DPC = 1055 → OPC=H'41f). - MGW should have ADD M3DE where DPC under ADD M3DE in MGW is matching with SPC (in Decimal) under ADD OPC in RNC (e.g. SPC=1822→ OPC=H' 71e). - MGW Name is the name of above found MGW. |
| Interface Name | [Mandatory] | - =”RNC Name”_”MGW Name” |
| Interface Type | [Mandatory] | - “IUCS-C”. |
| Interface Technology | [Mandatory] | - “IP” |
| OPCs | [Optional] | - Find ADD OPC in RNC where SPX under ADD OPC and under ADD N7DPC are matching. - = SPC under ADD OPC. - The value is in decimal. |
| DPCs | [Optional] | - To be deduced from MGW. |
| RNC C/SH/S/I/P | [Optional] | - To be deduced from both Logical and Physical Dumps. - Get “LOCIP1” and LOCIP2 values under “ADD SCTPLNK”. - If LOCIP1 or LOCIP2 under ADD SCTPLNK is matching with IPADDR under any ADD ETHIP then: - Shelf, slot and port are equal to SRN, SN and PN under ADD ETHIP respectively. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD ETHIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD ETHIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” is included in its range. - Else: - Find “ADD DEVIP” where “IPADDR” attribute is matching with “LOCIP1” or LOCIP2 value under “ADD SCTPLNK”. - Get “SRN” and “SN” attributes under “ADD DEVIP” - Find Board with “FrameNo”= “SRN” and “SlotNo”= “SN” under “Board” table in physical dumps. - Participant1 Cabinet, Shelf, Slot and equal to Cabinet, Shelf and Slot of obtained board. - Index on Slot=0 - Port Index Deduction: - Match the "SRN" and "SN" under "ADD DEVIP" tag with the "SRN" and "SN" under "ADD ETHIP". - From IPADDR and MASK attributes under ADD ETHIP find the range of valid Connected IPs for each "ADD ETHIP". - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD DEVIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD DEVIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” is included in its range. - Get NEXTHOP attribute under ADD IPRT tag. - Select among above deduced ADD ETHIP the ones with NEXTHOP under ADD IPRT is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "ADD ETHIP". - Sub-Interface ID = IPINDEX under "ADD ETHIP". - Multiple ports might be found. |
| RNC IP Address | [Optional] | - =”IPADDR” value under “ADD ETHIP” tag. |
| RNC Subnet Mask | [Optional] | - =”MASK” value under “ADD ETHIP” tag. |
| MGW C/SH/S/I/P | [Optional] | - To be deduced from MGW. |
| MGW IP Address | [Optional] | - To be deduced from MGW. |
| MGW Subnet Mask | [Optional] | - To be deduced from MGW. |
| RNC Gateway | [Optional] | - = NEXTHOP attribute under above ADD IPRT tag. |
| RNC Vlan |  | - Find ADD VLANID with IPADDR is matching with above deduced Destination RNC NEXTHOP under ADD IPRT. - VLANID under ADD VLANID. - Else, =Null. |
| MGW Gateway | [Optional] | - To be deduced from MGW. |
| MGW Vlan | [Optional] | - To be deduced from MGW. |

## “IuCS-U” Over IP Interface (NSN MGW)

- “IuCS-U” over IP interface is represented in logical dumps with one ADD ADJNODE tag where NODET=IUCS and TRANST=IP.

- ADD ADJNODE has one or several ADD IPPATH where ANI under ADD ADJNODE and under ADD IPPATH are matching.

- Below are the attributes of an “IuCS-U” over IP interface:

| Attributes | Importance | Parsing |
| --- | --- | --- |
| RNC Name | [Mandatory] | - Node Name |
| MGW Name | [Mandatory] | - Get PEERIPADDR attribute under ADD IPPATH in RNC. - PEERIPADDR is constituted of 4 numbers separated with dots. - Get the first 3 numbers from the left (e.g. PEERIPADDR="10.192.92.0" → “10.192.92”). - Find NSN MGW with addrValueIPV4 under item under list under IEIF class starting with same 3 numbers deduced above(e.g. IPADDR="10.192.92.18") - MGW Name is the name of above found MGW. |
| Interface Name | [Mandatory] | - =”Source Name ”_”Destination Name” |
| Interface Type | [Mandatory] | - “IuCS-U”. |
| Interface Technology | [Mandatory] | - “IP” |
| RNC C/SH/S/I/P | [Optional] | - To be deduced from both Logical and Physical Dumps. - Get “IPADDR” value under “ADD IPPATH”. - Get “IPADDR” value under “ADD IPPATH”. - If IPADDR under ADD IPPATH is matching with IPADDR under any ADD ETHIP then: - Shelf, slot and port are equal to SRN, SN and PN under ADD ETHIP respectively. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD ETHIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD ETHIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Else: - Find “ADD DEVIP” where “IPADDR” attribute is matching with “IPADDR” value under “ADD IPPATH”. - Get “SRN” and “SN” attributes under “ADD DEVIP” - Find Board with “FrameNo”= “SRN” and “SlotNo”= “SN” under “Board” table in physical dumps. - Participant1 Cabinet, Shelf, Slot and are equal to Cabinet, Shelf and Slot of obtained board. - Index on Slot=0. - Port Index Deduction: - Match the "SRN" and "SN" under "ADD DEVIP" tag with the "SRN" and "SN" under "ADD ETHIP". - Find the range of valid Connected IPs for each "ADD ETHIP” from IPADDR and MASK attributes under ADD ETHIP - Get PEERIPADDR attribute under “ADD IPPATH”. - Find the range of valid Connected IPs for each ADD IPRT from DSTIP and DSTMASK attributes under ADD IPRT. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD DEVIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD DEVIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Get NEXTHOP attribute under ADD IPRT tag. - Select among above deduced ADD ETHIP the ones with NEXTHOP under ADD IPRT is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "ADD ETHIP". - Sub-Interface ID = IPINDEX under "ADD ETHIP". - Multiple ports might be found. |
| RNC IP Address | [Optional] | - =”IPADDR” value under “ADD ETHIP” tag. |
| RNC Subnet Mask | [Optional] | - =”MASK” value under “ADD ETHIP” tag. |
| MGW C/SH/S/I/P | [Optional] | - To be deduced from MGW. |
| MGW IP Address | [Optional] | - To be deduced from MGW. |
| MGW Subnet Mask | [Optional] | - To be deduced from MGW. |
| TX BW | [Optional] | - =”TXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found then consider the maximum TXBW value. |
| RX BW | [Optional] | - =”RXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found then consider the maximum RXBW value. |
| RNC Gateway | [Optional] | - = NEXTHOP attribute under above ADD IPRT tag. |
| RNC Vlan |  | - Find ADD VLANID with IPADDR is matching with above deduced NEXTHOP under ADD IPRT. - VLANID under ADD VLANID. |
| MGW Gateway | [Optional] | - To be deduced from MGW. |
| MGW Vlan | [Optional] | - To be deduced from MGW. |

## “IuCS-C” Over IP Interface (NSN MGW)

- ADD N7DPC tag where DPCT=IUCS represents one or multiple “IUCS-C” interface.

- ADD N7DPC has one or several ADD M3DE where DPX under ADD N7DPC and under ADD M3DE are matching.

- ADD M3DE has one or several ADD M3LKS where DENO under ADD M3DE and under ADD M3LKS are matching.

- ADD M3LKS has one or several ADD M3LNK where SIGLKSX under ADD M3LKS and under ADD M3LNK are matching.

- ADD M3LNK has one or several ADD SCTPLNK where SRN, SN, SCTPLNKN under ADD M3LNK and under ADD SCTPLNK are matching.

- Below are the attributes of an “IUCS-C” over IP interface:

| Attributes | Importance | Parsing |
| --- | --- | --- |
| RNC Name | [Mandatory] | - =Node Name |
| MGW Name | [Optional] | - Find ADD N7DPC in RNC where DPCT=IUCS. - Get DPC under above deduced ADD N7DPC and convert it to Hexadecimal. - Find ADD OPC in RNC where SPX under ADD OPC and under ADD N7DPC are matching. - Get SPC under above deduced ADD OPC. - Connected MGW should fulfil below 2 conditions: - MGW should have OSP class where signallingPointCodeDecimal under OSP class in MGW is matching with DPC under ADD N7DPC (DPCT=IUCS) in RNC (e.g. DPC=15757 → signallingPointCodeDecimal">15757</p>). - MGW should have DSP where signallingPointCodeDecimal under DSP in MGW is matching with SPC under ADD OPC in RNC (e.g. SPC=15771 → signallingPointCodeDecimal">15771</p>). - MGW Name is the name of above found MGW. |
| Interface Name | [Mandatory] | - =”RNC Name”_”MGW Name” |
| Interface Type | [Mandatory] | - “IUCS-C”. |
| Interface Technology | [Mandatory] | - “IP” |
| OPCs | [Optional] | - Find ADD OPC in RNC where SPX under ADD OPC and under ADD N7DPC are matching. - = SPC under ADD OPC. - The value is in decimal. |
| DPCs | [Optional] | - To be deduced from MGW. |
| RNC C/SH/S/I/P | [Optional] | - To be deduced from both Logical and Physical Dumps. - Get “LOCIP1” and LOCIP2 values under “ADD SCTPLNK”. - If LOCIP1 or LOCIP2 under ADD SCTPLNK is matching with IPADDR under any ADD ETHIP then: - Shelf, slot and port are equal to SRN, SN and PN under ADD ETHIP respectively. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD ETHIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD ETHIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” is included in its range. - Else: - Find “ADD DEVIP” where “IPADDR” attribute is matching with “LOCIP1” or LOCIP2 value under “ADD SCTPLNK”. - Get “SRN” and “SN” attributes under “ADD DEVIP” - Find Board with “FrameNo”= “SRN” and “SlotNo”= “SN” under “Board” table in physical dumps. - Participant1 Cabinet, Shelf, Slot are equal to Cabinet, Shelf and Slot of obtained board. - Index on Slot=0. - Port Index Deduction: - Match the "SRN" and "SN" under "ADD DEVIP" tag with the "SRN" and "SN" under "ADD ETHIP". - From IPADDR and MASK attributes under ADD ETHIP find the range of valid Connected IPs for each "ADD ETHIP". - Get PEERIP1 and PEERIP2 attributes under “ADD SCTPLNK”. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD DEVIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD DEVIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” is included in its range. - Get NEXTHOP attribute under ADD IPRT tag. - Select among above deduced ADD ETHIP the ones with NEXTHOP under ADD IPRT is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "ADD ETHIP". - Sub-Interface ID = IPINDEX under "ADD ETHIP". - Multiple ports might be found. |
| RNC IP Address | [Optional] | - =”IPADDR” value under “ADD ETHIP” tag. |
| RNC Subnet Mask | [Optional] | - =”MASK” value under “ADD ETHIP” tag. |
| MGW Interface Name | [Optional] | - To be deduced from MGW. |
| MGW IP Address | [Optional] | - To be deduced from MGW. |
| MGW Subnet Mask | [Optional] | - To be deduced from MGW. |
| RNC Gateway | [Optional] | - = NEXTHOP attribute under above ADD IPRT tag. |
| RNC Vlan |  | - Find ADD VLANID with IPADDR is matching with above deduced NEXTHOP under ADD IPRT. - VLANID under ADD VLANID. - Else, =Null. |
| MGW Gateway | [Optional] | - To be deduced from MGW. |
| MGW Vlan | [Optional] | - To be deduced from MGW. |

## Over IP Interface

- RNC might have multiple IuPS interfaces.

- Each IuPS over IP interface is represented in logical dumps with one “ADD N7DPC” tag where “DPCT” equals “IUPS” and “BEARTYPE” equals “M3UA”.

- “ADD N7DPC” has one “ADD ADJNODE” tag where “DPX” value is matching with “DPX” under “ADD N7DPC” tag.

- IuPS over IP interface has one or several IP paths represented each with one “ADD IPPATH” tag where “ANI” value is matching with “ANI” under “ADD ADJNODE” tag.

- Below are the attributes of an IuPS over IP interface:

| Attributes | Importance | Example |
| --- | --- | --- |
| RNC Name | [Mandatory] | - To be deduced from Logical and Physical dumps. - Get “RNCID” attribute under tag of “ADD URNCBASIC” from Logical dump. - Participant1 Name=RNC Name of above deduced RNC ID in Physical configuration. |
| Interface Name | [Mandatory] | - =”RNC Name”_”SGSN Name” |
| Interface Type | [Mandatory] | - “IuPS”. |
| Interface Technology | [Mandatory] | - “IP” |
| SGSN Name | [Mandatory] | - Convert “DPC” under “ADD N7DPC” tag to Hex and add “0x” prefix (e.g. DPC=1901 → 0x76d). - Find SGSN with “OPC” attribute under “ADD M3LE” is matching with above deduced DPC value. - = Name of above deduced Node. |
| TX BW | [Optional] | - =”TXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found then consider the maximum ”TXBW” value. |
| RX BW | [Optional] | - =”RXBW” attribute under “ADD IPPATH” tag. - If multiple “ADD IPPATH” were found then consider the maximum ”RXBW” value. |
| OPCs | [Optional] | - Deduced from SGSN. |
| DPCs | [Optional] | - Deduced from SGSN. |
| RNC C/SH/S/I/P | [Optional] | - 2 Methods to be followed consecutively to deduce the Participant2 C/SH/S/P: - Method1: - Find “ADD ADJNODE” tag where “DPX” under “ADD ADJNODE” and under “ADD N7DPC” are matching. - Find “ADD IPPATH” tag with “ANI” value matching with “ANI” value under “ADD ADJNODE” tag. - Get “IPADDR” value under “ADD IPPATH”. - If IPADDR under ADD IPPATH is matching with IPADDR under any ADD ETHIP then: - Shelf, slot and port are equal to SRN, SN and PN under ADD ETHIP respectively. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD ETHIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD ETHIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Else: - Find “ADD DEVIP” where “IPADDR” attribute is matching with “IPADDR” value under “ADD IPPATH”. - Get “SRN” and “SN” attributes under “ADD DEVIP” - Find Board with “FrameNo”= “SRN” and “SlotNo”= “SN” under “Board” table in physical dumps. - RNC Cabinet, Shelf and Slot are equal to Cabinet, Shelf and Slot of obtained board. - Port Index Deduction: - Match the "SRN" and "SN" under "ADD DEVIP" tag with the "SRN" and "SN" under "ADD ETHIP". - From IPADDR and MASK attributes under ADD ETHIP find the range of valid Connected IPs for each "ADD ETHIP". - Get PEERIPADDR attribute under “ADD IPPATH”. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIPADDR under “ADD IPPATH” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD DEVIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD DEVIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIPADDR under “ADD IPPATH” is included in its range. - Get NEXTHOP attribute under ADD IPRT tag. - Select among above deduced ADD ETHIP the ones with NEXTHOP under ADD IPRT is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "ADD ETHIP". - Multiple ports might be found. - Method2: - Find “ADD M3DE” tag where “DPX” under “ADD M3DE” and under “ADD N7DPC” are matching. - Find “ADD M3LKS” tag where “DENO” under “ADD M3LKS” and under “ADD M3DE” are matching. - Find “ADD M3LNK” tag where “SIGLKSX” under “ADD M3LNK” and under “ADD M3LKS” are matching. - Find “ADD SCTPLNK” tag where “SRN”, “SN” and “SCTPLNKN” under “ADD SCTPLNK” and under “ADD M3LNK” are matching. - Get “LOCIP1” and LOCIP2 values under “ADD SCTPLNK”. - If LOCIP1 or LOCIP2 under ADD SCTPLNK is matching with IPADDR under any ADD ETHIP then: - Shelf, slot and port are equal to SRN, SN and PN under ADD ETHIP respectively. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD ETHIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD ETHIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” is included in its range. - Else: - Find “ADD DEVIP” where “IPADDR” attribute is matching with “LOCIP1” or LOCIP2 values under “ADD SCTPLNK”. - Get “SRN” and “SN” attributes under “ADD DEVIP” - Find Board with “FrameNo”= “SRN” and “SlotNo”= “SN” under “Board” table in physical dumps. - RNC Cabinet, Shelf and Slot are equal to Cabinet, Shelf and Slot of obtained board. - Port Index Deduction: - Match the "SRN" and "SN" under "ADD DEVIP" tag with the "SRN" and "SN" under "ADD ETHIP". - From IPADDR and MASK attributes under ADD ETHIP find the range of valid Connected IPs for each "ADD ETHIP". - Get PEERIP1 and PEERIP2 attributes under “ADD SCTPLNK”. - Find ADD IPRT tag: - Find ADD IPRT tag with DSTIP under ADD IPRT is matching with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” and SRN, SN under ADD IPRT are matching with SRN, SN under ADD DEVIP. - Else: - Find ADD IPRT with SRN and SN are matching with ADD DEVIP. - From DSTIP and DSTMASK under all above ADD IPRT find the range of valid IPs for each ADD IPRT and then get the ADD IPRT with PEERIP1 or PEERIP2 attributes under “ADD SCTPLNK” is included in its range. - Get NEXTHOP attribute under ADD IPRT tag. - Select among above deduced ADD ETHIP the ones with NEXTHOP under ADD IPRT is included in the range of valid connected IPs. - Port Index = "PN" attribute under selected "ADD ETHIP". - Multiple ports might be found. - Sub-Interface ID = IPINDEX under "ADD ETHIP". |
| RNC IP | [Optional] | - =“IPADDR” under “ADD ETHIP”. |
| RNC Subnet Mask | [Optional] | - =”MASK” under “ADD ETHIP”. |
| SGSN C/SH/S/P | [Optional] | - To be deduced from SGSN. |
| SGSN IP | [Optional] | - To be deduced from SGSN. |
| SGSN Subnet Mask | [Optional] | - To be deduced from SGSN. |
| RNC Gateway | [Optional] | - Method1: - = NEXTHOP attribute under above ADD IPRT tag.. - Method2: - = NEXTHOP attribute under above ADD IPRT tag. |
| RNC Vlan |  | - Find ADD VLANID with IPADDR is matching with above deduced Destination RNC NEXTHOP under ADD IPRT. - VLANID under ADD VLANID. - Else, =Null. |
| SGSN Gateway | [Optional] | - To be deduced from SGSN. |
| SGSN Vlan | [Optional] | - To be deduced from SGSN. |

# Software Version

- Below additional info is to parse the node software version and backup software version for SRAN nodes. And with the same attribute we can also find out for NodeB/RNC.

- Add info need to be parsed from the physical dump of the SRAN nodes.

- Need to be consider for the dump which has all below 3 attributes.

<TABLE attrname="UCELL">

<TABLE attrname="GCELL">

<TABLE attrname="LCELL">

For Example:

<ROW HostVerType="NodeBFunctionHOSTVER" HostVer="BTS3900_5900 V100R012C10SPC280" sDesc="URAT SOFTWARE VERSION" />

SOFTWARE_VERSION_3G = BTS3900_5900 V100R012C10SPC280

_____________________________________________________________________________

<ROW HostVerType="NodeBFunctionBACKVER" HostVer="BTS3900 V100R011C10SPC210" sDesc="URAT BACK SOFTWARE VERSION" />

BACKUP_SOFTWARE_VERSION_3G: BTS3900 V100R011C10SPC210

| Attribute | Category | Value |
| --- | --- | --- |
| SOFTWARE_VERSION_3G | SOFTWARE_VERSION | Take the value of “HostVer=” attribute where HostVerType="NodeBFunctionHOSTVER" under class “<TABLE attrname="HostVer">” Example given above |
| BACKUP_SOFTWARE_VERSION_3G | BACKUP_SOFTWARE_VERSION | Take the value of “HostVer=” attribute where HostVerType="NodeBFunctionBACKVER" under class “<TABLE attrname="HostVer">” Example given above |