# Huawei RAN Connector FRS (Functional Requirement Specifications) Auto-Import connector

Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft | Indrasan Yadav | 05-05-2025 |

Table of Contents

Introduction	3

1	RAN Nodes	5

1.1	RAN Physical configuration	8

1.1.1	Cabinet	8

1.1.2	Shelf	9

1.1.3	Boards	11

1.2 Antenna…………………………………………………………………………………………………………………………13

# Introduction

- This FRS is intended for Huawei RAN Nodes extraction.

- It describes how to parse the nodes details and physical configuration.

- The dump files provided are of xml file per node.

- The folder location is to be précised later.

- The files format is as follow:

# RAN Nodes

- Only NEtype=”BSC6900GSM”, ”BSC6900UMTS”, "BSC6910GU", ”BSC6910GSM”, "BSC6910UMTS",  “eNodeB”, “MBTS”, ”GSMBTS”, “NodeB”, “BTS”, “BTS3900”, “BTS5900”, “BTS59005G”, “DBS3900IBS”, “MICROBTS3900” are to be considered during parsing.

- NEtype=BSC6900GU or BSC6910GU then node should be created under SRAN Controller.

- NEType= “BTS3900”/ “MICROBTS3900”/ “BTS5900”/ “MBTS”/ “BTS59005G”/ “DBS3900IBS” then the node is a SRAN.

- NEType= “BTS”/ “GSMBTS” then the node is a BTS.

- NEType= “NodeB” and “eNodeB” then the node is NodeB and eNodeB respectively.

- NEType= “BSC6900GSM”/ “BSC6910GSM” then the node is a BSC.

- NEType= “BSC6900UMTS”/ “BSC6910UMTS” then the node is an RNC.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE External Code | [Mandatory] | - NE ID |
| NE ID | [Mandatory] | - =NE name. |
| NE Name | [Mandatory] | NE Name will be extracted from attribute “NEName=”” in individual dump. |
| Site id | [Mandatory] | - Explain later as per requirement |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | To be deduced from node matching file |
| Model | [Mandatory] | - The value of “NEType=” in node dump |
| Manufacturer | [Mandatory] | - = Huawei |
| IP + subnet mask | [Optional] | - =NULL |
| Technology Supported | [Optional] | - First, the Technology supported will be deduced as follow through matching the “NEType” within the below table: NEType; Technology Supported. GSMBTS; 2G. BSC6900GSM; 2G. BSC6900UMTS; 3G. BSC6900GU; 2G/3G. BSC6910GSM; 2G. BSC6910UMTS; 3G. BSC6910GU; 2G/3G. NodeB; 3G. MBTS(GL); 2G/4G. MBTS(GU); 2G/3G. MBTS(UL); 3G/4G. MBTS(GUL); 2G/3G/4G. eNodeB; 4G. BTS; 2G. MBTSGU; 2G/3G. BSC6000BTS; 2G. BTS3900/BTS5900/BTS59005G; 2G/3G/4G. |
| Parent BSC | [Mandatory] | - The node (BTS) with SiteName= which is available in that BSC dump under attribute <TABLE attrname="BTS"> then that BSC will be parent BSC for all available node (BTS). Note: same rule will be applied also for if parent will be SRAN Controller. |
| Parent RNC | [Mandatory] | - The node (NodeB) with NodeBName= which is available in that RNC dump under attribute <TABLE attrname="UCELL"> then that RNC will be parent RNC for all available node (NodeB). Note: same rule will be applied also for if parent will be SRAN Controller. |
| Software version | [Optional] | - =Null |
| Additional Information | [Optional] | - =Null |

## RAN Physical configuration

### Cabinet

- Each XML file of each node contains information about cabinets. From this file we will be taking the data for the cabinet.

- each type has one attribute <TABLE attrname="Cabinet"> for the cabinet’s info.

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| NE External code | [Mandatory] | - Same as NE ID |
| Cabinet index | [Mandatory] | - The value of “CabinetNo” under attribute <TABLE attrname="Cabinet"> - Eg: CabinetNo="0" Cabinet index=0 - If “CabinetNo” is empty, then ignore the line. - Sometimes we can get more than one cabinet. Parse all of them. Note: If any node which contains blank physical (cabinet, shelf, board) information, then in that case only node will be parse & physical data (cabinet, shelf & boards & antenna) will not be parse. |
| Cabinet id | [Mandatory] | - = NE External code _cabinet index - E.g.: BARF011_0 (For above example) |
| Model | [Mandatory] | - The value of “RackType” attribute - Eg: RackType="VIRTUAL" VIRTUAL |
| Part Number | [optional] | - =Value of “BOMCode” attribute - Eg: BOMCode="01071087” |
| Date Of Manufacturer | [optional] | - =Value of “DateOfManufacture” attribute - Eg: DateOfManufacture="2011-03-10" DateOfManufacture=2011-03-10 |
| Description | [optional] | - =Value of “ManufacturerData” attribute - If empty = null |
| Serial Number | [optional] | - =Value of “SerialNumber” attribute - E.g.: SerialNumber="21010710876TB3000615 |
| Manufacturer | [Mandatory] | - =Huawei |
| Power Consumption | [optional] | - =null |
| Additional Information | [optional] | - =null |
| Dimensions and Weight | [optional] | - =null |

### Shelf

- Each row under attribute <TABLE attrname="Subrack"> shows shelf information.

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet Id | [Mandatory] | - = NE External code _cabinet index - E.g.: BARF011_0 |
| Shelf Id | [Mandatory] | - Concatenation between NEName, “_” and CabinetNo=, “_” and SubrackNo in related row under attribute <TABLE attrname="Subrack"> |
| Shelf index | [Mandatory] | - The value of “SubrackNo=” under attribute <TABLE attrname="Subrack"> - E.g.: SubrackNo="0" Shelf index=0 - If “SubrackNo” is empty, then ignore the line. - Sometimes we can get more than one subrack. Parse all of them. |
| RU Index | [Mandatory] | - To be followed static sheet “HW_MatchingFile” same as in OmanTel. |
| Part Number | [Optional] | - =Value of “BOMCode” attribute - Eg: BOMCode="01071087” |
| Model | [Mandatory] | - =”FrameType” attribute, e.g.: MPS or EPS. |
| Date Of Manufacture | [Optional] | - =Value of “DateOfManufacture” attribute - Eg: DateOfManufacture="2011-03-10" DateOfManufacture=2011-03-10 |
| Issue Number | [Optional] | - Take the Value of “IssueNumber=” under same attribute <TABLE attrname="Subrack"> for related shelf. - =Value of “IssueNumber” attribute Eg: IssueNumber="00” |
| Description | [Optional] | - =Value of “ManufacturerData” attribute - If empty = null |
| Serial Number | [Optional] | - =Value of “SerialNumber” attribute - E.g.: SerialNumber="21010710876TB3000615 |
| Power Consumption | [Optional] | - Null |
| Additional Information | [Optional] | - Null |
| Dimensions and Weight | [Optional] | - Null |

### Boards (Need to cross check again for slot no. & index on slot once full dump received & SFP will be parse as board or transceiver?)

- Each row under attribute <TABLE attrname="Board"> shows boards information.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - Concatenation between NEName, “_”, CabinetNo,” _” and SubrackNo |
| Shelf Index | [Mandatory] | - The value of “SubrackNo=” under attribute <TABLE attrname="Subrack"> - E.g.: SubrackNo="0" Shelf index=0 - If “SubrackNo” is empty, then ignore the line. - Sometimes we can get more than one cabinet. Parse all of them. |
| Index on slot | [Mandatory] | =0 By default |
| Board Id | [Mandatory] | - Concatenation between NEName, “_”, “CabinetNo”, “_”, SubrackNo,” _”, “SlotNo” and Index on slot. - E.g.: BARF011_0_0_6_0 |
| Slot Index | [Mandatory] | - =Value of “SlotNo” attribute in each Row. |
| Board Type | [Mandatory] | Take the value of “BoardName=” under attribute “<TABLE attrname="Board">” Note: The existing rule can be retained with the following data also: - We need to add BoardName="FModule" in the ‘npsboards’ section under the <TABLE attrname="Board"> class. - The “FModule” board will be parse as described below, it should be parsed as boards with the required cabinet, shelf, slot, and board index as specified in the dump. Sample Case: The snippet below illustrates an example: In this case, the “FModule” boards (count: 6) are present under cabinet-0 and are distributed across shelf-0, shelf-60, shelf-61 & shelf-62 occupying slots 0 and 6. All instances of “FModule” should be parsed in the XML as per the dump details. |
| Part Number | [Optional] | - =Value of “BOMCode” attribute - E.g.: BOMCode="03051122" Part Number= 03051122 |
| Description | [Optional] | =Value of “ManufacturerData” attribute E.g.: ManufacturerData="PARC WP11OIUa 1-port channelized Optical Interface Unit REV:a" then description will be = "PARC WP11OIUa 1-port channelized Optical Interface Unit REV:a" |
| Date Of Manufacture | [Optional] | - =Value of “DateOfManufacture” attribute Eg: DateOfManufacture="2011-05-07" DateOfManufacture=2011-05-07 |
| Serial number | [Optional] | - =Value of “SerialNumber” attribute E.g.: SerialNumber="210305112210B5000522" SerialNumber=210305112210B5000522 |
| Issue Number | [Optional] | - =Value of “IssueNumber” attribute Eg: IssueNumber="00" Issue Number=00 |
| Power Consumption | [Optional] | - =null |
| Additional Information | [Optional] | - =null |
| Dimensions and Weight | [Optional] | - =null |

## Antenna Parsing

Antenna details will be parse under table of “radioantenna” in XML.

Antenna Parsing rule described below:

| Attribute | Importance |  | Comments |  |
| --- | --- | --- | --- | --- |
| Antenna Id/Object ID | [Mandatory] |  | - =Concatenate of “NEName=” and the value of “InventoryUnitId” (“InventoryUnitId” will be available under attribute <TABLE attrname="Antenna"> |  |
| Name | [Mandatory] |  | - =Attribute “UnitPosition” take the value before the “,” indicator. - Example: UnitPosition="AntennaDeviceName=RET_UMTS-A,AntennaDeviceNo=1" Name = RIY2657-P1-HUB_SRAN-FON_1,RET_UMTS-A. |  |
| RadioantennaType | [Mandatory] |  | - =attribute “AntennaDeviceType=” - Example: AntennaDeviceType="RCU136" type =”RCU136” |  |
| VersionID | [Optional] |  | - Value of HardwareVersion, e.g if HardwareVersion="RF11CLPT_VER.D" then VersionId is RF11CLPT_VER.D |  |
| Band | [Optional] |  | - =attribute “ManufacturerData” |  |
| Sector | [Optional] |  |  |  |

Additional information: Serial number will be parse under addinfo in XML under attribute “Value”.

| Attribute | Importance | Comments |
| --- | --- | --- |
| SerialNumber_<serialnumber> | [Optional] | - =Value of “SerialNumberEx” attribute - Eg:SerialNumber="030LPM4MB5618639" SerialNumber=030LPM4MB5618639 |
| SerialNumber_<serialnumber> |  | - Value of “SerialNumber” attribute Eg:SerialNumber="030LPM4MB5618639" SerialNumber=030LPM4MB5618639 |

Note:

Current Implementation:

The system currently parses SerialNumberEx as the primary source for the antenna serial number.

Issue:

SerialNumberEx is often found to be empty in the data dump, which causes it to come blank for some Antenna nodes in the NEP report.

1. Objective

Ericsson antennas are connected to Huawei RAN. To correctly identify and import serial numbers (SN) without duplications in this unique scenario, the active assets include:

- Ericsson antennas connected to Huawei RAN.

- Serial Numbers are in either SerialNumberEx or AntennaSerialNumber attribute whereby SerialNumberEx is for Huawei and AntennaSerialNumber is for Ericsson antenna

3. Data Import Logic

To correctly import the active assets without duplications, we propose the following logic:

- Step 1: Check Column U (SerialNumberEx)) for SN

  - If Column U has a value, use it as the SN for Huawei antennas and proceed with the import. If it is empty Check AntennaSerialNumber

  - The value from Column U is imported only if it is unique

- Step 2: Check AntennaSerialNumber for Ericsson Antennas

# If it is empty, use the SN from SerialNumberEx and proceed with the import, ensuring no duplicates