| # NSN Physical Connector FRS (Functional Requirement Specifications) Auto-Import connector |  |
| --- | --- |
|  | Indrasan Yadav |
|  | 04-06-2025 |

Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft | Indrasan Yadav | 04-06-2025 |

Table of Contents

Introduction	4

1	EnodeB	5

1.1	Physical configuration	5

1.1.1	Cabinet	5

1.1.2	Shelf	5

1.1.3	Boards	6

1.1.3.1	Boards Slot Indexes:	7

2	BSC	20

2.1	Physical configuration	20

2.1.1	Cabinet	20

2.1.2	Shelf	21

2.1.3	Boards	21

3	BTS:	23

3.1	Physical configuration	23

3.1.1	Cabinet	23

3.1.2	Shelf	24

3.1.3	Boards	24

# Introduction

- This FRS is intended for NSN Nodes extraction for project Ooredoo Tunisia.

- It describes how to parse the sites, nodes and their correspondent physical configuration.

- The dump files provided are just one .xml contains the configuration of all nodes or one file per technology (e.g. one file for 2G physical and logical, one file for 3G physical and logical).

- The folder location to be specified later.

# EnodeB

Each class="MRBTS" where the version not start with “SBTS” represents one enodeB node.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="MRBTS" ,distName attribute - The distName attribute context is as follows: "PLMN-PLMN/MRBTS-ID" OR distName="PLMN-PLMN/SBTS-ID" - E.g distName="PLMN-PLMN/MRBTS-41017" ID = 41017 |
| NE Name | [Mandatory] | - Name attribute in the same class - E.g <p name="name">BAABDA</p> Name= BAABDA |
| Site id | [Mandatory] | - Match the deduced “NE Name” with values under “Column A” from “Sites_Nodes_Matching” file and deduce site Name from column B |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = eNodeB |
| Model |  | - = Flexi LTE Base Station |
| Manufacturer | [Optional] | - = NSN |

## Physical configuration for MRBTS

### Cabinet

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="HW" version="HW1.0" distName="PLMN-PLMN/MRBTS-41246/HW-1" 
 the node ID for this cabinet is: 41246

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = 1 to be assigned by default |
| Model | [Mandatory] | - = "systemTitle" attribute - E.g. <p name="systemTitle">Flexi LTE Base Station</p> - If attribute not found= Flexi LTE Base Station |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="MODULE" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="MODULE" version="HW1.0" distName="PLMN-PLMN/MRBTS-41246/HW-1/MODULE-101"
 the node ID for this cabinet is: 41246

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - =1 |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - = “userLabel” attribute for the same class - E.g <p name="userLabel">FSMF</p> - If class is missing defualt shelf model=“ Subrack NSN ENODEB“ |
| RU Index | [Mandatory] | - =[(Shelf id -1)*3]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =4 |
| Description | [Optional] | - Null |
| Part Number | [Optional] | - = "identificationCode" attribute under the same class - E.g. <p name="identificationCode">084792A.102</p> |

### Boards

- Each class="SUBMODULE" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="SUBMODULE" version="HW1.0" distName="PLMN-PLMN/MRBTS-41246/HW-1/MODULE-101/SUBMODULE-102"
 Node ID= 41246

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - <managedObject class="SUBMODULE" version="HW1.0" distName="PLMN-PLMN/MRBTS-41246/HW-1/MODULE-101/SUBMODULE-102"

  - <managedObject class="SUBMODULE" version="HW1.0" distName="PLMN-PLMN/MRBTS-41246/HW-1/MODULE-101/SUBMODULE-124"

  - these two boards belong to the same shelf (MODULE-101)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Check table 1.1.3.1 |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class - E.g. <p name="unitType">CORE_FSMF</p> |
| Part Number | [Optional] | - = "identificationCode" attribute under the same class - E.g. <p name="identificationCode">084792A.102</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">L1132012743</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">102</p> |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

### Boards Slot Indexes:

| Board Type | Comments |
| --- | --- |
| - CORE_FHEB. - CORE_FRMB. - CORE_FSME. - CORE_FSMF. - CORE_FXEA. - CORE_FXEB. - CORE_FXED.. | - Slot index=1 |
| - FSPD | - Slot index= 2 |
| - FSPC | - Slot index= 2, and should be increased by one for each additional board - E.g 3 FSPC boards slots 2,3,4 |
| - FBBC | - Slot index= 3, and should be increased by one for each additional board - E.g 2 FBBC boards slots 3,4 |
| - FTIB | - Slot index= 5 |
| - FSM_FANA | - Slot index= 6, and should be increased by one for each additional board |

# SRAN Physical configuration

- If we have 2 Node with the same ID but different class “MRBTS” Or “SBTS”, in this case we have to refer to the version after the class, example

As you can see in the above screen shot these 2 nodes have the same ID 905708, in this case we need to parse only one Node based on the software version, we need to take the last updated software version based on the number and alphabetic after the word “SBTS”, where in the attached case we should parse MRBTS-905708 and discard SBTS-905708, and give an error report for SBTS contain the following error message “the Node SBTS-905708 should be removed as it has old version “version name””

Special case: MRBTS with version FL16 and SBTS with version SBTS we need to parse the SBTS as SRAN and the MRTBS version “FL16” to be discarded with the error alarm

- Each class=“MRBTS” where the version start with “SBTS” represent one SRAN with the following role :

  - GSM if the Node has class=”GNBCF”

  - 3G if the Node has class=”WNBTS”

  - LTE if the Node has class=“LNBTS”

- For GSM role:

  - Under class=“ GNBCF”,

  - BSC ID= attribute “bscId”

  - BTS ID= attribute “bcfId”

  - GSM role distName will be equal to distName="PLMN-PLMN/BSC-bscId/BCF-bcfId

- For LTE role:

  - From Class=“LNBTS”

  - eNodeB ID= from distName="PLMN-PLMN/MRTBS-SRANID/LNBTS-eNodeBID

- Each class=“SBTS” represent one SRAN with the following role :

  - GSM if the Node has class=”BTSSCC”

  - 3G if the Node has class=”BTSSCW”

  - LTE if the Node has class=“LNBTS”

- For GSM role:

  - Under class=“BTSSCC”,

  - BSC ID= attribute “bscId”

  - BTS ID= attribute “bcfId”

  - GSM role distName will be equal to distName="PLMN-PLMN/BSC-bscId/BCF-bcfId

- For LTE role:

  - From Class=“LNBTS”

  - eNodeB ID= from distName="PLMN-PLMN/SBTS-SRANID/LNBTS-eNodeBID

3G Role:

- each class “WBTS” with the value of “SBTSId” attribute not null, represents one 3G role

- the value of SBTSId will be the ID of the parent SRAN

- if both class=”MRBTS” and class=”SBTS” was found as a parent for the 3G role, the choosing will be as described before.

- If the parent SRAN was not found  discard the WBTS and generate an error that contains the WBTS id and SBTSID

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class=“SBTS”, distName attribute - The distName attribute context is as follows: distName="PLMN-PLMN/SBTS-ID" - E.g distName="PLMN-PLMN/SBTS-900008" ID= 900008 |
| NE Name | [Mandatory] | - Name attribute in the same class - E.g <p name="sbtsName">BAABDA</p> Name= BAABDA |
| Site id | [Mandatory] | - Match the deduced “NE Name” with values under “Column A” from “Sites_Nodes_Matching” file and deduce site Name from column B |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = SRAN |
| Model |  | - = SRAN NSN |
| Manufacturer | [Optional] | - = NSN |

### Cabinet for class SBTS

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="HW" version="HW1.0" distName="PLMN-PLMN/SBTS-41246/HW-1" 
 the node ID for this cabinet is: 41246

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = 1 to be assigned by default |
| Model | [Mandatory] | - = Flexi LTE Base Station by default |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf for class SBTS

- Each class="FSM" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="FSM" version="HW1.0" distName="PLMN-PLMN/SBTS-904537/HW-1/FSM-1"
 the node ID for this cabinet is: 904537

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - =1 |
| Shelf index | [Mandatory] | - =from class=“FSM” from the distName="PLMN-PLMN/SBTS-904537/HW-1/FSM-X - Shelf index= X - E.g distName="PLMN-PLMN/SBTS-904537/HW-1/FSM-1 - Shelf index =1 |
| Shelf Id | [Mandatory] | - = shelf index |
| Model | [Mandatory] | - = FSM by default |
| RU Index | [Mandatory] | - =[(Shelf id -1)*3]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =4 |

### Boards for class SBTS

- Each class Not equal to “HW” represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="XXX" version="HW1.0" distName="PLMN-PLMN/SBTS-41246/HW-1/MODULE-101/XXX-102"
 Node ID= 41246

- The mapping between the boards and the shelves will be based on distName attribute for classes equal to “FBB” and on the name="unitPosition" attribute under classes equal to “FR”,

- e.g.

- The mapping between the boards and the shelves will be based on distName attribute for class=“FSM”

- Example: <managedObject class="FSM" version="HW1.0" distName="PLMN-PLMN/SBTS-904537/HW-1/FSM-1" shelf index=1

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Index | [Mandatory] | - the index of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - To be increment by for each borad |
| Board Type | [Mandatory] | - = “inventoryUnitType” attribute under the same class - E.g. name="inventoryUnitType">FBBC</p> |
| Part Number | [Optional] | - = " vendorUnitTypeNumber" attribute under the same class - E.g. |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">L1132012743</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">102</p> |

### 2.2.1 Cabinet for class MRBTS version “SBTS

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="HW" version="HW1.0" distName="PLMN-PLMN/MRBTS-41246/HW-1" 
 the node ID for this cabinet is: 41246

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = 1 to be assigned by default |
| Model | [Mandatory] | - = Flexi LTE Base Station by default |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### 2.2.2 Shelf for class MRBTS version “SBTS

- Each class=" CABINET" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="HW1.0" distName=" PLMN-PLMN/MRBTS-905708/HW-1/CABINET-1"
 the node ID for this cabinet is: 905708

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - =1 |
| Shelf index | [Mandatory] | - =from class=“ CABINET” from the distName=" PLMN-PLMN/MRBTS-905708/HW-1/CABINET-X - Shelf index= X - E.g distName=" PLMN-PLMN/MRBTS-905708/HW-1/CABINET-1 - Shelf index =1 |
| Shelf Id | [Mandatory] | - = shelf index |
| Model | [Mandatory] | - = FSM by default |
| RU Index | [Mandatory] | - =[(Shelf id -1)*3]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =4 |

### 2.2.3 Boards for class MRBTS version “SBTS

- All classes with with distname “PLMN-PLMN/MRBTS-ID/HW-ID/classname-ID” or “PLMN-PLMN/MRBTS-ID/HW-ID/CABINET-ID/classname-ID”

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="XXX" version="HW1.0" distName=" PLMN-PLMN/MRBTS-905708/HW-1/CABINET-1/SMOD-1"
 Node ID= 905708

- The mapping between the boards and the shelves will be based on distName attribute for example class="XXX" version="HW1.0" distName=" PLMN-PLMN/MRBTS-905708/HW-1/CABINET-1/SMOD-1 this board is on shelf 1, if in the distName there is no CABINET for example “distName="PLMN-PLMN/MRBTS-905708/HW-1/RMOD-1” then take the shelf ID = 1 in this case by default

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Index | [Mandatory] | - the index of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - To be increment by for each borad |
| Board Type | [Mandatory] | - = “inventoryUnitType” attribute under the same class - E.g. name="inventoryUnitType">FBBC</p> - If the value contain “FSMF” then take the board type equal to FSMF for example <p name="inventoryUnitType">Flexi System Module Outdoor FSMF</p> in this case Board type equal FSMF - If the value contain “FBBC” than board type =” FBBC” - If the value contain “FBBA” than board type =” FBBA” - Else take the value as it is for board type |
| Part Number | [Optional] | - = " vendorUnitTypeNumber" attribute under the same class - E.g. |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">L1132012743</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">102</p> |

### 2.2.4 Boards for class “INVUNIT”

- All classes with with distname “PLMN-PLMN/MRBTS-ID/HW-ID/classname-ID” or “PLMN-PLMN/MRBTS-ID/HW-ID/INVUNIT-ID/”

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
<managedObject class="INVUNIT" version="EQMHW19A_1904_002" distName="PLMN-PLMN/MRBTS-407735/HW-1/INVUNIT-8" id="11964577">
 Node ID= 407735

- The mapping between the boards and the shelves will be based on distName="PLMN-PLMN/MRBTS-407735/HW-1/INVUNIT-8" id="11964577">

Cabinet: HW-1 Cabinet = 1,

Shelf:  =1 by default.

Board: “INVUNIT-8  slot index = 8

Index On Slot: =0 by default.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Index | [Mandatory] | - Default Shelf index = 1 |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = value of digit after – in distname INVUNIT-X |
| Board Type | [Mandatory] | - = “inventoryUnitType” attribute under the same class - E.g. <p name="inventoryUnitType">FPFD-ODM</p> |
| Part Number | [Optional] | - = " vendorUnitTypeNumber" attribute under the same class |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">E4161300687</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">101</p> |

### 2.2.5 Boards for class “RMOD_INV”

- All classes with with distname “PLMN-PLMN/MRBTS-ID/HW-ID/classname-ID” or “PLMN-PLMN/MRBTS-ID/HW-ID/INVUNIT-ID/”

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
<managedObject class="RMOD_INV" version="APEQMHW19_1811_010" distName="PLMN-PLMN/MRBTS-731014/HW-1/APEQM_INV-1/RMOD_INV-3" id="13024847">

- Node ID= 731014

- The mapping between the boards and the shelves will be based on " distName="PLMN-PLMN/MRBTS-731014/HW-1/APEQM_INV-1/RMOD_INV-3" id="13024847">

- Cabinet: HW-1 Cabinet = 1,

Shelf:  =1 by default.

Board: “APEQM_INV-1”  slot index = 1

Index On Slot: =0 by default.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Index | [Mandatory] | - Default Shelf index = 1 |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = value of digit after – in distname APEQM_INV-1 |
| Board Type | [Mandatory] | - = “inventoryUnitType” attribute under the same class - E.g. <p name="inventoryUnitType">AAHF</p> |
| Part Number | [Optional] | - = " vendorUnitTypeNumber" attribute under the same class |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">E4161300687</p> |
| Issue Number | [Optional] | - = " "versionNumber"" attribute under the same class - E.g. <p name="versionNumber">102</p> |

# BSC

- Each class="BSC" represents one BSC node.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="BSC", distName attribute - The distName attribute context is as follows: "PLMN-PLMN/BSC-ID" - E.g distName="PLMN-PLMN/BSC-208844" ID = 208844 |
| NE Name | [Mandatory] | - “Name” attribute in the same class - E.g <p name="name">BAABDA</p> Name= BAABDA |
| Site id | [Mandatory] | - Match the deduced “NE Name” with values under “Column A” from “Sites_Nodes_Matching” file and deduce site Name from column B |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = BSC |
| Model | [Mandatory] | - attribute= “BSCtype” under class=“BSC”, - When the bsctype= 44  mcBSC 41flexiBSC 36BSC3i - If none is found than take the default one as BSC3i |
| Manufacturer | [Optional] | - = NSN |

## Physical configuration for flexiBSC and BSC3i

Note: If there is cloud BSC then no need to parse physical for BSC, only node & logical data will be parsed.

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/BSC-208844/HW-1/CABINET-11"
 the node ID for this cabinet is: 208844

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute take Cabinet ID and subtract 10 from it - E.g. distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13" cabinet ID= 3 |
| Model | [Mandatory] | - = "cabinetType" attribute - E.g. <p name="cabinetType">BSCC</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="CARTRIDGE" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CARTRIDGE" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13100"
 the node ID for this cabinet is: 226365

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - = “cartridgeType" attribute for the same class - E.g <p name="cartridgeType">CLOC_B</p> |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-ET16_17_17"
 Node ID= 226365

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-ET16_17_17""

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-TR3E_1_1" id="274505"

  - these two boards belong to the same shelf (CARTRIDGE-13500)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">17</p> slot index =17 |
| Board Type | [Mandatory] | - = “unitTypeExpected” attribute under the same class - E.g. <p name="unitTypeExpected">ET16</p> |
| Manufacturer |  | - = “vendorName” attribute under the same class - E.g - <p name="vendorName">Nokia</p> |
| Part Number | [Optional] | - = Null |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = Null |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

Special case for BSC3i for board ETS2:

- This is to be done only for BSC type “BSC3i”

- From the classes “FUUNIT” under the BSC, check the values under the list name=”supportedByUnit”

- Take the values that contains “ETS2”

- Remove duplicated values

- The values will have the following format:

PLMN-PLMN/BSC-bscid/HW-\d/CABINET-cabinetid/CARTRIDGE-shelfid/UNIT-slotindex_ETS2_\d

- Every unique value of these represents an ETS board with the below attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet ID |  | - =cabinetid (to be matched with cabinet id deduced before in cabinet section and to got its xml id) |
| Shelf Id | [Mandatory] | - A new shelf should be created for every unique shelfid under the same cabinet id: - Shelf id= the id of the latest created shelf +1 - RU index =[(Shelf id -1)*8]+1 - Shelf type= SGC1C_A |
| Index on slot | [Mandatory] | - = 0 |
| Slot index | [Mandatory] | - = slotindex |
| Board Type | [Mandatory] | - = ETS2 |
| Part Number | [Optional] | - = Null |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = Null |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |

## Physical configuration for mcBSC

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/BSC-208844/HW-1/CABINET-11"
 the node ID for this cabinet is: 208844

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute take Cabinet ID and subtract 10 from it - E.g. distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13" cabinet ID= 3 |
| Model | [Mandatory] | - = "cabinetType" attribute - E.g. <p name="cabinetType">BSCC</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="CARTRIDGE" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CARTRIDGE" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13100"
 the node ID for this cabinet is: 226365

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - = “cartridgeType" attribute for the same class - E.g <p name="cartridgeType">CLOC_B</p> |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-ET16_17_17"
 Node ID= 226365

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-ET16_17_17""

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-TR3E_1_1" id="274505"

  - these two boards belong to the same shelf (CARTRIDGE-13500)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Check table 3.2.3.1 |
| Board Type | [Mandatory] | - = “unitTypeExpected” attribute under the same class - E.g. <p name=" unitTypeExpected ">BMFU-B</p> |
| Part Number | [Optional] | - = "identificationCode" attribute under the same class - E.g. <p name="identificationCode">C112982.A1A</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">83134200155</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">C110598.B3A</p> |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |

### Boards Slot Indexes:

To be deduced from class=“UNIT”, to be extract the board at the end of the

distName="PLMN-PLMN/BSC-402962/HW-1/CABINET-11/CARTRIDGE-11100/UNIT-BJC_A_1_1_AIC-1”

| Board model | Comments |
| --- | --- |
| - SHM | - Slot index= 0 |
| - AIC | - Slot index is showed after “AIC” in “distName” attribute in class=”UNIT” - E.g: distName=" PLMN-PLMN/BSC-398435/HW-1/CABINET-11/CARTRIDGE-11100/UNIT-BMPP2_B_1_1_AIC-4" Slot index= 4 |
| - AMC | - Slot index= 14 and should be increased by one for each additional board. |
| - PEM | - Slot index= 9, and should be increased by one for each additional board. |
| - FAN | - Slot index= 11, and should be increased by one for each additional board. |

# mcRNC

- Each class="RNC" represents one RNC node.

- The below parsing is based on the RNC version

- The following attributes are defined at node level :

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - = distName attribute within each line containing class="RNC" - E.g.: distName="PLMN-PLMN/RNC-603" Node ID = 603 |
| NE Name | [Mandatory] | - = name="name" attribute under each line containing class=”RNC” attribute - E.g.: name="name">RNCYA0102</p> |
| Site id | [Mandatory] | - Match the deduced “NE Name” with values under “Column A” from “Sites_Matching” file and deduce site Name from column B. |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = RNC |
| Model |  | - = mcRNC |
| Manufacturer | [Optional] | - = NSN |

## Physical configuration

### Cabinet

- Each class="CABINET" represents one cabinet. The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute take Cabinet ID - E.g. distName="PLMN-PLMN/RNC-602/HW-1/CABINET-cabinet-1" cabinet ID= 1 |
| Model | [Mandatory] | - = mcRNC |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="RACK" represents one shelf. The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = <managedObject class="RACK" version="1.0" distName="PLMN-PLMN/RNC-13/HW-1/CABINET-cabinet-1/RACK-c1 - Shelf Id equal the integer after RACK-c |
| Model | [Mandatory] | - = BCN |
| RU Index | [Mandatory] | - =Shelf index |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board. Only Unit that contain HW-1 in distName attribute will be taken.

  - E.g: distName="PLMN-PLMN/RNC-602/HW-1/CABINET-cabinet-1/RACK-c1/UNIT-AMC-1"

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName=" PLMN-PLMN/RNC-602/HW-1/CABINET-cabinet-1/RACK-c1/UNIT-AMC-1"

  - class="UNIT" version="1.0" distName="PLMN-PLMN/RNC-602/HW-1/CABINET-cabinet-1/RACK-c1/UNIT-ft-1"

  - these two boards belong to the same shelf (RACK-c1)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Check table 2.1.3.1 |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class - E.g. <p name="unitType">BMFU-B</p> |
| Part Number | [Optional] | - = "identificationCode" attribute under the same class - E.g. <p name="identificationCode">C112982.A1A</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">83134200155</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">C110598.B3A</p> |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |

### Boards Slot Indexes:

To be deduced from class=“UNIT”, to be extract the board after UNIT-

from the distName="PLMN-PLMN/RNC-17/HW-1/CABINET-cabinet-1/RACK-c3/UNIT-power-supply-1

| Board | Comments |
| --- | --- |
| - motherboard | - Slot index= 0 |
| - slot | - Slot index is showed after ”slot” in “distName” attribute in class=”UNIT” - E.g: distName="PLMN-PLMN/RNC-602/HW-1/CABINET-cabinet-1/RACK-c8/UNIT-slot-2" Slot index= 2 |
| - AMC | - Slot index= 9 |
| - power-supply | - Slot index= 10, and should be increased by one for each additional board. |
| - ft | - Slot index= 12, and should be increased by one for each additional board. |

# RNC

## Physical configuration

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2"
 the node ID for this cabinet is: 2

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = Cabinet ID from “distName” attribute take Cabinet ID - E.g. distName distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2" cabinet ID= 2 |
| Model | [Mandatory] | - = "cabinetType" attribute - E.g. <p name="cabinetType">EC216</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="SUBRACK" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class=" SUBRACK " version="1.0" distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-1"
 the node ID for this cabinet is: 12

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - =Subrack ID from “distName” attribute take Subrack ID - E.g. "PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-2" Shelf ID= 2 |
| Model | [Mandatory] | - = “subrackType" attribute for the same class - E.g <p name="subrackType">SRA3</p> |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |
| Serial number | [Optional] | - = “serialNumber" attribute for the same class - E.g <p name="serialNumber">EA115040939</p> |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-2/UNIT-1"  Node ID= 12

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-2/UNIT-1"  the board belong to the cabinet 2 and shelf 2.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">17</p> slot index =17 |
| Board Type | [Mandatory] | - = “unitTypeExpected” attribute under the same class - E.g. name="unitTypeActual">NP2GE_B</p> |
| Part Number | [Optional] | - = “identificationCode” attribute under the same class |
| Serial Number | [Optional] | - = “serialNumber” attribute under the same class |

# MGW

- Each class="MGW" represents one MGW node.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="MGW", distName attribute - The distName attribute context is as follows: "PLMN-PLMN/MGW-ID" - E.g distName=" PLMN-PLMN/MGW-38" ID = 38 |
| NE Name | [Mandatory] | - “Name” attribute in the same class - E.g <p name="name">MGW-38</p> Name= MGW-38 |
| Site id | [Mandatory] | - To be deduced from Sites_matching file. |
| Type | [Mandatory] | - = MGW |
| Model |  | - = |
| Manufacturer | [Optional] | - = NSN |

## Physical configuration

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/ MGW-12/HW-1/CABINET-2"
 the node ID for this cabinet is: 2

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = Cabinet ID from “distName” attribute take Cabinet ID - E.g. distName distName="PLMN-PLMN/ MGW-12/HW-1/CABINET-2" cabinet ID= 2 |
| Model | [Mandatory] | - = "cabinetType" attribute - E.g. <p name="cabinetType">EC216</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="SUBRACK" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class=" SUBRACK" version="1.0" distName="PLMN-PLMN/ MGW-12/HW-1/CABINET-2/SUBRACK-1"
 the node ID for this cabinet is: 12

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = name="locationName"> attribute - The value will be the last 2 interger before the “-“ - Example: - <p name="locationName">01A001-000</p> …> shelf index=01 |
| Shelf Id | [Mandatory] | - =Subrack ID from “distName” attribute take Subrack ID - E.g. "PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-2" Shelf ID= 2 |
| Model | [Mandatory] | - = “name="presentationName" " attribute for the same class - E.g <p name="presentationName" ">SRA3</p> |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |
| Serial number | [Optional] | - = “serialNumber" attribute for the same class - E.g <p name="serialNumber">EA115040939</p> |
| Part Number | [Optional] | - = "identificationCode" attribute under the same class - E.g. <p name="identificationCode">084792A.102</p> |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/ MGW-12/HW-1/CABINET-2/SUBRACK-2/UNIT-1"  Node ID= 12

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/ MGW-12/HW-1/CABINET-2/SUBRACK-2/UNIT-1"  the board belong to the cabinet 2 and shelf 2.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">17</p> slot index =17 |
| Board Type | [Mandatory] | - name="presentationName" attribute under the same class - E.g. name="presentationName">NP2GE_B</p> |
| Part Number | [Optional] | - = “identificationCode” attribute under the same class |
| Serial Number | [Optional] | - = “serialNumber” attribute under the same class |

# BTS:

- Each class="BCF" represents one BTS node.

- The following attributes are defined at node level :

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="BCF", distName attribute - = distName attribute - E.g distName="PLMN-PLMN/BSC-208844/BCF-1" ID= PLMN-PLMN/BSC-208844/BCF-1 |
| NE Name | [Mandatory] | - “Name” attribute in the same class - E.g <p name="name">LOUIZE</p> - If duplicate names were found concatenate the name with “_” and an incremented integer for each duplication - The integer should be reset for each node name |
| Site id | [Mandatory] | - Match the deduced “NE Name” with values under “Column A” from “Sites _Matching” file , if found then Site ID is the correspondent value in Column B, else, Site ID is the value from NE Name after last “_”(e.g. NE Name= 0791_V_NAMOS-BRIDGE_0326AL→ Site ID=0326AL) |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = BTS |
| Model |  | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. <managedObject class="HW" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-109/HW-1" id="308288"> <p name="operationalState">1</p> <p name="systemTitle">Flexi Edge</p> - The Model for “PLMN-PLMN/BSC-208844/BCF-109” = Flexi Edge - If no model found then take default Model as “Flexi Multiradio” |
| Manufacturer | [Optional] | - = NSN |

## Physical configuration

### Cabinet

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute,
E.g 
class="HW" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1"
 the node ID for this cabinet is: PLMN-PLMN/BSC-208844/BCF-1

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute - E.g. distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1" cabinet ID= 1 |
| Model | [Mandatory] | - If the value of “unitType” attribute of one UNIT is equal to FCIA Model = FCIA - If the value of “unitType” attribute of one UNIT is equal to FCOA Model = FCOA - If “FCIA” and “FCOA” are not found, Model = Flexi Multiradio Cabinet: |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="SUBRACK" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute,
E.g 
class="SUBRACK" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1/SUBRACK-1"
 the node ID for this cabinet is: PLMN-PLMN/BSC-208844/BCF-1

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - “Subrack NSN BTS” to add by default |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute,
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1/SUBRACK-1/UNIT-BBU_1_4"
 Node ID= PLMN-PLMN/BSC-208844/BCF-1

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1/SUBRACK-1/UNIT-BBU_1_4"

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1/SUBRACK-1/UNIT-ESMB_1_1"

  - these two boards belong to the same shelf (SUBRACK-1)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">17</p> slot index =17 |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class - E.g. <p name="unitType">BBU</p> |
| Part Number | [Optional] | - = “identificationCode” attribute - E.g. <p name="identificationCode">FPRA</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = “serialNumber” attribute - E.g. <p name="serialNumber">068556548652</p> |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

# NodeB:

- Each class="WBTS" represents one NodeB.

- The following attributes are defined at node level :

- if the attribute=“SBTSId” under class=“WBTS” then to ignore this Node for physical configuration parsing

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="WBTS", distName attribute - = distName attribute - E.g distName= distName="PLMN-PLMN/RNC-14/WBTS-76 ID= PLMN-PLMN/RNC-14/WBTS-76 |
| NE Name | [Mandatory] | - “Name” attribute in the same class - E.g name="name">U_V_AT-DAYROUT2_0618UP</p> |
| Site id | [Mandatory] | - Match the deduced “NE Name” with values under “Column A” from “Sites _Matching” file , if found then Site ID is the correspondent value in Column B, else, Site ID is the value from NE Name after last “_”(e.g. NE Name= U_A_NAMOS-BRIDGE_0326AL→ Site ID=0326AL) |
| Type | [Mandatory] | - = NodeB |
| Model |  | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. - <p name="systemTitle">Nokia Flexi WCDMA Base Station</p> |
| Manufacturer | [Optional] | - = NSN |

## Physical configuration

### Cabinet

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute,
E.g. distName="PLMN-PLMN/RNC-40/WBTS-83/HW-1"

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute - E.g. distName=" distName="PLMN-PLMN/RNC-40/WBTS-83/HW-1" cabinet ID= 1 |
| Model | [Mandatory] | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. - <p name="systemTitle">Nokia Flexi WCDMA Base Station</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="Module" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute,
The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - “userLabel” under class="Module" |
| RU Index | [Mandatory] | - =[(Shelf id -1)*2]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =3 |
| Description | [Optional] | - Null |

### Boards

- Each class=" SUBMODULE"  represents one board

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - Shelf ID to be deduced from distname. - E.g. distName="PLMN-PLMN/RNC-40/WBTS-83/HW-1/MODULE-173/SUBMODULE-225" → Shelf ID is the shelf deduced from ="PLMN-PLMN/RNC-40/WBTS-83/HW-1/MODULE-173” |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Slot index of the first board on the shelf is 1 and to be incremented by 1 automatically for each new board. |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class - E.g. name="unitType">FTIF</p> |
| Part Number | [Optional] | - = “identificationCode” attribute - E.g. <p name="identificationCode">472311A</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = “serialNumber” attribute - E.g. <p name="serialNumber">068556548652</p> |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |