| # NSN Physical Connector FRS (Functional Requirement Specifications) Auto-Import connector |  |
| --- | --- |
|  | Indrasan Yadav |
|  | 04-06-2025 |

Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft | Indrasan Yadav | 04-06-2025 |
| 0.2 | Added site id parsing rule under section 9 | Indrasan Yadav | 03-07-2025 |
| 0.3 | Added case for SRAN where classes contain extra tags in section 2 Note (Special case) highlighted with yellow | Indrasan Yadav | 11-08-2025 |

Table of Contents

Introduction	4

1	EnodeB	5

1.1	Physical configuration	5

1.1.1	Cabinet	5

1.1.2	Shelf	5

1.1.3	Boards	6

1.1.3.1	Boards Slot Indexes:	7

2	BSC	20

2.1	Physical configuration	20

2.1.1	Cabinet	20

2.1.2	Shelf	21

2.1.3	Boards	21

3	BTS:	23

3.1	Physical configuration	23

3.1.1	Cabinet	23

3.1.2	Shelf	24

3.1.3	Boards	24

# Introduction

- This FRS is intended for NSN Nodes extraction for project Ooredoo Tunisia.

- It describes how to parse the sites, nodes and their correspondent physical configuration.

- The dump files provided are just one .xml contains the configuration of all nodes or one file per technology (e.g. one file for 2G physical and logical, one file for 3G physical and logical).

- The folder location to be specified later.

# EnodeB

Each class=”MRBTS” or class=”SBTS” where there exists class LNCEL only under the class LNBTS which would be under class MRBTS or SBTS would represent one enodeB. Do not consider version.

Note: Only consider classes where LNCEL would be directly under LNBTS which would be directly under SBTS/MRBTS as in screenshot above. Ignore other LNCEL classes like in SS below

If we have multiple nodes with same node ID, then need to check operationalState for LNCELL for each node. Take node for which operationalState=1 for at least one LNCEL under that node, discard the other node with operationalState=0. If for both its 1 then keep both ( we use operstate & not admin state because admin state will be one for both therefore can’t distinguish based on admin state )

Now for SBTS-900350, we see that operationalState=1 for LNCEL under it however for MRBTS-900350 its 0 so we will keep SBTS-900350 & discard MRBTS-900350

Note that duplicates nodes can both Intra & Inter OSS, if for both OperationalState=1 for LNCEL then keep both but generate error in XML saying duplicate node found, otherwise keep then one with OperationalState=1

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="MRBTS" ,distName attribute - The distName attribute context is as follows: "PLMN-PLMN/MRBTS-ID" OR distName="PLMN-PLMN/SBTS-ID" - E.g distName="PLMN-PLMN/MRBTS-41017" ID = 41017 |
| NE Name | [Mandatory] | - Name attribute in the same class - E.g <p name="name">BAABDA</p> Name= BAABDA |
| Site id | [Mandatory] | - Please follow the section 9. |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = eNodeB |
| Model | [Mandatory] | - = Flexi LTE Base Station |
| Manufacturer | [Mandatory] | - = Nokia |

## Physical configuration for MRBTS

### Cabinet

- Each class="Cabinet_R" for which distname contains EQM_R will represents one cabinet.

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="Cabinet_R" version="EQMR17A_1708_008" distName="PLMN-PLMN/MRBTS-905010/EQM_R-1/APEQM_R-1/Cabinet_R-1" 
 the node ID for this cabinet is: MRBTS-905010

- If this class does not find in Node. Take Cabinet Model “Flexi LTE Base Station”.

- If this class have product name as N/A and class="SMOD_R" have board model “ASOE AirScale Core” then mention cabinet as “ASOE AirScale Core”.

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet Index | [Mandatory] | - = 1 to be assigned by default. To be consider from attribute <p name="configDN"> from class=”CABINET_R”. <p name="configDN">MRBTS-15142/EQM-1/APEQM-1/CABINET-1</p> Here cabinet Id will be 1 as given in above attribute CABINET-1. To be increased for next cabinet by +1. |
| Cabinet id | [Mandatory] | - = Concatenate of Node Id and Cabinet Index with “_”. |
| Model | [Mandatory] | - = "productname" attribute under class="CABINET_R". - If “productname” is not present in the class or “productname” is “unknown”, Take model “Flexi LTE Base Station”. - If serial number is duplicate anywhere in that dump, then we can parse model/type from there corresponding "productname". - If this class have product name as N/A and class="SMOD_R” have board model “ASOE AirScale Core” then mention cabinet as “ASOE AirScale Core”. |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |
| Serial Number | [Mandatory] | - =”serialNumber” |
| Part Number | [Mandatory] | - =”productcode” |

### Shelf

- The Shelf should be same as cabinet.

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet Id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf Id | [Mandatory] | - = Concatenate of Cabinet Id and Shelf Index with “_”. |
| Shelf Index | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - Same as cabinet model |
| RU Index | [Mandatory] | - =[(Shelf id -1)*2]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =3 |
| Description | [Optional] | - Null |
| Serial Number | [Optional] | - Same as cabinet serial number |
| Part Number | [Optional] | - Same as cabinet part number |

### Boards

- Priority one is each class RMOD_R, BBMOD_R, SMOD_R, SMOD_EXT_R and INVUNIT_R, GNSSE_R, PWRMOD_R and SFP_R for which distName contain “EQM_R” will represent a Board. If two boards have the same serial no., ignore the one.

- FAN is not considered here as board so there is no need to parse the same.

- The 2nd priority is to parse board in each class INVUNIT which distName contain HW-1 and an attribute <p name="inventoryUnitType"> which start with CORE will be parsed as board but to ensure that same serial number should be not parsed under any above class as well as for cabinet also.

In above snippet in class="INVUNIT" an attribute <p name="inventoryUnitType"> has name CORE_Flexi System Module Outdoor FSMF which is start with CORE so it will be parse as a board, but if serial number will be duplicate need to be ignore.

- And if any serial number is already parsed for cabinet, then there is no need to parse for other one like example below: priority to be follow: Cabinet>Shelf>Board. (If already parsed for cabinet no need to parse for shelf/board, if parsed for shelf no need to parse for board)

Here only one serial number is available at 3 places first for cabinet under class=CABINET_R.

2nd for board under class=”SMOD_R” which distname contains EQM_R even it will be not parse as this is already parse for cabinet which is the same.

3rd one is under class=”INVUNIT” which is as board, but it will be ignore as same serial number which is already parse for cabinet.

Ex for class=”RMOD_R”

Ex For Class=”BBMOD_R”

Ex For Class=”SMOD_R”

Ex for Class=”INVUnit_R”

Ex for Class=”GNSSE_R”:-

Ex for class=”SFP_R”:-

Example for class="PWRMOD_R”

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of correspondent shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Start with 1. Increased with +1 for every new Board. |
| Board Type | [Mandatory] | - =”productname” for class=”BBMOD_R”/”SMOD_R”/”RMOD_R”/”SMOD_EXT_R”/”PWRMOD_R” - =”inventoryunitType” for class=”INVUNIT_R” and class="INVUNIT". - =gnssUnitName for class=”GNSSE_R”. - = “SFP_R” for class=”SFP_R”. We will take BoardType Name SFP_R as hardcore. |
| Part Number | [Optional] | - =”productCode” for class=”BBMOD_R”/”SMOD_R”/”RMOD_R”/”SMOD_EXT_R” - =”vendorunitTypeNumber” for class=”INVUNIT_R”. |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">L1132012743</p> |
| Issue Number | [Optional] | - =”hwversion” for class=”BBMOD_R”/”SMOD_R”/”RMOD_R”/”SMOD_EXT_R” - =”versionNumber” for class=”INVUNIT_R”. |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

# SRAN Physical configuration

Note (Special Case): For node type “SRAN” & in their dump each classes have some extra tags like below for example class=”MRBTS” here it is with extra tags “com.nokia.srbts:” which is extra than “MRBTS” so we will consider the value after “:” and mapped the class and apply the rule accordingly.

So please exclude the extra tags in each class which will be before “:” and consider the existing rule accordingly.

There is a list of few classes with extra tags & without extra tags which will be considered.

A node is classified as SRAN if it has more than one role as per below table if only one role, then classify as BTS/Node-B/eNode-B accordingly.

|  | 2G role | 3G role | 4G role | 5G role |
| --- | --- | --- | --- | --- |
| class="MRBTS" | GNCEL AND corresponding BCF class | WNCEL AND corresponding WBTS class | LNCEL AND corresponding LNBTS class | NRCELL |
| class=SBTS" | LCELC AND corresponding BCF class | LCELW AND corresponding WBTS class | LNCEL AND corresponding LNBTS class | NRCELL |

If we have multiple SRAN nodes with same node ID, then need to check operationalState for LNCELL for each node. Take node for which operationalState=1 for at least one LNCEL under that node, discard the other node with operationalState=0. If for both its 1 then keep both. Refer example under enodeB section.

For Role 2G

- For class=“MRBTS

- For GSM role:

- Each node with class=”MRTBS” would have 2G role if it has class GNCEL AND corresponding BCF class exists, if corresponding BCF class not found then it doesn’t have 2G role.

- Class hierarchy is MRBTS-GNBTS-GNBCF-GNCEL. Ignore other hierarchies

  - Under class=“GNBCF”,

  - BSC ID= attribute “bscId”

  - BTS ID= attribute “bcfId”

  - GSM role distName will be equal to distName="PLMN-PLMN/BSC-bscId/BCF-bcfId and name will be the value of attributes <p name="name">Cite Ennasr 1_TUN1142</p>

  - Match the MRBTS ID with “SBTSId” attribute under class=”BCF”, If value is not matched no need to consider it as role 2G.

If value is matched, consider it as role 2G for the MRBTS or node i.e. MRBTS-15142 is matched with SBTSId 15142, so it is role 2G for MRBTS-15142.

- For 3G role:

- Each node with class=”MRBTS” would have 3G role if it has class WNCEL AND corresponding WBTS class exists, if corresponding WBTS class not found then it doesn’t have 3G role

- Class hierarchy is MRBTS-WNBTS-WNCEL. Ignore other hierarchies

  - Under class=“WNBTS”

  - RNC ID = under attribute “rncConfigList”, RNC ID= attribute “identifier”

  - WBTS ID= attribute “wbtsId”

  - 3G role distName will be equal to distName="PLMN-PLMN/RNC-RNCID/WBTS-wbtsId

- For 4G Role:

- Each class=”MRBTS” where there exists class LNCEL under the class LNBTS which would be under class MRBTS would represent one enodeB.

  - From Class=“LNBTS”

  - eNodeB ID= from distName="PLMN-PLMN/MRTBS-SRANID/LNBTS-eNodeBID

- For 5G Role:

- Each class=”NRBTS” where there exists class NRCELL under the class NRBTS which would be under class MRBTS would represent one gNodeB.

g-NB name will be come under class=”NRBTS” as given below:

Here, same g-NB node is coming under same class with MRBTS & hierarchy will be as below:

MRBTS-NRBTS-NRCEL.

- For class=“SBTS”

- For 2G Role:

- Each node with class=”SBTS” would be 2G if it has class LCELC AND corresponding BCF class exists, if corresponding BCF class not found then it doesn’t have 2G role.

- Class hierarchy is SBTS-BTSSCC-LCELC. Ignore other hierarchies

  - Under class=“BTSSCC”,

  - BSC ID= attribute “bscId”

  - BTS ID= attribute “bcfId”

  - GSM role distName will be equal to distName="PLMN-PLMN/BSC-bscId/BCF-bcfId

- For 3G Role:

- Each node with class=”SBTS” would be 3G role if it has class LCELW AND corresponding WBTS class exists, if corresponding WBTS class not found then it doesn’t have 3G role

Class hierarchy is SBTS-BTSSCW-LCELW. Ignore other hierarchies

  - Under class=“BTSSCW”

  - RNC ID = under attribute “rncConfigList” , RNC ID= attribute “identifier”

  - WBTS ID= attribute “wbtsId”

  - 3G role distName will be equal to distName="PLMN-PLMN/RNC-RNCID/WBTS-wbtsId

- For 4G Role:

- Each class=”SBTS” where there exists class LNCEL under the class LNBTS which would be under class SBTS would be 4G role

  - From Class=“LNBTS”

eNodeB ID= from distName="PLMN-PLMN/SBTS-SRANID/LNBTS-eNodeBID

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class=“SBTS/MRBTS”, distName attribute - The distName attribute context is as follows: distName="PLMN-PLMN/MRBTS-ID" - E.g distName="PLMN-PLMN/MRBTS-15142" ID= MRBTS-15142. |
| NE Name | [Mandatory] | - Name attribute in the same class - E.g <p name="sbtsName">TUN1142</p> Name= TUN1142. |
| Site id | [Mandatory] | - Please refer section 9. |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = SRAN |
| Model | [Mandatory] | - = SRAN NSN |
| Manufacturer | [Mandatory] | - = Nokia |

### Cabinet for class SBTS

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="HW" version="HW1.0" distName="PLMN-PLMN/SBTS-41246/HW-1" 
 the node ID for this cabinet is: 41246

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = 1 to be assigned by default |
| Model | [Mandatory] | - = Flexi LTE Base Station by default |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |

### Shelf for class SBTS

- Each class="FSM" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="FSM" version="HW1.0" distName="PLMN-PLMN/SBTS-904537/HW-1/FSM-1"
 the node ID for this cabinet is: 904537

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - =1 |
| Shelf index | [Mandatory] | - =from class=“FSM” from the distName="PLMN-PLMN/SBTS-904537/HW-1/FSM-X - Shelf index= X - E.g distName="PLMN-PLMN/SBTS-904537/HW-1/FSM-1 - Shelf index =1 |
| Shelf Id | [Mandatory] | - = shelf index |
| Model | [Mandatory] | - = FSM by default |
| RU Index | [Mandatory] | - =[(Shelf id -1)*3]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =4 |

### Boards for class SBTS

- Each class Not equal to “HW” represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="XXX" version="HW1.0" distName="PLMN-PLMN/SBTS-41246/HW-1/MODULE-101/XXX-102"
 Node ID= 41246

- The mapping between the boards and the shelves will be based on distName attribute for classes equal to “FBB” and on the name="unitPosition" attribute under classes equal to “FR”,

- e.g.

- The mapping between the boards and the shelves will be based on distName attribute for class=“FSM”

- Example: <managedObject class="FSM" version="HW1.0" distName="PLMN-PLMN/SBTS-904537/HW-1/FSM-1" shelf index=1

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Index | [Mandatory] | - the index of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - To be increment by for each borad |
| Board Type | [Mandatory] | - = “inventoryUnitType” attribute under the same class - E.g. name="inventoryUnitType">FBBC</p> |
| Part Number | [Optional] | - = " vendorUnitTypeNumber" attribute under the same class - E.g. |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">L1132012743</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">102</p> |

### 2.2.1 Cabinet for class MRBTS version “SBTS

Same as 1.1.1.

### 2.2.2 Shelf for class MRBTS version “SBTS

Same as 1.1.2.

### 2.2.3 Boards for class MRBTS version “SBTS

Same as 1.1.3.

## 2.2.4 Antenna parsing details for SRAN Node

Each class="ALD_R" whose distName= contains EQM_R represents one antenna.

- The following attributes are defined at antenna node level in radioantenna csv & serial number will be parsed under addinfo csv.

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - Concatenate of Parent Node & last integer value in attribute <p name="configDN">: Example:1 <p name="configDN">MRBTS-15142/EQM-1/APEQM-1/ALD-1</p> NE ID: MRBTS-15142_1 |
| NE Name | [Mandatory] | - Same as NE ID |
| Site id | [Mandatory] | - Site Id of related parent node. |
| Site Name | [Optional] | - = Site id |
| Type | [Mandatory] | - Hard core as “SingleAntennaDevice”. |
| Model | [Optional] | - =Type |
| Serial Number | [Mondatory] | - Serial number will get from below attribute - <p name="serialNumber">00014CN1013118483</p> - within the class=”ALD_R”. |
| Manufacturer | [Optional] | - = Nokia |
| Part Code | [Optional] | - Part code will get from below attribute - <p name="productCode">RV4PX310B1Q</p> - Within the class=”ALD_R”. |
| Issue Number | [Optional] | - Issue number will get from below attribute - <p name="hwVersion">332</p> - Within the class=”ALD_R” |

# BSC

Note: In Ooredoo all BSCs are AirScale/cloud BSC then no need to parse physical for BSC, only node & logical data will be parsed.

Note: (For logical data like IP/VLAN/GW/Mask will be shared by customer in separated file which will explain here later once received and for 2g cells we will parse it from BSC dump itself)

- Each class="BSC" represents one BSC node.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="BSC", distName attribute - The distName attribute context is as follows: "PLMN-PLMN/BSC-ID" - E.g distName="PLMN-PLMN/BSC-208844" ID =BSC-415338 |
| NE Name | [Mandatory] | - “Name” attribute in the same class - E.g <p name="name">BSCMNB006</p> Name= BSCMNB06 |
| Site id | [Mandatory] | - Please follow section 9. |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = BSC |
| Model | [Mandatory] | - attribute= <p name="bscType">51</p> under class= “BSC”, - When the bsctype= 44  mcBSC 41flexiBSC 36BSC3i 51AirScale BSC - If none is found than take the default one as AirScale BSC. |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration for flexiBSC and BSC3i

Note: If there is AirScale/cloud BSC then no need to parse physical for BSC, only node & logical data will be parsed.

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/BSC-208844/HW-1/CABINET-11"
 the node ID for this cabinet is: 208844

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute take Cabinet ID and subtract 10 from it - E.g. distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13" cabinet ID= 3 |
| Model | [Mandatory] | - = "cabinetType" attribute - E.g. <p name="cabinetType">BSCC</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="CARTRIDGE" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CARTRIDGE" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13100"
 the node ID for this cabinet is: 226365

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - = “cartridgeType" attribute for the same class - E.g <p name="cartridgeType">CLOC_B</p> |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-ET16_17_17"
 Node ID= 226365

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-ET16_17_17""

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-TR3E_1_1" id="274505"

  - these two boards belong to the same shelf (CARTRIDGE-13500)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">17</p> slot index =17 |
| Board Type | [Mandatory] | - = “unitTypeExpected” attribute under the same class - E.g. <p name="unitTypeExpected">ET16</p> |
| Manufacturer |  | - = “vendorName” attribute under the same class - E.g - <p name="vendorName">Nokia</p> |
| Part Number | [Optional] | - = Null |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = Null |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

Special case for BSC3i for board ETS2:

- This is to be done only for BSC type “BSC3i”

- From the classes “FUUNIT” under the BSC, check the values under the list name=”supportedByUnit”

- Take the values that contains “ETS2”

- Remove duplicated values

- The values will have the following format:

PLMN-PLMN/BSC-bscid/HW-\d/CABINET-cabinetid/CARTRIDGE-shelfid/UNIT-slotindex_ETS2_\d

- Every unique value of these represents an ETS board with the below attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet ID |  | - =cabinetid (to be matched with cabinet id deduced before in cabinet section and to got its xml id) |
| Shelf Id | [Mandatory] | - A new shelf should be created for every unique shelfid under the same cabinet id: - Shelf id= the id of the latest created shelf +1 - RU index =[(Shelf id -1)*8]+1 - Shelf type= SGC1C_A |
| Index on slot | [Mandatory] | - = 0 |
| Slot index | [Mandatory] | - = slotindex |
| Board Type | [Mandatory] | - = ETS2 |
| Part Number | [Optional] | - = Null |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = Null |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |

## Physical configuration for mcBSC

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/BSC-208844/HW-1/CABINET-11"
 the node ID for this cabinet is: 208844

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute take Cabinet ID and subtract 10 from it - E.g. distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13" cabinet ID= 3 |
| Model | [Mandatory] | - = "cabinetType" attribute - E.g. <p name="cabinetType">BSCC</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="CARTRIDGE" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CARTRIDGE" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13100"
 the node ID for this cabinet is: 226365

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - = “cartridgeType" attribute for the same class - E.g <p name="cartridgeType">CLOC_B</p> |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-ET16_17_17"
 Node ID= 226365

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-ET16_17_17""

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-226365/HW-1/CABINET-13/CARTRIDGE-13500/UNIT-TR3E_1_1" id="274505"

  - these two boards belong to the same shelf (CARTRIDGE-13500)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Check table 3.2.3.1 |
| Board Type | [Mandatory] | - = “unitTypeExpected” attribute under the same class - E.g. <p name=" unitTypeExpected ">BMFU-B</p> |
| Part Number | [Optional] | - = "identificationCode" attribute under the same class - E.g. <p name="identificationCode">C112982.A1A</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">83134200155</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">C110598.B3A</p> |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |

### Boards Slot Indexes:

To be deduced from class=“UNIT”, to be extract the board at the end of the

distName="PLMN-PLMN/BSC-402962/HW-1/CABINET-11/CARTRIDGE-11100/UNIT-BJC_A_1_1_AIC-1”

| Board model | Comments |
| --- | --- |
| - SHM | - Slot index= 0 |
| - AIC | - Slot index is showed after “AIC” in “distName” attribute in class=”UNIT” - E.g: distName=" PLMN-PLMN/BSC-398435/HW-1/CABINET-11/CARTRIDGE-11100/UNIT-BMPP2_B_1_1_AIC-4" Slot index= 4 |
| - AMC | - Slot index= 14 and should be increased by one for each additional board. |
| - PEM | - Slot index= 9, and should be increased by one for each additional board. |
| - FAN | - Slot index= 11, and should be increased by one for each additional board. |

# McRNC (To be consider in Ooredoo)

- Each class="RNC" represents one RNC node.

- The below parsing is based on the RNC version

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - = distName attribute within each line containing class="RNC" - E.g.: distName="PLMN-PLMN/RNC-14" Node ID = RNC-14 |
| NE Name | [Mandatory] | - = name="name" attribute under each line containing class=”RNC” attribute - E.g.: name="name">RNCMGH03</p> |
| Site id | [Mandatory] | - Please follow section 9. |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = RNC |
| Model |  | - = mcRNC |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration

### Cabinet

- Each class="CABINET" represents one cabinet. The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet index | [Mandatory] | - = from “distName” attribute take Cabinet ID which will be Cabinet Index. - E.g. distName="PLMN-PLMN/RNC-14/HW-1/CABINET-cabinet-1" Cabinet Index= 1 |
| Cabinet id | [Mandatory] | - Concatenate of Node Id & Cabinet Index with “_”. - Cabinet Id = RNC14_1 as described above |
| Model | [Mandatory] | - = mcRNC |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |

### Shelf

- Each class="RACK" represents one shelf. The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = The id of correspondent cabinet |
| Shelf index | [Mandatory] | - = The last numeric value associated with RACK-c1/RACK-c2/RACK-c3/RACK-c4 in below which is highlighted under class=”RACK” Here, Shelf Index will be 1,2,3 & 4 which is coming from RACK-c1/RACK-c2/RACK-c3/RACK-c4. |
| Shelf Id | [Mandatory] | - Concatenate of Cabinet Id and Shelf Index with “_”. - Shelf Index equal the integer after RACK-c Here, Shelf Id will be: RNC-14_1_1 RNC-14_1_2 RNC-14_1_3 RNC-14_1_4 |
| Model | [Mandatory] | - = BCN |
| RU Index | [Mandatory] | - =Shelf index |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board. Only Unit that contain HW-1 in distName attribute will be taken.

  - E.g: distName="PLMN-PLMN/RNC-14/HW-1/CABINET-cabinet-1/RACK-c1/UNIT-AMC-1"

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName=" PLMN-PLMN/RNC-14/HW-1/CABINET-cabinet-1/RACK-c1/UNIT-AMC-1"

  - class="UNIT" version="1.0" distName="PLMN-PLMN/RNC-14/HW-1/CABINET-cabinet-1/RACK-c1/UNIT-ft-1"

  - these two boards belong to the same shelf (RACK-c1)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Check table 4.1.3.1 |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class - E.g. <p name="unitType">HDSAM-A</p> |
| Part Number | [Optional] | - = "identificationCode" attribute under the same class - E.g. <p name="identificationCode">MIC-5401-0000E</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = "serialNumber" attribute under the same class - E.g. <p name="serialNumber">AT173100039</p> |
| Issue Number | [Optional] | - = "version" attribute under the same class - E.g. <p name="version">C110598.B3A</p> |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |

### Boards Slot Indexes:

To be deduced from class=“UNIT”, to be extract the board after UNIT-

from the distName="PLMN-PLMN/RNC-14/HW-1/CABINET-cabinet-1/RACK-c3/UNIT-AMC-1

| Board | Comments |
| --- | --- |
| - motherboard | - Slot index= 0 |
| - slot | - Slot index is showed after ”slot” in “distName” attribute in class=”UNIT” - E.g: distName="PLMN-PLMN/RNC-14/HW-1/CABINET-cabinet-1/RACK-c8/UNIT-slot-1" Slot index= 1 |
| - AMC | - Slot index= 9 |
| - power-supply | - Slot index= 10 and should be increased by one for each additional board. |
| - ft | - Slot index= 12 and should be increased by one for each additional board. |

# RNC

## Physical configuration

### Cabinet

- Each class="CABINET" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="CABINET" version="1.0" distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2"
 the node ID for this cabinet is: 2

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = Cabinet ID from “distName” attribute take Cabinet ID - E.g. distName distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2" cabinet ID= 2 |
| Model | [Mandatory] | - = "cabinetType" attribute - E.g. <p name="cabinetType">EC216</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = NSN |

### Shelf

- Each class="SUBRACK" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute, which will contain the node ID
E.g 
class=" SUBRACK " version="1.0" distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-1"
 the node ID for this cabinet is: 12

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - =Subrack ID from “distName” attribute take Subrack ID - E.g. "PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-2" Shelf ID= 2 |
| Model | [Mandatory] | - = “subrackType" attribute for the same class - E.g <p name="subrackType">SRA3</p> |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |
| Serial number | [Optional] | - = “serialNumber" attribute for the same class - E.g <p name="serialNumber">EA115040939</p> |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute, which will contain the node ID
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-2/UNIT-1"  Node ID= 12

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/RNC-12/HW-1/CABINET-2/SUBRACK-2/UNIT-1"  the board belong to the cabinet 2 and shelf 2.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">17</p> slot index =17 |
| Board Type | [Mandatory] | - = “unitTypeExpected” attribute under the same class - E.g. name="unitTypeActual">NP2GE_B</p> |
| Part Number | [Optional] | - = “identificationCode” attribute under the same class |
| Serial Number | [Optional] | - = “serialNumber” attribute under the same class |

# BTS: (BTS Model & physical will be check once standalone sample dump received)

- Each class="BCF" represents one BTS node.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="BCF" under distName we will take NE ID for BTS will be BSC ID with BCF ID. - E.g <managedObject class="BCF" version="ASBSCFP23R1" distName="PLMN-PLMN/BSC-415338/BCF-1" id="189274601"> - NE ID= BSC-415338/BCF-1 |
| NE Name | [Mandatory] | - “Name” attribute in the same class - E.g <p name="name">TUN1661</p> - If duplicate names were found concatenate the name with “_” and an incremented integer for each duplication - The integer should be reset for each node name |
| Site id | [Mandatory] | - Please follow section 9. |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | - = BTS |
| Model | [Mandatory] | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. <managedObject class="HW" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-109/HW-1" id="308288"> <p name="operationalState">1</p> <p name="systemTitle">Flexi Edge</p> - The Model for “PLMN-PLMN/BSC-208844/BCF-109” = Flexi Edge - If no model found, then take default Model as “Flexi Multiradio” |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration

### Cabinet

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute,
E.g 
class="HW" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1"
 the node ID for this cabinet is: PLMN-PLMN/BSC-208844/BCF-1

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute - E.g. distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1" cabinet ID= 1 |
| Model | [Mandatory] | - If the value of “unitType” attribute of one UNIT is equal to FCIA Model = FCIA - If the value of “unitType” attribute of one UNIT is equal to FCOA Model = FCOA - If “FCIA” and “FCOA” are not found, Model = Flexi Multiradio Cabinet: |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |

### Shelf

- Each class="SUBRACK" represents one shelf

- The mapping between the shelf and the node will be depending on distName attribute,
E.g 
class="SUBRACK" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1/SUBRACK-1"
 the node ID for this cabinet is: PLMN-PLMN/BSC-208844/BCF-1

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = the id of correspondent cabinet |
| Shelf index | [Mandatory] | - = shelf id |
| Shelf Id | [Mandatory] | - = 1 for the first shelf found for a specific node, and to be incremented by one for each additional shelf |
| Model | [Mandatory] | - “Subrack NSN BTS” to add by default |
| RU Index | [Mandatory] | - =[(Shelf id -1)*8]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =9 |
| Description | [Optional] | - Null |

### Boards

- Each class="UNIT" represents one board

- The mapping between the boards and the node will be depending on distName attribute,
E.g 
class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1/SUBRACK-1/UNIT-BBU_1_4"
 Node ID= PLMN-PLMN/BSC-208844/BCF-1

- The mapping between the boards and the shelves will be based on distName attribute, which will contain the shelf
e.g.

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1/SUBRACK-1/UNIT-BBU_1_4"

  - class="UNIT" version="1.0" distName="PLMN-PLMN/BSC-208844/BCF-1/HW-1/SUBRACK-1/UNIT-ESMB_1_1"

  - these two boards belong to the same shelf (SUBRACK-1)

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of the related shelf |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - = “position” attribute under the same class - E.g. <p name="position">17</p> slot index =17 |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class - E.g. <p name="unitType">BBU</p> |
| Part Number | [Optional] | - = “identificationCode” attribute - E.g. <p name="identificationCode">FPRA</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = “serialNumber” attribute - E.g. <p name="serialNumber">068556548652</p> |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

# NodeB: (NodeB Model & physical will be check once standalone sample dump received)

- Each class="WBTS" represents one NodeB.

- The following attributes are defined at node level:

- if the attribute=“SBTSId” under class=“WBTS” then to ignore this Node for physical configuration parsing

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="WBTS", distName attribute - = distName attribute - E.g distName= distName="PLMN-PLMN/RNC-14/WBTS-76 ID= PLMN-PLMN/RNC-14/WBTS-76 |
| NE Name | [Mandatory] | - “Name” attribute in the same class - E.g name="name">U_V_AT-DAYROUT2_0618UP</p> |
| Site id | [Mandatory] | - Please follow section 9. |
| Type | [Mandatory] | - = NodeB |
| Model |  | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. - <p name="systemTitle">Nokia Flexi WCDMA Base Station</p> |
| Manufacturer | [Optional] | - = Nokia |

## Physical configuration

### Cabinet

- Each class="HW" represents one cabinet

- The mapping between the cabinet and the node will be depending on distName attribute,
E.g. distName="PLMN-PLMN/RNC-40/WBTS-83/HW-1"

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - = from “distName” attribute - E.g. distName=" distName="PLMN-PLMN/RNC-40/WBTS-83/HW-1" cabinet ID= 1 |
| Cabinet Index | [Mandatory] | - Default 1 |
| Model | [Mandatory] | - = “systemTitle” attribute under the correspondent Class=”HW” - E.g. - <p name="systemTitle">Nokia Flexi WCDMA Base Station</p> |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |

### Shelf

- The below table describes how to parse the shelf attributes:

- It should be same as cabinet.

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf id | [Mandatory] | - = Concatenate of cabinet if & shelf index. |
| Cabinet Id | [Mandatory] | - = the id of correspondent cabinet Id |
| Shelf index | [Mandatory] | - Shelf index of the first shelf on the first cabinet is 1 and to be incremented by 1 automatically for each new shelf for that related cabinet. |
| Model | [Mandatory] | - Same as cabinet |
| RU Index | [Mandatory] | - =[(Shelf id -1)*2]+1 - E.g. Shelf id =1 → RU index =1 - E.g. Shelf id =2 → RU index =3 |
| Date Of Manufacturer | [Optional] | - Null |
| Issue Number | [Optional] | - = Null |
| Description | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

### Boards

- Each class="Module" and class=" SUBMODULE" represents one board for related parent node which will be available in same class under attribute “distname” and distname will have HW-1.

- Example:

Note: please ensure that same serial can be available under class=”Module”/class=”SUBMODULE” for related parent node so ignore to parsing in board.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - the id of correspondent shelf id |
| Index on slot | [Mandatory] | - = Default index on slot = 0 |
| Slot index | [Mandatory] | - Slot index of the first board on the shelf is 1 and to be incremented by 1 automatically for each new board. |
| Board Type | [Mandatory] | - = “unitType” attribute under the same class=”SUBMODULE” - E.g. name="unitType">FTIF</p> - = “userLabel” attribute under the same class=”MODULE” - E.g. <p name="userLabel">FSME</p> |
| Part Number | [Optional] | - = “identificationCode” attribute - E.g. <p name="identificationCode">472311A</p> |
| Description | [Optional] | - = Null |
| Date Of Manufacture | [Optional] | - = Null |
| Serial Number | [Optional] | - = “serialNumber” attribute - E.g. <p name="serialNumber">068556548652</p> |
| Issue Number | [Optional] | - = Null |
| Power Consumption | [Optional] | - = Null |
| Additional Information | [Optional] | - = Null |
| Dimensions and Weight | [Optional] | - = Null |

### Antenna Parsing for NodeB

Each class="RET" whose distName= contains “/RET” represents one antenna details.

- The following attributes are defined at antenna node level in radioantenna csv & serial number will be parsed under addinfo csv.

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - Concatenate of Parent Node & last integer value in attribute distname after “RET-”: Example:1 distName="PLMN-PLMN/RNC-14/WBTS-4165/MRBTS-1/RET-5" NE ID: RNC-14/WBTS-4165_5 |
| NE Name | [Mandatory] | - The value of attribute <p name="baseStationID"> as given below under class=”RET”: |
| Site id | [Mandatory] | - Site Id of related parent node. |
| Site Name | [Optional] | - = Site id |
| Type | [Mandatory] | - The value of attribute <p name="antModel"> under same class=”RET”. |
| Model | [Optional] | - =Type |
| Serial Number | [Mondatory] | - Serial number will get from below attribute - <p name="antSerial"> - within the class=”RET”. Note: please ensure that same serial can be available under class=”Module”/class=”SUBMODULE” for related parent node so ignore to parsing in board. |
| Manufacturer | [Optional] | - = Nokia |
| Part Code | [Optional] | - Part code will get from below attribute - <p name="prodCode"> - Within the class=”RET”. |
| Issue Number | [Optional] | - Issue number will get from below attribute - <p name="hwVersion"> - Within the class=”RET” |

# g-NodeB

Each class=”MRBTS” where there exist class=”NRCELL” only under the class=”NRBTS” which would be under the class=”MRBTS” or class=”SBTS” would represent one g-NB. Do not consider version.

Note: Only consider classes where NRCELL would be directly under NRBTS which would be directly under MRBTS/SBTS as in screenshot above

If we have multiple nodes with same Node Id, then need to check operationState for NRCELL for each node. Take node for which operationalState=1 or enable for at least 1 NRCELL under that node, discard the other node with operationalState=0 or disable. If for both its 1 then keep both (We use operstate & not admin state because admin state will be one for both therefore can’t distinguish based on admin state)

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE ID | [Mandatory] | - From class="MRBTS", distName attribute - = distName attribute - E.g distName= distName="PLMN-PLMN/MRBTS-512008 ID= MRBTS-512008 |
| NE Name | [Mandatory] | - “Name” attribute in the same class - E.g: |
| Site id | [Mandatory] | - Please refer section 9. |
| Site Name | [Mandatory] | - Site Id |
| Type | [Mandatory] | - = gNodeB |
| Model |  | - = NSN gNodeB |
| Manufacturer | [Optional] | - = Nokia |

## 8.1 Physical Configuration For gNodeB

### 8.1.1 Cabinet

Each class=”CABINET_R” for which distname contains EQM_R will represent one cabinet.

The mapping between the cabinet and the node will be depending on the distname attribute which will contain the node id.

E.g.:

Class=”CABINET_R” version=”EQMR21A_2012_002” distname=”PLMN-PLMN/MRBTS-1585637/EQM_R-1/APEQM_R-1/CABINET_R-1”

The node id for this cabinet is MRBTS-1585637.

If this class does not find in the node, take cabinet model “NSN gNodeB BTS”.

The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - Concatenate of Node Id & Cabinet Index |
| Cabinet Index | [Mandatory] | - 1 to be assigned by default |
| Model | [Mandatory] | - “productname” attribute - If productname is not present in the class or “prductname” is “unknown”, then take model NSN gNodeB BTS. |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |
| Serial Number | [Mandatory] | - =”serialNumber” |
| Part Number | [Mandatory] | - =”productCode” |

### 8.1.2 Shelf

- The shelf should be same as cabinet

- The below table describe how to parse the shelf attributes.

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet id | [Mandatory] | - The Id of correspondent cabinet |
| Shelf Index | [Mandatory] | - 1 to be assigned by default |
| Shelf Id | [Mandatory] | - Cabinet Id |
| Model | [Mandatory] | - Same as cabinet model |
| Manufacturer | [Mandatory] | - To be assigned by default - E.g.: Manufacturer = Nokia |
| RU Index | [Mandatory] | - =[(shelf Id-1)*2]+1 - E.g.: Shelf Id=1 then RU Index=1 - E.g.: Shelf Id=2 then RU Index=3 |

### 8.1.3 Board

Same as 1.1.3

# Site Id Parsing Rule

- 1st Priority: Site Matching

- 2nd Priority: Naming Convention Rule

- 3rd Priority: If No rule applicable it will come with default site with default Lat/Long

## 9.1 SRAN follow site matching

Note: For role2g/role3g/role4g site id will be same as parent site id.

## 9.2 BSC follow site matching

## 9.3 RNC follow site matching

## BTS

### 9.4.1. Parse the last value after special character “_” from ‘NE Name’.

### 9.4.2. If there is no special character in ‘NE Name’ then parse the same value of ‘NE Name’ as “Site Id”.

## NodeB

### 9.5.1. Parse the last value after special character “_” from ‘NE Name’.

### 9.5.2. If there is no special character in ‘NE Name’ then parse the same value of ‘NE Name’ as “Site Id”.

### 9.5.3. For Lat/Long we will check the value of <p name="LinkedCellDN"> after “PLMN-PLMN/” and before “/WCEL-XXX” under class="WLCSE", It means consider the node id only after “PLMN-PLMN/” and before “/WCEL” then parse the value of <p name="AntennaCoordLatitude"> as Latitude and <p name="AntennaCoordLongitude"> as Longitude against that node id.

Note: Please put a decimal value after start 2 digit.

Example:

## 9.6 eNodeB