NSN 5G Parsing

FRS (Functional Requirement Specifications)

Auto-Import

|  | Draft version 0.7 2025-09-16 |
| --- | --- |

Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft. | Indrasan Yadav | 2025-09-16 |

# PREAMBLE

- This document specifies how to parse the logical inventory attributes for gNodeB and its cells.

- It covers the parameters defined for gNodeB RAN logical attributes.

- The provided dump files are of “.xml” format for each node.

  - File will be name of node id like “MRBTS115228”.

- This is motivated by Ooredoo Tunisia Projects and can be used for other operators having same file structure in case of 5G.

# Connector name

NSN_RAN_Ooredoo Tunisia

# Connector Information

- The dump file is an “.xml” file composed of set of Managed Objects for each node.

- Each Managed Object starts with “<managedObject “and ends with “</managedObject>”.

- Inside each managed object, there is a Tag name, followed by “attributes” and finally by the different set of parameters with their values.

# GNODEB logical parameters

The gNodeB logical inventory attributes are classified accordingly in the below categories:

  - Global Parameters

  - Networking Parameters

  - Cell Parameters

- Note: The internal mapping between the Physical and Logical parameters will be based on the Node ID.

### Global Parameters

The below global attributes defined at gNodeB level are as follows:

Class=“MRBTS”

| Attributes | Importance | Example |
| --- | --- | --- |
| Site Name/Site ID | [Mandatory] | - Please follow section 9 in physical FRS. |
| Node ID | [Mandatory] | - = “distName” attribute under the <managedObject Tag with having the “class” attribute = “NRBTS”. - E.g.: <managedObject class="com.nokia.srbts.nrbts:NRBTS" version="NRBTSCL24R2_2404_130" distName="MRBTS-515870/NRBTS-115870" operation="create"> gNodeB ID = NRBTS-115870 |
| Name | [Mandatory] | “Name” OR <p name="btsName"> attribute in the same class=”MRBTS. |
| gNodeB Name | [Mandatory] | The value of an attribute: <p name="gNbCuName"> under class=”NRBTS”. Example: <p name="gNbCuName">TUNN870</p> Note: If class”NRBTS” is not found please consider the attribute “name” from class=”MRBTS”. |
| Type | [Mandatory] | - Model of that gNodeB - If Model not found Type will be “NSN gNodeB” |
| OAM IP | [Optional] | - Mplane IP parsing rule under section 5 and take IP address |
| OAM Subnet Mask | [Optional] | - Mplane IP parsing rule under section 5 and take subnetmask for that IP address |

### Networking Parameters

The below networking attributes defined at gNodeB level are as follows:

| Attributes | Importance | Example |
| --- | --- | --- |
| MCC | [Mandatory] | - The value of an attribute <p name="mcc"> under the “class”=“NRBTS” E.g.: name= <p name="mcc">605</p> - The “name” attribute mentioned under same class=”NRBTS” refers the node name. |
| MNC | [Mandatory] | - The value of an attribute <p name="mnc"> under the “class”=“NRBTS” E.g.: name= <p name="mnc">3</p> - The “name” attribute mentioned under same class=”NNRBTS” refers the node name. |
| TAC list | [Optional] | - =NEP tool calculated, TAC list of children cells. |
| Served Cells | [Optional] | - NEP tool calculated. |
| DU ID | [Optional] | - “gNbDuId” attribute under class=”NRDU” |
| CU ID | [Optional] | - "gNbCuUpId" attribute under class=”NRCUUP” |
| Status |  |  |
| Parent AMFs | [Optional] | the value of " “amfName” _”amfIPAddress" in class=“ SERVEDAMF”, If amfName or amfIPAddress is not available put blank value for the same. |

### Cell Logical Parameters

| Attributes | Importance | Comment |
| --- | --- | --- |
| Node ID | Mandatory | - In class=”NRBTS” under attribute below: distName="MRBTS-515870/NRBTS-115870/NRCELL-11" - Node ID = MRBTS-515870 |
| Parent Site ID | Mandatory | - (NEP tool calculated) OR Site id of parent node. |
| Cell ID | Mandatory | - It is the value of an attribute <p name="physCellId"> under class=”NRCELL”. - Exp: <p name="physCellId">561</p> |
| Cell Name | Mandatory | - The value of attribute <p name="cellName"> under class=”NRBTS” - Exp: <p name="cellName">TUNN870F11</p> |
| Display Name/Cell Object Id | Mandatory | - Concatenate of the value <p name="physCellId"> attribute <p name="nrCellIdentity"> under class=”NRCELL” with a special character “_”. - <p name="physCellId">561</p> - <p name="nrCellIdentity">1898414091</p> - E.g.: 561_1898414091 |
| External Code | Mandatory | - Is the concatenation of “gnodeBID”, “_” and “Cell Object ID”. - E.g. NRBTS-115870_561_1898414091 |
| Cell Technology | Mandatory | - NR |
| MCC | [Mandatory] | - The same as described under section-4.2. |
| MNC | [Mandatory] | - The same as described under section-4.2. |
| Neighbor Cells 2G | Optional | - NULL |

- Specific attributes for 5G cells

| Attributes | Example | Comments |
| --- | --- | --- |
| Cell Duplexing Mode | Mandatory | - under class=“NRCELL” If “cellTechnology”>FDD< Duplexing Mode= FDD If “cellTechnology”>0< Duplexing Mode= FDD If “cellTechnology”>TDD< Duplexing Mode= TDD If “cellTechnology”>1< Duplexing Mode= TDD |
| PCID | Mandatory | - Under Class=”NRCELL” name=" physCellId” - "E.g.: <PhysCellId>561</PhyCellId> |
| TAC lD | Mandatory | - Under: Class=”NRCELL” An attribute - <p name="configuredEpsTac">4201</p> |
| Downlink Bandwidths | Mandatory | - For FDD: Match the distname of class=”NRCELL” with class=”NRCELL_FDD” - It is found Under : Class=”NRCELL_FDD” name=" chBwDl " E.g.: name=" chBwDl ">20MHz</p> - For TDD: Match the distname of class=”NRCELL” with class=”NRCELL_TDD” - it will be found under class=”NRCELL_TDD” ” name=" chBwDl " E.g.: name=" chBwDl ">200</p> - Divide the value of dlChBw/ChBw by 10 to obtain Downlink Bandwidth E.g.: name="ChBw">200</p> Then bandwidth will be 20 If “NRCELL_TDD” not found for the distname get the value of Attribute “chBw” that will be downlink Bandwidth - Divide the value of dlChBw/ChBw by 10 to obtain Downlink Bandwidth E.g.: name="ChBw">80MHz</p> Then Downlink Bandwidth will be 8. |
| Uplink Bandwidths | Mandatory | - For FDD: Match the distname of class=”NRCELL” with class=”NRCELL_FDD” - It is found Under : Class=”NRCELL_FDD” name=" chBwUl " E.g.: name=" chBwUl ">20MHz</p> - For TDD: Match the distname of class=”NRCELL” with class=”NRCELL_TDD” - it will be found under class=”NRCELL_TDD” ” name=" chBwUl " E.g.: name=" chBwUl ">200</p> - Divide the value of chBwUl /ChBw by 10 to obtain Uplink Bandwidth E.g.: name="ChBw">200</p> Then Uplink bandwidth will be 20 If “NRCELL_TDD” not found for the distname get the value of Attribute “chBw” that will be downlink Bandwidth - Divide the value of chBwUl /ChBw by 10 to obtain Uplink Bandwidth E.g.: name="ChBw">80MHz</p> Then Uplink bandwidth will be 8. |
| Downlink Narfcn | Mandatory | - For FDD: Match the distname of class=”NRCELL” with class=”NRCELL_FDD” - It is found Under : Class=”NRCELL_FDD” name=" nrarfcnDl" E.g.: name="nrarfcnDl">127400</p> - For TDD: Match the distname of class=”NRCELL” with class=”NRCELL_TDD” - it will be found under class=”NRCELL_TDD” ” name=" nrarfcnDl " E.g.: name="nrarfcnDl">127400</p> So, downlink NARFCN will be “127400” If “NRCELL_TDD” not found for the distname get the value of Attribute “nrarfcn” that will be downlink Bandwidth So, downlink NARFCN will be “630668” |
| Uplink Narfcn | Mandatory | - For FDD: Match the distname of class=”NRCELL” with class=”NRCELL_FDD” - It is found Under : Class=”NRCELL_FDD” name=" nrarfcnDl" E.g.: name="nrarfcnUl">127400</p> So Uplink NARFCN will be 136600 - For TDD: Match the distname of class=”NRCELL” with class=”NRCELL_TDD” - it will be found under class=”NRCELL_TDD” ” name=" nrarfcnDl " E.g.: name="nrarfcnUl">127400</p> So Uplink NARFCN will be “127400” If “NRCELL_TDD” not found for the distname get the value of Attribute “nrarfcn” that will be downlink Bandwidth So, Uplink NARFCN will be “630668” |
| MIMO configuration | Mandatory | - For FDD: Following the value of “dlMimoMode”, the “MIMO configuration” will be deduced. - “dlMimoMode” is found under Class=”NRCELL” name="dlMimoMode" - E.g.: name="dlMimoMode">40</p> OR E.g.: name="dlMimoMode">4x4 or 4x2 Closed Loop Spatial Multiplexing</p> - For TDD: Following the value of <p name="dlMimoMode">, the “MIMO configuration” will be deduced under class=“NRCELL”. - E.g.: <p name="dlMimoMode">8x4 Closed Loop Spatial Multiplexing</p> If this value is in numeric form, then follow the following: Below mapping to be used to get mimo configuration from dlMimoMode - 0 : SingleTX; 10 : TXDiv; 11 : 4-way TXDiv; 30 : Dynamic Open Loop MIMO; 40 : Closed Loop Mimo; 41 : Closed Loop MIMO (4x2); 43 : Closed Loop MIMO (4x4) 50: MASSIVEMIMO_SINGLE_BEAM_64x64 60: MASSIVEMIMO_DUAL_BEAM_64x64 70: Closed Loop Spatial Multiplexing_8x4 |
| Cell Status | Mandatory | - Under each Cell of a gnodeB (Class= “NRCELL”) - Take the value from attribute <p name="administrativeState">Locked</p> - If value is equal to Unlocked, then the Cell will be in ACTIVE state. - If value is equal to Locked, then the Cell will be in HALT state. - If value is coming in Numeric below mentioned mapping is there: Value; Name. 0; Locked. 1; Shutting Down. 2; Unlocked. |
| Manufacturer | Optional | - Nokia |
| maximumTransmissionPower | Mandatory | Get the value of attribute “pMax” under class=”NRCELL” and divide it by 10. So maximumTransmissionPower will be: 37 |
| NR GSCN | Mandatory | Get the value of attribute <p name="gscnOrSsPbchArfcn">7798</p> under class=”NRCELL” So, NR GSCN will be 7798 |
| Cell Type | Mandatory | Get the value of attribute “nrCellType” under class=”NRCELL” And map the value as per given below table: If it is in numeric format the following are: Value; Name. 20; 2DL0UL. 21; 2DL1UL. 22; 2DL2UL. 24; 2DL4UL. 42; 4DL2UL. 44; 4DL4UL. 82; 8DL2UL. 84; 8DL4UL. 88; 8DL8UL. 164; 16DL4UL. 168; 16DL8UL. 1616; 16DL16UL. So, Cell Type will be 4DL2UL |
| RSI | Mandatory | Get the value of attribute “prachRootSequenceIndex” under class=”NRCELL” So, the RSI will be “760” |
| Azimuth | Optional | Get the value of attribute “antBearing” from class=”RAEU_R” after cell to AMT mapping given in section 4.4. So, Azimuth will be 2415 |
| M Tilt | Optional | Get the value of attribute “mechanicalAngle” from class=”RAEU_R” after cell to AMT mapping given in section 4.4. So, M Tilt will be “-4” |
| Antenna Height(m) | Optional | Get the value of attribute “antAltitude” from class=”RAEU_R” after cell to AMT mapping given in section 4.4. Devide “antAltitude” value by 10 So Antenna Height(m) will be “61.6” |
| AMT Operational State | Optional | Get the value of attribute “operationalState” from class=”RAEU_R” after cell to AMT mapping given in section 4.4. So AMT Operational State will be “1” |
| AMT Model No | Optional | Get the value of attribute “antModel” from class=”RAEU_R” after cell to AMT mapping given in section 4.4. So AMT Model No will be “SXQ4C1765IVTBFR” |
| AMT Serial No | Optional | Get the value of attribute “antSerial” from class=”RAEU_R” after cell to AMT mapping given in section 4.4. AMT Serial No will be “0000AV21078000184” |
| AMT Make | Optional |  |
| RET E Tilt | Mandatory | Get the value of attribute “tiltOffset” under class=”NRCELL” And map the value as per given below table: Value; Tilt Value. -4; 2. -3; 3. -2; 4. -1; 5. 0; 6. 1; 7. 2; 8. 3; 9. 4; 10. 5; 11. 6; 12. 7; 13. 8; 14. So, RET E Tilt will be 6 |
| RET Operational state | Optional | Get the value from RET Operational state Section 4.5 |
| Antenna Model | Optional |  |
| Beamset | Mandatory | - under class=“NRCELL” get the value of “beamSetInUse” as Beamset |
| Cell radius | Optional |  |
| SSB Power | Mandatory | - under class=“NRCELL” get the value of “ssPbchBlockPower” as SSB Power |
| SSB power Boost | Mandatory | - under class=“NRCELL” get the value of “ssPbchBlockPower” as SSB Power Boost |
| Sub frame configuration | Mandatory | - under class=“NRCELL” get the value of “frameStructureType” as Sub frame configuration |
| Special sub frame configuration | Mandatory | under class=“NRCELL” get the value of “ulDlDataSlotRatio” as Special sub frame configuration |
| RU Model | Optional | - Check attribute value “name="rModDNList">” under class=” NRCELL” So value got from the rule is “MRBTS-365006/EQM-1/APEQM-1/RMOD-1” - Match the value got from Step-1 with "configDN" attribute under class=”RMOD_R” and if matched get the value of attribute “productName” from class=”RMOD_R” as RU Model. So, RU Model will be: “AZQS” |
| RU serial number | Optional | - Check attribute value “name="rModDNList">” under class=” NRCELL” So value got from the rule is “MRBTS-365006/EQM-1/APEQM-1/RMOD-1” - Match the value got from Step-1 with "configDN" attribute under class=”RMOD_R” and if matched get the value of attribute “serialNumber” from class=”RMOD_R” as RU Model. So, RU serial number will be: “1M212807001” |
| GPS Longitude | Optional | Get the value of attribute <p name="gnssAntennaLongitude">102936311</p> from class=”GNSSE_R”. Rule of conversion - Unit in dump = 1/10,000 arc-seconds. - 1 degree = 3,600 arc-seconds. - So, to convert to decimal degrees (°): Decimal Degrees=Value/ (10,000×3600) Exp: gnssAntennaLatitude = 368729156 Latitude= (368729156)/ (10000×3600) = (368729156)/ (36000000)​ = 10.2424776° Latitude ≈ 10.24248° |
| GPS Latitude | Optional | Get the value of attribute <p name="gnssAntennaLongitude">102936311</p> from class=”GNSSE_R”. Rule of conversion - Unit in dump = 1/10,000 arc-seconds. - 1 degree = 3,600 arc-seconds. - So, to convert to decimal degrees (°): Decimal Degrees=Value/ (10,000×3600) Exp: gnssAntennaLongitude = 102936311 Longitude= (102936311)/ (10000*3600) = (102936311)/ (36000000) = 2.85934197° Longitude ≈ 2.85934° |

### Cell to AMT mapping: (Ignore for now)

- Check attribute value “name="rModDNList">” under class=” NRCELL”

So attribute value is “MRBTS-356341/EQM-1/APEQM-1/RMOD-1” from the rule

- Match attribute value got from Step-1 with attribute name="firstEndpointDN" under class="LOGLINK" i.e. “MRBTS-356341/EQM-1/APEQM-1/RMOD-1/RSL-1” we can ignore RSL-1 while matching

- Get the value of name="secondEndpointDN" from same class after matching found from above. So secondEndpointDN value will be MRBTS-356341/EQM-1/APEQM-1/ALD-1

- Match the attribute value got from Step-3 i.e. MRBTS-356341/EQM-1/APEQM-1/ALD-1 with attribute "configDN" under class=” RAEU_R”

- Get the required attributes from this class=”RAEU_R” with respect to cell after mapping

### Cell to RET Mapping: (Ignore for now)

RET parameter will parse as below.

- From class=”NRBTS” take MRBTS for that node.

- check class=”NRCELLL” for that NRBTS

- From the above step we got “NRCELL-1” so we match “LCELNR-1” in distName under class= “CHANNEL”

Example:

distName="PLMN-PLMN/MRBTS-365006/MNL-1/MNLENT-1/CELLMAPPING-1/LCELNR-1

under class”CHANNEL” take attribute name=”antlDN”

example:

<p name="antlDN">MRBTS-365006/EQM-1/APEQM-1/RMOD-1/ANTL-1</p>

If antlDN did not found take attribute “ressourceDN”.

- Match ” antlDN” value got from Step-3 with “configDN” attribute under class=”ANTL_R” , if matched get distName of class=”ANTL_R” starts from MRBTS

i.e. MRBTS-365006/EQM_R-4/APEQM_R-1/RMOD_R-1/ANTL_R-1

- Match distName value got from Step-4 with “antlDNList” values under class “RETU_R” for same MRBTS,

SS:-

- If Matched get the values of attributes from class=”RETU__R”

This is how cell will map to RETU_R class.

- Below attributes will taken from class=”RETU_R as per step 7.

| Object Type | Category | Attribute | Value |
| --- | --- | --- | --- |
| NR CELL | RET | RET E Tilt |  |
| NR CELL | RET | Max E Tilt |  |
| NR CELL | RET | Min E Tilt |  |
| NR CELL | RET | RET Operational state |  |
| NR CELL | RET | Ret Subunit iD |  |
| NR CELL | RET | RET Antenna Model |  |
| NR CELL | RET | RET Antenna Serial Number |  |
| NR CELL | RET | RET Antenna Group |  |

- Below is attribute and dumps mapping: -

| Attribute | Parameters detail |
| --- | --- |
| RET E Tilt | angle |
| Max E Tilt | maxAngle |
| Min E Tilt | minAngle |
| RET Operational state | operationalState |
| Ret Subunit iD | subunitNumber |
| RET Antenna Model | antModel |
| RET Antenna Serial Number | antSerial |
| RET Antenna Group | antlDNList |

# IP Parsing gNodeB and description

x2Uplane/x2Cplane and s1Uplane IP parsing:

| Interface | Full Form (Description) | Plane | Used For | Protocol |
| --- | --- | --- | --- | --- |
| X2-U | X2 User Plane | User (data) | Data forwarding eNB↔eNB / eNB↔gNB | GTP-U/UDP |
| X2-C | X2 Control Plane | Control (signaling) | Handover prep, dual connectivity, coordination | SCTP/IP |
| S1-U | S1 User Plane | User (data) | Data transport eNB ↔ EPC SGW | GTP-U/UDP |

Note: X2-U & S1-U are same so we can consider any one of them.

- To be parsed from class=“NRBTS”, where the disttname will as below:

<managedObject class="com.nokia.srbts.nrbts:NRBTS" version="NRBTSCL24R2_2404_130" distName="MRBTS-515870/NRBTS-115870" operation="create">
where NRBTS-115870 represent the gNodeB ID

Value of <p name="ipV4AddressDN1"> under attribute <list name="x2Uplane">, <list name="s1Uplane"> and <list name="x2Cplane"> under class="NRBTS" and mapping can be done with class="IPADDRESSV4_R"

Step-1: get the value of "ipV4AddressDN1" under s1Uplane, x2Uplane and x2Cplane respectively.

i.e.:

MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-5/IPADDRESSV4-1 under s1Uplane

MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-5/IPADDRESSV4-1 under x2Uplane

MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-4/IPADDRESSV4-3 under x2Cplane

Step-2:  search the value of "ipV4AddressDN1" which is got from step-1 in the distname of class="IPADDRESSV4_R" and get the IP address from attribute “localIpAddrAllocated” under class="IPADDRESSV4_R"

For: MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-5/IPADDRESSV4-1 under s1Uplane &
MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-5/IPADDRESSV4-1 under x2Uplane

For: MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-4/IPADDRESSV4-3 under x2Cplane

Step 3: For VLAN, take the value of "ipV4AddressDN1" got in step-1 till IPIF value i.e. MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-5 and same will be search in distname of class=”IPIF” and get the value of attribute <p name="interfaceDN">.

i.e. <p name="interfaceDN">MRBTS-515870/TNLSVC-1/TNL-1/ETHSVC-1/ETHIF-1/VLANIF-4</p>

Step-4: Search the value of attribute <p name="interfaceDN"> got in step-3 in distname of class=”VLANIF” and get the value of vlanId i.e. 33.

So, the VLAN will be 33 for this IP.

Step-5: for subnet search the value of "ipV4AddressDN1" got in step-1and match with distname of class="IPADDRESSV4”

i.e. MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-5/IPADDRESSV4-1

match the value of localIpAddr value with IP address got in step-2, if both are same get the value of subnet mask from attribute “localIpPrefixLength” from class=” IPADDRESSV4”

## Physical port

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Physical port |
| Node ID | [Mandatory] | - = from the distName |
| Physical Port C/Sh/S/I/P | [Mandatory] | - To be provided later. |
| IP address | [Optional] | - = attribute "localIpAddr" |
| Subnet Mask | [Optional] | - = attribute " localIpPrefixLength " |
| VLAN | [Optional] | - = attribute " vlanId" |
| Gateway | [Optional] | - From class=“ IPRT_R” - Deduce the range of IP from attribute <p name="gateway"> and attribute <p name="destinationIpPrefixLength">. - Match the above IP address with the deduce range of IP - When matched take Gateway = attribute "gateway" |
| Description | [Optional] | - The value of attribute below in class=”VLANIF” against given VLAN. - <p name="userLabel">OM</p> |

# IP Parsing SRAN

## Physical port “SBTS” (Ignore for now as No SBTS class)

- To be parsed from class=“IPIF”, where the distName="PLMN-PLMN/SBTS-xxx/TNL-y/IPNO-z/IPIF-x where xxx represent the node ID and each x represent an interface

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Physical port |
| Node ID | [Mandatory] | - = from the distName |
| Physical Port C/Sh/S/I/P | [Mandatory] | - To be provided later. |
| IP address | [Optional] | - = attribute "localIpAddr" |
| Subnet Mask | [Optional] | - = attribute "localIpPrefixLength" |
| VLAN | [Optional] | - = attribute "vlanId" under class=“VLANIF” - Matching is described in below snapshot |
| Gateway | [Optional] | - From class=“RTPOL” - Match deduced IP address above with attribute "srcIpAddress" - Take the attribute "routingTablePtr" for the deduced "srcIpAddress" - From class=“IPRT” Put the value of "routingTablePtr" in distName="PLMN-PLMN/SBTS-5672/TNL-1/IPNO-1/Value_OF_routingTablePtr” - Gateway= attribute "gateway" |

## Physical Port “MRBTS” version “SBTS

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Physical port |
| Node ID | [Mandatory] | - = from the distName |
| Physical Port C/Sh/S/I/P | [Mandatory] | - To be provided later. |
| IP address | [Optional] | - = attribute "localIpAddr" |
| Subnet Mask | [Optional] | - = attribute " localIpPrefixLength " |
| VLAN | [Optional] | - = attribute " vlanId" |
| Gateway | [Optional] | - From class=“ IPRT_R” - Deduce the range of IP from attribute <p name="gateway"> and attribute <p name="destinationIpPrefixLength">. - Match the above IP address with the deduce range of IP - When matched take Gateway = attribute "gateway" |
| Description | [Optional] | - The value of attribute below in class=”VLANIF” against given VLAN. - <p name="userLabel">OM</p> |

## IP description Logic: (Describe in starting under section-5)

CPLANE and UPLANE:

- Get IP Address from class=” IPADDRESSV4” and class=”IPADDRESSV6”

- Match the section from above class start by MRBTS till IPIF in the distname example “MRBTS-905488/TNLSVC-1/TNL-1/IPNO-1/IPIF-2” with value under attribute “cPlaneipV4AddressDN1, ipV4AddressDN1 and ipV6AddressDN1” under the class “class="TRSNW"” where the distname start with “PLMN-PLMN/MRBTS-ID

So Description of IP address matched distname with given below attributes will be:

| Attribute | IP Description |
| --- | --- |
| cPlaneipV4AddressDN1 | CPLANE |
| ipV4AddressDN1 | UPLANE |
| ipV6AddressDN1 | ENB-ENDC |

MPLANE:

- Match the section from IPADDRESSV4 class start by MRBTS till IPIF in the distname example “MRBTS-905488/TNLSVC-1/TNL-1/IPNO-1/IPIF-3” with value under attribute “mPlaneIpv4AddressDN” under the class “class=" MPLANENW"” where the distname start with “PLMN-PLMN/MRBTS-ID

So Description of IP address matched distname with given below attributes will be:

| Attribute | IP Description |
| --- | --- |
| mPlaneIpv4AddressDN | MPLANE |

ABIS and BTS-MPLANE:

IP which are not able to get description from above 2 points, search the IP from class=”BCF” under attribute btsCuPlaneIpAddress and btsMPlaneIpAddress.

If IP available under btsCuPlaneIpAddress class IP description will be “ABIS”

If IP available under btsMPlaneIpAddress class IP description will be “BTS-MPLANE”

x2Uplane and s1Uplane IP parsing:

Description of IP will be as follows:

IP got from s1Uplane: NR-UPLANE

IP got from x2Uplane: NR- ENDC

- To be parsed from class=“NRBTS”, where the distName=" PLMN-PLMN/MRBTS-1585637/NRBTS-XXX  where xxx represent the gNodeB ID

Value of name=" ipV6AddressDN1 under attribute name="x2Uplane">, name="s1Uplane"> under class="NRBTS" and mapping can be done with class="IPADDRESSV6_R"

Step-1: get the value of ipV6AddressDN1 under x2Uplane and s1Uplane

i.e.:

MRBTS-365006/TNLSVC-1/TNL-1/IPNO-1/IPIF-1/IPADDRESSV6-1

IPV6:

Step-2:  search the value of ipV6AddressDN1 got from step 1 in the distname of class="IPADDRESSV6_R" and get the IP address from attribute “localIpAddrAllocated” under class="IPADDRESSV6_R"

Step 3:  for vlan search the value of =" ipV6AddressDN1" got in step no 1 till IPIF value i.e. “MRBTS-901227/TNLSVC-1/TNL-1/IPNO-1/IPIF-10” in distname of class=”IPIF” and get the value of attribute “interfaceDN"

i.e. MRBTS-901227/TNLSVC-1/TNL-1/ETHSVC-1/ETHIF-1/VLANIF-10

Step-4: search the value of attribute “interfaceDN" got in step 3 in distname of class=”VLANIF” and get the value of vlanId i.e. 3050.

So vlan will be 3050 for the IP

Step-5: for subnet search the value of “ipV4AddressDN1” got in STEP 1and match with distname of class="IPADDRESSV6”

i.e. MRBTS-365006/TNLSVC-1/TNL-1/IPNO-1/IPIF-1/IPADDRESSV6-1

match the value of localIpAddr value with IP address got in step no 2, if both are same get the value of subnetmask from attribute “localIpPrefixLength” from class=” IPADDRESSV6” or class=” IPADDRESSV4”

# Additional info for gNodeB (Ignore for now as no SA-gNodeB)

## 6.2 NR-S1-U

- This section related to MRBTS dump which contain class=”NRBTS”

- From class=“NRBTS”, from the distName="MRBTS-515870/NRBTS-115870" where after “NRBTS-” parse the gNodeB ID.

- Value of name=“ipV4AddressDN1”under attribute name="s1Uplane">  under class="NRBTS" and mapping can be done with class="IPADDRESSV4_R"

Step-1: Get the value of “ipV4AddressDN1” under <list name="s1Uplane">

i.e. MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-5/IPADDRESSV4-1

Step-2:  Search the value of “ipV4AddressDN1” got from step-1 in the distname of class="IPADDRESSV4_R" and get the IP address from attribute “localIpAddrAllocated” under class="IPADDRESSV4_R"

- In the same class take the Local_IP from the attribute " localIpAddrAllocated "

- From class="GTPU", where the distName="PLMN-PLMN/MRBTS-365006/NRBTS-XXX/SERVEDAMF-Y" where xxx is the gNodeB ID, each Y represent an additional info

- From the same class, each attribute "sgwIpAddress" represent one remote IP, increase the attribute of the additional info by 1 for each remote IP

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | MRBTS-ID/GTPU-Y | - CONNECTION_TYPE=NR-S1-U - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | MRBTS-ID/GTPU-Y | - LOCAL_IP= the value of " localIpAddrAllocated " in class="IPADDRESSV6_R" or REMOTE_IP= the value of "sgwIpAddress" in class=“ GTPU” |

## 6.3 Lat/Long parsing rule gnodeB

Under <managedObject class="GNSSE_R" take value of <p name="gnssAntennaLatitude"> for latitude and <p name="gnssAntennaLongitude"> for Longitude and divide it with 10 million (10^7).

Take the corresponding value of parent node for MRBTS.

| Category | Attribute | Value |
| --- | --- | --- |
| gnodeb | latitude | Value of latitude |
| gnodeb | longitude | Value of longitude |

## 5G Cell to RRU Mapping

- From class=”NRBTS” take MRBTS for that node.

- check class=”NRCELL” for that MRBTS

- From the above step we got “NRCELL-3” so we match “LCELNR-3” in distName under class= “CHANNEL”

Example:

distName="PLMN-PLMN/ MRBTS-1585637/MNL-1/MNLENT-1/CELLMAPPING-1/LCELNR-3

under class”CHANNEL” take attribute name=”antlDN”

example:

<p name="antlDN">MRBTS-1585637/EQM-1/APEQM-1/RMOD-3/ANTL-1</p>

If antlDN did not found, take attribute “ressourceDN”.

- under class”CHANNEL” take attribute name=”antlDN”/resourceDN  till RMOD type, and match it with attribute name=”configDN” under class= ”RMOD_R”

so, we match attribute like given below screenshots:

name=”antlDN

<p name="antlDN">MRBTS-1585637/EQM-1/APEQM-1/RMOD-3

name=”configDN”

i.e. <p name="configDN"> MRBTS-1585637/EQM-1/APEQM-1/RMOD-3</p>

For cell to RRU mapping from class="RMOD_R”, parse the value of name="productName" in RU Model and name="serialNumber" in Serial Number of additionalinfo.

| Object ID | Object Type | Attribute | Value | Category |
| --- | --- | --- | --- | --- |
| CELLID | NRCell | Serial Number | Value of SerialNumber: Value of SerialNumber | RADIO |
| CELLID | NRCell | RU Model | Value of productName: Value of productName | RADIO |

## Physical Site ID

- Get the value of “siteTemplateName” attribute under class=”NRBTS” and map with the node.

- So Physical Site ID will be: KLVLPT02113ID11

| elementId | elementType | attribute | Value | category | note |
| --- | --- | --- | --- | --- | --- |
| MRBTS-123 | gNodeB/SRAN | Physical Site Id | Value of attribute “siteTemplateName” | Physical Site |  |

# Additional info for sranbs

## 7.2 NR-S1-U

- This section related to MRBTS dump which contain class=”NRBTS”

- From class=“NRBTS”, from the distName="MRBTS-515870/NRBTS-115870" where after “NRBTS-” parse the gNodeB ID.

- Value of name=“ipV4AddressDN1”under attribute name="s1Uplane">  under class="NRBTS" and mapping can be done with class="IPADDRESSV4_R"

Step-1: Get the value of “ipV4AddressDN1” under <list name="s1Uplane">

i.e. MRBTS-515870/TNLSVC-1/TNL-1/IPNO-1/IPIF-5/IPADDRESSV4-1

Step-2:  Search the value of “ipV4AddressDN1” got from step-1 in the distname of class="IPADDRESSV4_R" and get the IP address from attribute “localIpAddrAllocated” under class="IPADDRESSV4_R"

- In the same class take the Local_IP from the attribute " localIpAddrAllocated "

- From class="GTPU", where the distName="PLMN-PLMN/MRBTS-365006/NRBTS-XXX/SERVEDAMF-Y" where xxx is the gNodeB ID, each Y represent an additional info

- From the same class, each attribute "sgwIpAddress" represent one remote IP, increase the attribute of the additional info by 1 for each remote IP

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | SRAN-ID/GTPU-Y | - CONNECTION_TYPE=NR-S1-U - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | eNodeB-ID/GTPU-Y | - LOCAL_IP= the value of " localIpAddrAllocated " in class="IPADDRESSV6_R" or REMOTE_IP= the value of "sgwIpAddress" in class=“ GTPU” |