NSN 2G Logical Parsing

FRS (Functional Requirement Specifications)

Auto-Import

|  | Indrasan Yadav |
| --- | --- |
|  | 04-06-2025 |

Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft. | Indrasan Yadav | 16-05-2025 |
| 0.2 | Updated the rule to parse GSM frequency band (via static sheet) under section 5 highlighted with green color. | Indrasan Yadav | 20-01-2026 |

Table of Contents

# PREAMBLE

- This document specifies how to parse the logical inventory attributes for the BSC, BTS, and Cell logical inventory.

- It covers the parameters defined for 2G RAN Logical attributes

- The provided dump files are of “.xml”, named as “BSC-xxxxxx_ACTUAL_EXPORT_RESULT” where the “x” are digits.

  - E.g.: BSC-385502_ACTUAL_EXPORT_RESULT

- OR one dumps for all nodes, dump name is ActExp_full_DATE

- Only files with “<managedObject class="BSC”” are to be taken into consideration for this connector.

- Each “.xml” file represents the whole configuration network for each BSC, it served BTSs, and its cells.

- All the “.xml” files are expected to exist in one location folder; folder location and name will be provided later.

- This is motivated by Azer-connect Project and can be used for other operators having the same file structure.

- Another source type of dump files could be provided later for Antenna attributes.

- NOTE: The internal mapping between the physical and logical connectors will be based on the Node ID.

# Connector Information

- Each “.xml” file is composed of different class types (i.e. BSC, BTS, BCF, TRX...)

Below is a description of the main class types

- Each class=”BSC” represents a BSC node.

- Each class=”BCF” represents a new BTS node.

- Each class="BTS" represents the served cells for the parent BTS.

- Each class=“ADCE” or class=”ADJW” or class=”ADJL” represents the neighbouring 2G, 3G and 4G cells respectively for a specific cell mentioned under class=”BTS”.

Each class=”TRX” represents the specific TRX’s information of the parent cell.

# Connector name

NSN_RAN_Ooredoo_Tunisia

# BSC logical parameters

The BSC logical inventory attributes are classified accordingly in categories as follows:

  - Global Parameters

  - Networking Parameters

### Global Parameters

- The global attributes mentioned below and defined at BSC level are to be extracted as follows:

| Attribute | Importance | Example |
| --- | --- | --- |
| Site Name | [Mandatory] | - Describe later |
| Node ID | [Mandatory] | - = “distName” attribute on the same line as: <managedObject in class="BSC" - E.g.: distName="PLMN-PLMN/BSC-415338" Node ID = BSC-415338 |
| Node Name | [Mandatory] | - = “name” attribute under <managedObject class=”BSC” - E.g.: name="name">BSCMNB06</p> Node Name = BSCMNB06 |
| Type | [Mandatory] | - BSC OR - Deduced from physical connector. - Else couldn’t find it is extracted from the first <managedObject “class” type mentioned in the beginning of the file, as follows: If class=”BSC”, therefore Type = BSC. Else, if the first “class” type is different from “BSC”, the file should be ignored. |
| OAM IP | [Optional] | - =null |
| OAM Subnet Mask | [Optional] | - =null |

### Networking Parameters

- The networking attributes mentioned below and defined at BSC level are to be extracted as follows:

| Attributes | Importance | Example |
| --- | --- | --- |
| MCC | [Mandatory] | - Extracted from “locationAreaIdMCC” attribute under <managedObject class="BTS”. - It is sufficient to parse only one value from the “locationAreaIdMCC” attribute across the entire file. - E.g.: name=” locationAreaIdMCC”>655</p> |
| MNC | [Mandatory] | - Extracted from “locationAreaIdMNC” attribute under <managedObject class="BTS”. - It is sufficient to parse only one value from the “locationAreaIdMNC” attribute across the entire file. - E.g.: name=“locationAreaIdMNC”>03</p> |
| Parent MSC | [Optional] | - check within file name “Parent_MSC_SGSN.xlsx” within sheet MSC if the RNC Name is matched with column A, then set the Parent MSC attribute equal to value in Column B. - Else, not found, default value =Null - Note: “Parent_MSC_SGSN.xlsx” file will be provided later |
| Parent SGSN | [Optional] | - Check within file name “Parent_MSC_SGSN.xlsx” in sheet SGSB if the RNC Name is matched with column A, then set the Parent SGSN attribute equal to value in Column B. - Else, not found, default value =Null Note: “Parent_MSC_SGSN.xlsx” file will be provided later. |
| Attached BTSs | [Mandatory] | - (NEP tool calculated) |
| TRAU Boards | [Optional] | - Refer to TRAU Logical attributes section. - All TRAU boards parsed from a BSC dump file to be assigned to the same BSC. |
| PCU card location | [Optional] | - NEP tool calculated |
| PCU Card Type | [Optional] | - To be deduced from both Physical and Logical dumps as per below: - From Logical Dumps, extract the value of the attribute “bcsuID” stated under <managedObject class="PCU”. - If “bcsuID” was found multiple times, then take only first value. - Find in matching physical dump the Unit List supporting Functional Role=”BCSU” and “functionalUnitId” equal to the value of BSCU extracted from logical dump. - Only boards with board type starting with “PCU” to be considered, e.g. “PCU2_E”. - PCU Board Type= Board Type of above boards. - Please refer to below photo. |
| PCU Card ID | [Optional] | - Is the concatenation of below attributes separated with “_”: - “Cartridge ID”. - “Cartridge shelf Index” - “Board Slot Index” - E.g. “NE-FlexiBSC-373889_1_4_1_2” |
| PCU Card Name | [Optional] | - =PCU Card Type |
| PCU Card Parent SGSN | [Optional] | - =Null |

### IP Parsing

- This section to be parsed from a separated dump

- Each section will start with date and time than the BSC ID and name

- Under ZQRI section

- Example: --------

Each section lists Network Interfaces of the OMU with their status and IP information:

- UNIT: The hardware/software unit being queried (here, OMU).

- INTERFACE (EL0, EL1, EL4, EL5, EMB0, EMB1): These are logical Ethernet interfaces of the OMU.

- ADMIN STATE: Indicates whether the interface is UP (active) or DOWN.

- MTU (Maximum Transmission Unit): Packet size supported, typically 1500 bytes.

- ATTR (Attribute): Interface role, like:

  - PI – Primary Interface

  - P – Physical Interface

- IP ADDRESS: The configured IP with subnet (e.g., 10.20.98.4/24).

- Usage Label (right side, not part of table but shown below each IP): Defines purpose of IP interface:

  - NTP → Used for time synchronization.

  - Monitoring → Used for monitoring traffic / O&M access.

  - OAM (Operations, Administration, Maintenance) → Used for management access to BSC.

- Example:

Meaning:

- EL0 → Interface used for NTP sync (clock).

- EL1 → Interface for Monitoring network.

- EL4 → Interface for OAM traffic (remote management).

- EL5 → Interface is UP but no IP configured.

- EMB0/EMB1 → Backbone transport interfaces with broader subnets (likely for Abis traffic between BSC and BTS).

| Attributes | Importance | Example |
| --- | --- | --- |
| Interface name | [Mandatory] | - = NodeID_IP_Mask - Example: BSCMNB06_10.203.7.158_Mask (28) |
| IP Address | [Mandatory] | - To be parsed from header IP - For OAM - In the same line 10.203.7.158 with EL4 |
| Mask | [Mandatory] | - After the IP Address and separated by / |
| VLAN | [Mandatory] | - 506 (Shared by Ooredoo over an email) |
| Gateway | [Mandatory] | - For BSCMNB06 (10.203.13.1) And And, for BSCMGH03 (10.107.38.1) Shared by Ooredoo in a separate file over an same email of VLAN |
| Description | [Optional] | - It is available in dump not part of table but shown below each IP. Defines purpose of IP interface. |

# BTS 2G logical Attributes

The BTS 2G logical inventory attributes are classified accordingly in categories as follows:

- Global Parameters

- Networking Parameters

### Global Parameters for BTS

- The global attributes mentioned below and defined at BTS level are to be extracted as follows:

| Attribute | Importance | Example |
| --- | --- | --- |
| Node ID | [Mandatory] | - = “distName” attribute on the same line as <managedObject class=”BCF” - E.g.: distName="PLMN-PLMN/BSC-248098/BCF-1011" Node ID = PLMN-PLMN/BSC-248098/BCF-1011 |
| Node Name | [Mandatory] | - = “name” attribute under <managedObject class=”BCF” - E.g.: name="name">2G_Northwood_KZN</p> Node Name = 2G_Northwood_KZN |
| Site Name | [Mandatory] | - Describe later |
| Type | [Mandatory] | - deduced from node configuration |
| PS Technology | [Optional] | - The PS Technology can have the following values: GPRS, EDGE, None, or both GPRS and EDGE. - This can be deduced from the served cells mentioned in the below section (Cell Logical Configuration): - If one of the served cells has both GPRS and EDGE enabled, then the parent node would have GPRS and EDGE as PS Technology. - Else if all the served cells have only EDGE enabled and GPRS not enabled, then the PS technology will be set to “EDGE”. - Else if all the served cells have only GPRS enabled and EDGE not enabled, then the PS technology will be set to “GRPS”. - Else if none of them is enabled then the PS Technology will be then set to “NONE”. |
| OAM IP | [Optional] | - =Null |
| OAM Subnet Mask | [Optional] | - =Null |

| Status | [Mandatory] | - deduced from “AZCN_2G_SITE_STATUS_20” file from Sites folder, where the status of the SITE ID is equal to “ON_AIR” or HUB_ON_AIR” - If the Status is empty keep the value blank - Exclude the nodes with status “DEFINED” |
| --- | --- | --- |

### Global Parameters for SRAN

- The global attributes mentioned below and defined at SRAN Level for 2G role are to be extracted as follows:

| Attribute | Importance | Example |
| --- | --- | --- |
| Node ID | [Mandatory] | - = same as physical |
| Node Name | [Mandatory] | = same as physical |
| Site Name | [Mandatory] | - = same as physical |
| Type | [Mandatory] | - = same as physical |
| PS Technology | [Optional] | - PS Technology can have the following values: GPRS, EDGE, None, or both GPRS and EDGE. - This can be deduced from the served cells mentioned in the below section (Cell Logical Configuration): - If one of the served cells has both GPRS and EDGE enabled, then the parent node would have GPRS and EDGE as PS Technology. - Else if all the served cells have only EDGE enabled and GPRS not enabled, then the PS technology will be set to “EDGE”. - Else if all the served cells have only GPRS enabled and EDGE not enabled, then the PS technology will be set to “GRPS”. - Else if none of them is enabled then the PS Technology will be then set to “NONE”. |
| OAM IP | [Optional] | - =Null |
| OAM Subnet Mask | [Optional] | - =Null |

### Networking Parameters for BTS

- The networking attributes mentioned below and defined at BTS level are to be extracted as follows:

| Attributes | Importance | Example |
| --- | --- | --- |
| MCC | [Mandatory] | - (NEP tool calculated) |
| MNC | [Mandatory] | - (NEP tool calculated) |
| LAC | [Optional] | - (NEP tool calculated) |
| RAC | [Optional] | - (NEP tool calculated) |
| Parent BSC | [Mandatory] | - = “name” attribute under <managedObject class=“BSC” - E.g. : name=“name”>2G_UH11_DTJ_KZN</p> OR - From the managedObject class="BCF", in the same line after distName= BSC ID will be after BSC- - Example: - <managedObject class="BCF" version="S16_3" distName="PLMN-PLMN/BSC-206967/BCF-195" id="7286135"> - BSC ID= 206967 |
| Attached MSC | [Optional] | - Deduced from parent BSC |
| Attached SGSN | [Optional] | - Deduced from parent BSC |
| Served cells | [Optional] | - (NEP tool calculated) |

### Networking Parameters for SRAN

- The networking attributes mentioned below and defined at SRAN for 2G role are to be extracted as follows:

| Attributes | Importance | Example |
| --- | --- | --- |
| MCC | [Mandatory] | - (NEP tool calculated) |
| MNC | [Mandatory] | - (NEP tool calculated) |
| LAC | [Optional] | - (NEP tool calculated) |
| RAC | [Optional] | - (NEP tool calculated) |
| Parent BSC | [Mandatory] | - Under class="GNBCF" - =the Value of Attribute bscId - Map the value of bscid to the bsc name like in BSC section |
| Attached MSC | [Optional] | - Deduced from parent BSC |
| Attached SGSN | [Optional] | - Deduced from parent BSC |
| Served cells | [Optional] | - (NEP tool calculated) |

### Logical IP Parsing for BTS

- If the BTS is standalone, then parse directly the IPs

- If the BTS is a 2G Role under SRAN, then Parse the IP if it does not exist under the SRAN

| Attributes | Importance | Example |
| --- | --- | --- |
| Interface name | [Mandatory] | - = Node-Name_mPlan |
| IP Address | [Mandatory] | - Under class=“BCF” - = attribute “btsMPlaneIpAddress” - If above attribute is missing, then check attribute mPlaneLocalIpAddress in class “PABTRS” |
| Mask | [Mandatory] | - Under class=“BCF” - =attribute “btsIpv4SubnetMasklengthMplane” or "btsSubnetMasklengthMplane" - If above attribute is missing, then check attribute mPlaneSubnetMask in class “PABTRS” |
| GW | [Mandatory] | - Under class=“PABTRS” - =attribute “mPlaneGatewayIpAddress” |
| VLAN | [Mandatory] | - Under class=“PABTRS” - =attribute “mPlaneVlanId” |

| Attributes | Importance | Example |
| --- | --- | --- |
| Interface name | [Mandatory] | - = Node-Name_cPlan |
| IP Address | [Mandatory] | - Under class=“BCF” - = attribute “btsCuPlaneIpAddress” |
| Mask | [Mandatory] | - Under class=“BCF” - =attribute “btsIpv4SubnetMasklengthCUplane” Or =attribute “btsSubnetMasklengthCUplane" Or “btsCuPlaneIpAddress” |
| GW | [Mandatory] | - Under class=“PABTRS” - =attribute “cuPlaneGatewayIpAddress” |
| VLAN | [Optional] | - Under class=“PABTRS” - =attribute “cPlaneVlanId” |

### Logical IP Parsing for SRAN

- If the BTS is a 2G Role under SRAN, then Parse the below information

| Attributes | Importance | Example |
| --- | --- | --- |
| Interface name | [Mandatory] | - = Node-Name_mPlan |
| IP Address | [Mandatory] | - Under class=“GNBCF” - Take the value of “mPlaneLocalIpAddressDN” - Search the value in the same MRBTS for class="IPADDRESSV4" - The value of localIpAddr will be the IP address of SRAN with 2G role |
| Mask | [Mandatory] | - Under same class=“ IPADDRESSV4” - =Take attribute “localIpPrefixLength”. Map it with the table below for subnet mask localIpPrefixLength; Subnet Mask. 22; 255.255.252.0. 23; 255.255.254.0. 24; 255.255.255.0. 25; 255.255.255.128. 26; 255.255.255.192. 27; 255.255.255.224. 28; 255.255.255.240. 29; 255.255.255.248. 30; 255.255.255.252. |
| GW | [Mandatory] | - Search the dist name of class=“ mPlaneLocalIpAddressDN” in class="IPRT" without “/IPADDRESSV4-X” & “IPIF “ will be replaced by “IPRT” - The value of attribute “gateway “will be considered as GW for SRAN |
| VLAN | [Mandatory] | - Under class=“VLANIF” for the same MRBTS, check for the attribute “userLabel” - If the “userLabel” value is “GSM” check the attribute VlanId under this attribute. - =The value of “VlanId” is the VLAN for 2G role node of SRAN Or If userLabel is not found, follow the process below. - From class=” IPADDRESSV4” take the value of distname - Replace “IPNO-1/IPIF” with “ETHSVC-1/ETHIF-1/VLANIF” - Ignore the section “/IPADDRESSV4-X” in class=” IPADDRESSV4” - Match the value with distname of class="VLANIF" from the above deduced section - The value of vlanId under this class is the VLAN ID. |

# Cell LoGICAL PARAMETERS

| Attribute | Importance | Example |
| --- | --- | --- |
| Cell Name | [Optional] | - Deduced from name="name" attribute under each <managedObject class="BTS": - E.g.: |
| Cell Object ID | [Mandatory] | - The “Cell Object ID” is equal to the concatenation of: “LAC”, “_” and “Cell_ID” - Cell_ID is deduced from name="cellId" attribute under each <managedObject class="BTS": - E.g.: name="cellId">18851</p> - “LAC” is deduced through the attribute name “locationAreaIdLAC” within the same class. - E.g.: <p name="locationAreaIdLAC">1482</p> - Cell_ID = 1482_18851 - In case detected same “Cell_ID” value under the same node, thus only the 2nd detected duplicate cell ID will be a concatenation of “LAC”,”_”,”Cell_ID”,”-”,X”. - To get the value of “X”, get the value of “frequencyBandInUse” under <managedObject class=“BTS” for the 2nd detected duplicate cell ID and match it within the below table in column A. - The value in Column B will be then the value of “X” - E.g. Cell ID 1st=1482_18851 - Cell ID 2 (duplicate) =1482_18851-1800 Column A; Column B. 0; 900. 1; 1800. 2; 1900. 5; 800. Note: This processes it to be executed only for the 2nd detected Cell_ID. |
| Cell ID |  | - Cell_ID is deduced from name="cellId" attribute under each <managedObject class="BTS": - E.g.: name="cellId">18851</p> |
| Node ID | [Mandatory] | - Deduced from "distName" attribute on the same line as <managedObject class="BCF"attribute. - E.g.: distName="PLMN-PLMN/BSC-415338/BCF-1 |
| Parent Site ID | [Mandatory] | - Deduced from parent Node at saving |
| Cell Technology | [Mandatory] | - (NEP tool calculated) - Else = GSM |
| MCC | [Mandatory] | - (NEP tool calculated) |
| MNC | [Mandatory] | - (NEP tool calculated) |
| Operators | [Optional] | - (NEP tool calculated) |
| Neighbor Cells 2G | [Optional] | - Each <managedObject class="ADCE" mentioned under each <managedObject class="BTS" refers to a “Neighbor Cell 2G” for the mentioned Cell Name already parsed under <managedObject class=“BTS”. - Thus, the “Neighbor Cells 2G” attribute is obtained by extracting the name="adjacentCellIdCI" attribute and the name= "adjacentCellIdLac" mentioned under each <managedObject class=“ADCE”. - E.g.: name="adjacentCellIdCI">16337</p> - E.g: name="adjacentCellIdLac">1482</p> - Neighbor Cell 2G Object ID is equal to the concatenation of: “LAC”, “_” and “Cell_ID” - In the example mentioned above - Object ID= 1482_16337 |
| Neighbor Cells 3G | [Optional] | - Each <managedObject class="ADJW" mentioned under each <managedObject class="BTS" refers to a “Neighbor Cell 3G” for the mentioned Cell Name already parsed under <managedObject class="BTS". - Thus, the “Neighbor Cells 3G” attribute is obtained by extracting the name="AdjwCId” attribute and the attribute name=“lac” mentioned under each <managedObject class=“ADJW”. - E.g.: name="AdjwCId">1</p> - E.g: name=“Lac”>1</p> - Neighbor’s Cell Object ID= 1_1 And, - E.g.: name="AdjwCId">4</p> - E.g: name=“Lac”>1</p> - Neighbor’s Cell Object ID= 1_4 |
| Neighbor Cells 4G | [Optional] | - (to be provided later) |
| Last Audit Date | [Optional] | - =Null |
| Additional Information | [Optional] | - =Null |

Below are the specific attributes that need to be parsed in case the cell technology is GSM

- Specific attributes for 2G cells

| Attributes | Importance | Example |
| --- | --- | --- |
| Parent BSC ID | [Optional] | - NEP tool calculated. |
| BCCH ARFCNs | [Optional] | - NEP tool calculated. - Else through the following rule: - Lookup under <managedObject class=“ADCE” for the same "Cell Name" mentioned under name="name". - Once matched, the "BCCH ARFCN" will be obtained by extracting name="bcchFrequency" attribute under the same <managedObject class=“ADCE” where the “Cell Name” was matched. - The same “Cell Name” with the same “bcchFrequency “could be duplicated under different objects having <managedObject class=“ADCE”. Thus, ignore the repeated values. |
| #TRX installed | [Optional] | - (NEP tool calculated) |
| BSIC | [Optional] | - NEP Tool calculated |
| NCC | [Optional] | - = name="bsIdentityCodeNCC” attribute found under each line containing <managedObject class=“BTS”. - The obtained value refers to the cell ID value of name="cellId” attribute under the same class=“BTS”. |
| BCC | [Optional] | - = name="bsIdentityCodeBCC” attribute found under each line containing <managedObject class=“BTS”. - The obtained value refers to the cell ID value of name="cellId” attribute under the same class=“BTS”. |
| HSN | [Optional] | - = name="hoppingSequenceNumber1” attribute found under each line containing <managedObject class=“BTS”. - The obtained value refers to the cell ID value of name="cellId” attribute under the same class=“BTS”. |
| MAIO List | [Optional] | - NEP tool calculated. |
| LAC | [Optional] | - = name="locationAreaIdLAC" attribute found under each line containing <managedObject class=“BTS” - E.g. : name="locationAreaIdLAC">307</p> LAC = 307 - The obtained value refers to the cell ID value of name="cellId" attribute under the same class=“BTS” |
| RAC | [Optional] | - = name="rac" attribute found under each line containing <managedObject class=“BTS” - E.g. : name="rac">107</p> RAC = 107 - The obtained value refers to the cell ID value of name="cellId” attribute under the same <managedObject class=“BTS” |
| GPRS enabled | [Optional] | - A cell has “GPRS enabled” if name="gprsEnabled" attribute under <managedObject class=“BTS” is equal to “1”. - E.g.: name="gprsEnabled">1</p> - if name="gprsEnabled" attribute under <managedObject class=“BTS” is equal to “0”, the Default value = NO - The obtained value refers to the cell ID value of name="cellId" attribute under the same <managedObject class=“BTS” |
| EDGE enabled | [Optional] | - A cell has “EDGE enabled” if name="egprsEnabled" attribute under <managedObject class=“BTS” is equal to “1”. - E.g.: name="egprsEnabled">1</p> - if name="gprsEnabled" attribute under <managedObject class=“BTS” is equal to “0”, the Default value = NO - The obtained value refers to the cell ID value of name="cellId" attribute under the same <managedObject class=“BTS” |
| Frequency bands | [Optional] | - Deduced by extracting the value of name="frequencyBandInUse" attribute stated under each <managedObject class=“BTS”, where the parsed value will refer to Cell Name mentioned as the value of the name=“name” attribute under the same class type (<managedObject class= “BTS”). - Check if the value of name="frequencyBandInUse" attribute is equal to 0 or 1. - Map this value within static sheet “GSM_Frequency_Band” to parse the frequency band. |
| AMR support | [Optional] | - Check the value of “amrConfFrCodecModeSet” attribute if it is equal to 0, then AMR Support is equal to “NO”. - Else if any different value other than “0”, hence the AMR Support=YES. |
| PCU Card | [Optional] | - To be deduced from both Physical and logical dumps as per below: - Check in logical dumps “daPool_ID” value under <managedObject class="TRX" for any of the TRXs under <managedObject class=“BTS” (e.g. name="daPool_ID">1001). - PCU Card found below refers to the Cell represented with above <managedObject class="BTS". - Find <managedObject class="DAP" with “distName” value="PLMN-PLMN/BSC-BSC ID/DAP-daPool_ID"(e.g. "PLMN-PLMN/BSC-388214/DAP-1001"). - Get name="bcsuID" and name="pcuID" values under above <managedObject class="DAP”. - Find in matching physical dump the Unit List supporting Functional Role=”BCSU” and “functionalUnitId” equal to the value of name="bcsuID" extracted from logical dump. - PCU card is the PCU board in physical dumps under above deduced BSCU cartridge with unitTypeExpected="PCU2_E" and position is deduced based on “pcuID" from logical dumps as per below rule: - “pcuID"=4→ PCU2_E with lowest position in the BSCU cartridge. - “pcuID"=6→ PCU2_E with second lowest position in the BSCU cartride. - “pcuID"=8→ PCU2_E with third lowest position in the BSCU cartride. - “pcuID"=10→ PCU2_E with fourth lowest position in the BSCU cartride. - “pcuID"=12→ PCU2_E with fifth lowest position in the BSCU cartride. - Please refer to below snapshot. |

- Specific attributes for 2G TRXs in the instance level

| Attributes | Importance | Example |
| --- | --- | --- |
| TRX ID Object ID | Mandatory | - The “TRX ID” is equal to the concatenation of : “Cell ID”, “_” and “TRXID” - For each Cell, there exist multiple TRXs. All classes mentioned as <managedObject class=“TRX” stated under <managedObject class=“BTS” refer to the Cell Name mentioned under class <managedObject =“BTS” as mentioned previously. - The TRXID is deduced by extracting the value of the attribute “distName” stated on the same line as <managedObject class=“TRX” and taking the number following the keyword “TRX-” : - E.g.: distName="PLMN-PLMN/BSC-248098/BCF-1011/BTS-1011/TRX-14" TRXID = 14 |
| TRX ID |  | - The TRXID is deduced by extracting the value of the attribute “distName” stated on the same line as <managedObject class=“TRX” and taking the number following the keyword “TRX-” : - E.g.: distName="PLMN-PLMN/BSC-248098/BCF-1011/BTS-1011/TRX-14" - TRXID = 14 |
| ARFCN list | Mandatory | - Deduced by extracting the value of the name=“initialFrequency“ attribute for each <managedObject class="TRX" - E.g.: name="initialFrequency">33</p> - This value refers to the “TRX ID” as deduced by extracting the value of the attribute “distName” stated on the same line as <managedObject class=“TRX” and taking the number following the keyword “TRX-” . - Note : under each line containing (<managedObject class=“BCF”) we have many (<managedObject class=“TRX”) |
| Type | Optional | - Deduced by checking for each TRX ID the values of the attributes name="bbUnitSupportsEdge" and name="gprsEnabledTrx" mentioned under each <managedObject class=“TRX”, as follows: - If the value of both “bbUnitSupportsEdge” and “gprsEnabledTrx" is equal to “1” Type = “GPRS and EDGE” - Else if “bbUnitSupportsEdge” is equal to “1” and “gprsEnabledTrx” is equal to “0” Type = “EDGE” - Else if “bbUnitSupportsEdge” is equal to “0” and “gprsEnabledTrx” is equal to “1” Type = “GPRS”. - Else if both are equal to “0” Type = None. - This value refer to the "TRX ID" which is deduced by extracting the value of the attribute “distName” stated on the same line as <managedObject class=“TRX” and taking the number following the keyword “TRX-” . |
| MAIO | Optional | - = name="channel0Maio" value under <managedObject class="TRX". - Value refer to TRX represented with <managedObject class="TRX". |
| Half rate supported | Optional | - Deduced by checking for each TRX ID the value of the attribute name=“halfRateSupport” mentioned under each class=”TRX”, as follows: - if value=“1” Half rate supported =“Yes” - if value=“0” Half rate supported= “No” - E.g.: name="halfRateSupport">1</p> - This value refer to the "TRX ID" which is deduced by extracting the value of the attribute “distName” stated on the same line as <managedObject class=“TRX” and taking the number following the keyword “TRX-” |
| Channel type | Optional | - Deduced by checking the values of all the attributes having the shape name="channelxType" where x represents a digit under each class=”TRX” ( <managedObject class=“TRX” ) - According to the matching value, mentioned below, the “Channel type” can be deduced: - “0 or “2” or “15” Channel type = TCHFR. - “1” Channel type = TCHHR. - “3” or “8” Channel type = SDCCH - “4” or ”5” or “6” or “7”or “10” Channel type = BCCH - “14” Channel type = Fixed-GPRS - Any undefined number which is not defined in the above list is to be ignored - E.g.: - name="channel0Type">4</p> Channel type = BCCH - name="channel1Type">8</p> Channel type = SDCCH This value refer to the "TRX ID" which is deduced by extracting the value of the attribute “distName” stated on the same line as <managedObject class=“TRX” and taking the number following the keyword “TRX-” |
| Number of PDCH | Optional | - (NEP tool calculated) |
| Number of BCCH/SDCCH control channels | Optional | - (NEP tool calculated) |
| status | Optional | - Under the above class TRX, take the attribute “adminState” - If value equal to 1 then it is active - If the value equal to |

# TRAU Logical attributes

| Attributes | Importance | Example |
| --- | --- | --- |
| TRAU Board Type | [Mandatory] | - To be deduced from both Physical and Logical dumps as per below: - From Logical Dumps, extract the value of the attribute “distName” stated on the same line as <managedObject class=“TCSM” and take the value after last “/”, e.g. “TCSM-192”, multiple managed objects might be found for each BSC. - Find in matching physical dump the Unit List supporting Functional Role=”TCSM” and “functionalUnitId” equal to the value extracted from logical dump, multiple boards might be found. - Only boards with board type starting with “TR” to be considered, e.g. “TR3T”. - TRAU Board Type= Board Type of above boards. - Please refer to attached photo. |
| TRAU Board ID | [Mandatory] | - Is the concatenation of below attributes separated with “_”: - “Cartridge ID”. - “Cartridge shelf Index” - “Board Slot Index” - E.g. “NE-FlexiBSC-373889_3_2_6_1” |

| Attributes | Importance | Example |
| --- | --- | --- |
| TRAU ID | [Mandatory] | - Find “Cartridge shelf” comprising the TRAU Boards extracted above in physical dumps. - TRAU ID Is the concatenation of below attributes separated with “_”: - “Cartridge ID”. - “Cartridge shelf Index” - E.g. “NE-FlexiBSC-373889_3_2_6” |
| TRAU Name | [Optional] | - To be deduced from Physical dumps. - =”Cartridge Shelf Model”. - Please refer to Physical connector. |
| TRAU Type | [Mandatory] | - If BSC type is “FlexiBSC” then TRAU type is ”TCSM3i” |

# Additional attribute

- Below listed additional info to be added on BSC

- Attribute to be incremented from 1 for each Category

- The values in note field should be separated with “;”

- Each class “NSVL” represent one ADJNODE, from distName we deduce the BSC-ID example: <managedObject class="NSVL" version="S16_3" distName="PLMN-PLMN/BSC-ID/NSVL-XX

- Under the above class, we deduce the Remote_IP, where Remote_IP= ipAddressValue

- The Local_IP to be deduced from the separate dumps sheet named ZQRI

- Where each section start with: --------- DATE and Time PLMN-PLMN/BSC-ID BSC-Name ---------

- We get the Local_IP under section:

  - BCSU-X    -PCU2E-Y where X and Y are integer

  - BCSU-X    -PCU2D-X where X and Y are integer

  - PCUM-X    where X is an integer

    - For this section we get only the IP address where Vlan exist

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE Every class="NSVL" represents one connected ADJNODE | = distName of class=" NSVL"/1 and to be increment for each ADJNODE and Association e.g. distName="PLMN-PLMN/BSC-316953/NSVL-19113/1 | - CONNECTION_TYPE=GB - NODE_ID= IP_DERIVED |
| ASSOCIATION | = distName of class=" NSVL"/1 and to be increment for each ADJNODE and Association e.g. distName="PLMN-PLMN/BSC-316953/NSVL-19113/1 | - LOCAL_IP: From the ZQRI as described above - REMOTE_IP= the value of “ipAddressValue” under class=" NSVL" |

### Abis over TDM:

- Each class=”BCF” represents one Abis over TDM interface

- For each “BCF” class

  - go to the “TRX” classes that contain in “distName” the same value of “distName” of “BCF” class. Multiple matches will be found. All matches to be considered

  - for each matched “TRX” class

    - take the value of “channel0Pcm”

  - duplicated values are to be removed

  - These values are to be used in ASSOCIATION records

  - The number of ASSOCIATION records for each ADJNODE will be based on the found values. E.g. If two unique values were found  two ASSOCIATION should be created for this ADJNODE

| Category | Attribute | Value | Note |
| --- | --- | --- | --- |
| ASSOCIATION //The number of records for each ASSOCIATION depends on the unique values found under “channel0Pcm”// | = the value of “distName” attribute of “BCF” class concatenated with “_ASSOCIATION_” And with an integer increased for each ASSOCIATION under the same BCF e.g. PLMN-PLMN/BSC-367255/BCF-241_ASSOCIATION_2 | = the value of “distName” attribute of “BCF” class concatenated with “_” And with an integer increased for each ASSOCIATION under the same BCF | - LOCAL_PORT= has the format of port key C/S/S/I/P - Index on slot=0 - Port index= - take the unique value of “channel0Pcm” - match it with a range in the static file “NSN_2G_TDM_Matching”. Where every range starts with the value in column A and ends with the value of column B - take the value of column D of the matched record as port index - counting ETS2 boards: - give each board found in physical under the same BSC of the BCF with type “ETS2” a number - start from 0 and increment by one for each board - C/S/S= - take the unique value of all “channel0Pcm” - match it with a range in the static file “NSN_2G_TDM_Matching”. Where every range starts with the value in column A and ends with the value of column B - take the value of column C of the matched record. This value will be the value of group number - Sort the groups numbers for all ports for each BSC in ascending order - For each group number assign one ETS2. And assign the boards respectively. - If the BSC model is “flexiBSC” don’t use ETS2 with even numbers - E.g. groups numbers are: 0, 1, 5 assign the groups as follows: - Group 0: ETS number 0 - Group 1: ETS number 2 - Group 5: ETS number 4 - take C/S/S of an ETS2 board with the same number from previous step - LOCAL_TRIBUTARY= the unique value of “channel0Pcm” |
| ADJNODE //Every class=”BCF” represents one or more connected ADJNODE// Create one ADJNODE For each ASSOCIATION record | = the value of “distName” attribute of “BCF” class concatenated with “_” And with an integer increased for each ASSOCIATION under the same BCF | = the value of “distName” attribute of “BCF” class concatenated with “_” And with an integer increased for each ASSOCIATION under the same BCF | - CONNECTION_TYPE=Abis - INTERFACE_TECHNOLOGY=TDM - NODE_ID=OBJECT_ID - NODE_OBJECT_ID= the value of “distName” attribute of “BCF” class |

### Abis over IP

- Abis over IP to be added to Additional Info under BSC

- The parent BSC on which the additional info should be added, could be got from the “distName” attribute

| Category | Attribute | Value | Note |
| --- | --- | --- | --- |
| ADJNODE Every " PABTRS" class represents one ADJNODE | = the value of “distName” attribute of “PABTRS” class concatenated with “_ADJNODE” | = the value of “distName” attribute of “PABTRS” class | - CONNECTION_TYPE=Abis - INTERFACE_TECHNOLOGY=IP - NODE_ID=IP_DERIVED - NODE_OBJECT_ID= the value of “distName” attribute of “BCF” class - CONNECTION_NAME= The value of “distName” attribute of “PABTRS” class |
| ASSOCIATION 1st Association | = the value of “distName” attribute of “PABTRS” class concatenated with “_Association” | = the value of “distName” attribute of “PABTRS” class | - LOCAL_IP= the value of attribute "mPlaneRemoteIpAddress" under class “PABTRS” - REMOTE_IP= the value of attribute "mPlaneLocalIpAddress" under class “PABTRS” Note: No Space exists between LOCAL_IP and REMOTE_IP |

### BSC OPC

- This additional info is to be added under BSCs

- Every “BSC” class represents one OPC

- The parent BSC could be derived from distName attribute of “BSC” class

| Category | Attribute | Value | Note |
| --- | --- | --- | --- |
| OPC | “OPC_value” e.g. OPC_ 3882 | = the value of “signallingPointCodeNA1” attribute of “BSC” class | - OPC= the value of “signallingPointCodeNA1” attribute of “BSC” class e.g. OPC=3882 |

### Abis over IP for SRAN

- This section related to MRBTS will be added under BSC

- Remote IP will be parsed from attribute IP address of section 5.6

- From class="GNBCF", from the distName "PLMN-PLMN/MRBTS-xxxx/GNBTS-Y/GNBCF-Z" where xxxx is the SRAN ID & Y and Z are an integer

- From the same class, each attribute "mPlaneRemoteIpAddressOmuSig" represent one Local IP.

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | SRAN-ID/ /GNBTS-Y/GNBCF-Z | - CONNECTION_TYPE=Abis - INTERFACE_TECHNOLOGY=IP - NODE_ID=IP_DERIVED - NODE_OBJECT_ID= the value of “distName” attribute of “MRBTS” class - CONNECTION_NAME= The value of “distName” attribute of “GNBCF” class |
| ASSOCIATION // 1st association // | SRAN-ID/ /GNBTS-Y/GNBCF-Z | - LOCAL_IP= the value of "mPlaneRemoteIpAddressOmuSig" in class=“GNBCF” - REMOTE_IP= the value of "localIpAddr" in class=“IPADDRESSV4” |