NSN 4G Parsing

FRS (Functional Requirement Specifications)

Auto-Import

|  | Indrasan Yadav |
| --- | --- |
|  | 04-06-2025 |

Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft. | Indrasan Yadav | 29-05-2025 |
| 0.2 | Update the rule under section 4.4 regarding to parse channel bandwidth from dump directly instead of static sheet “Supported channel bandwidth” & add a line to explain from where to be parse frequency band (Highlighted with green color). | Indrasan Yadav | 20-01-2026 |

# PREAMBLE

- This document specifies how to parse the logical inventory attributes for the eNodeB and its cells.

- It covers the parameters defined for eNodeB RAN logical attributes.

- The provided dump files are of “.xml”.

- This is motivated by Airtel Projects and can be used for other operators having same file structure.

- Another source type of dump files could be provided later for Antenna attributes.

# Connector name

NSN_RAN_Ooredoo_Tunisia

# Connector Information

- The dump file is an “.xml” file composed of set of Managed Objects.

- Each Managed Object starts with “<managedObject “and ends with “</managedObject>”.

- Inside each managed object, there is a Tag name, followed by “attributes” and finally by the different set of parameters with their values.

# ENODEB logical parameters

The eNodeB logical inventory attributes are classified accordingly in the below categories:

  - Global Parameters

  - Networking Parameters

  - Radio Parameters

  - Cell Parameters

- Note: The internal mapping between the Physical and Logical parameters will be based on the Node ID.

- Note: If the Node ID parsed from the logical dumps wasn’t matched with the Node ID of the physical connector , hence remove the digit “0” from the Node ID parsed from the physical connector, where the digit “0” is found directly after the 1st digit as follow :  PLMN-PLMN/MRBTS-X0XXXX  PLMN-PLMN/MRBTS-XXXXX

  - E.g.: PLMN-PLMN/MRBTS-605531  PLMN-PLMN/MRBTS-65531

### Global Parameters

The below global attributes defined at eNodeB level are as follows:

Class=“MRBTS”

| Attributes | Importance | Example |
| --- | --- | --- |
| eNodeB ID | [Mandatory] | - = “distName” attribute under the <managedObject Tag with having the “class=“LNBTS”. - E.g.: <managedObject class="LNBTS" version="xL24R2_2404_110" distName="PLMN-PLMN/MRBTS-15142/LNBTS-15142" id="104777"> eNodeB ID = 15142 - If class=”LNBTS” is not found, then take it from class=”MRBTS” - E.g: <managedObject class="MRBTS" version="SBTS24R2_2325_100" distName="PLMN-PLMN/MRBTS-15142" id="103327"> eNodeB ID =15142 |
| eNodeB Name | [Mandatory] | - From class=”LNBTS”, take attribute “name” (<p name="name">TUNL142</p>), if name doesn’t exist then take “enbName” (<p name="enbName">TUNL142</p>). LNBTS would exist under corresponding MRBTS or SBTS class. |
| Site Id | [Mandatory] | - Please check in physical FRS. |
| Site Name | [Mandatory] | - Site Id |
| Type | [Mandatory] | - = eNodeB |
| OAM IP | [Optional] | - =“oamIpAddr” attribute associated with the <managedObject Tag having the “class” attribute = “IPNO” - E.g.: <p name="oamIpAddr">10.96.151.97</p>… OAM IP = 10.96.151.97 |
| OAM Subnet Mask | [Optional] | - OAM IP is consisting of 4 numbers separated with”.”(e.g. 10.96.151.97). - OAM Subnet Mask to be deduced based on the first number of the OAM IP as per below: - <=127255.0.0.0 - <=191 or >=128 255.255.0.0 - >=192 255.255.255.0 - E.g.: OAM IP = 10.96.151.97 OAM Subnet Mask = 255.0.0.0 |

### Networking Parameters

The below networking attributes defined at eNodeB level are as follows:

| Attributes | Importance | Example |
| --- | --- | --- |
| MCC | [Mandatory] | - = “mcc” attribute under the <p Tag associated with the <managedObject Tag having the “class” attribute = “LNBTS” - E.g.: <p name="mcc">605</p> - The “name” attribute mentioned under same class=”LNBTS” refers the node name. |
| MNC | [Mandatory] | - =”mnc” attribute under the <p Tag associated with the <managedObject Tag having the “class” attribute = “LNBTS” - E.g.: <p name="mnc">03</p> - The “name” attribute mentioned under same class=”LNBTS” refers the node name. |
| TAC list | [Optional] | - =NEP tool calculated, TAC list of children cells. |
| Parent MMEs | [Optional] | - More than one Parent MME could exist per each node. - The Parent MMEs Name is extracted according to the following steps: - = “value” concatenated with the “name” attribute having a “value” = “mmeName” under the <p present underneath the <managedObject Tag having the “class” attribute = “LNMME” - Therefore check if more than one “class” attribute=”LNMME” having different “mmeName” value. if yes, all the associated “mmeName” values are to be parsed. - E.g.: - Parent MME is deduced by matching Parent MME Name deduced from dumps with Column A in MME sheet in Parent_MME_SGW.xlsx matching file, if found then Parent MME is the value in column B on the same line, Else consider same value deduced from Dumps. - Matching file to be provided later. |
| Parent SGWs | [Optional] | - More than one Parent SGWs could exist per each node. - The Parent SGWs Name is extracted according to the following steps: - = “value” concatenated with the “name” attribute having a “value” = “sgwIpAddress” under the <p Tag associated with the <item> Tag located inside the <list> Tag present underneath the <managedObject Tag having the “class” attribute = “GTPU” - Therefore check if more than one “class” attribute=” GTPU” having different “sgwIpAddress” value. If yes all the associated “sgwIpAddress” values are to be parsed. - Under Class = GTPU”listitempname="sgwIpAddress” - E.g.: <p name="sgwIpAddress">10.41.4.5</p> - Parent SGW is deduced by matching Parent SGW Name deduced from dumps with Column A in SGW sheet in Parent_MME_SGW.xlsx matching file, if found then Parent SGW is the value in column B on the same line, Else consider same value deduced from Dumps. - Matching file to be provided later. |
| Served Cells | [Optional] | - NEP tool calculated. |
| Connected eNodeBs | [Optional] | - For each node there exists a set of connected eNodeBs. Each eNodeB is referred to by <managedObject class="LNADJ", whose name was already parsed by extracting the “name” attribute (name="name">LTE_Broadway_GSDN01_KZN</p>) - The “connected eNodeBs” attribute refers to the IDs of each of the connected eNodeBs, and could be obtained following the below steps: - Under each <managedObject class=“LNADJ”, extract the name=“adjEnbId” attribute. - E.g.: <p name="adjEnbId">631071</p> - Concatenate each parsed value with the key word “PLMN-PLMN/MRBTS-“to get the full node ID. - E.g.: PLMN-PLMN/MRBTS-631071 - If there is no matching concatenate it with the Key word “PLMN-PLMN/SBTS-“ - E.g.: PLMN-PLMN/SBTS-631071 - The Name of each “connected eNodeBs” can be then deduced through the node ID. Note: If same “adjEnbId” was detected, ignore the repeated value. |

### Radio Parameters

The following table summarizes the radio attributes, which have default value at catalog level but can be imported and editable at instance level.

| Attribute | Importance | Example |
| --- | --- | --- |
| TDD enabled | [Optional] | - each eNodeB has multiple cell - TDD enabled: - if at least one cell have these attributes tddFrameConf and tddSpecSubfConf under class=“LNCEL” - example: <managedObject class="LNCEL" version="TL16A" distName="PLMN-PLMN/MRBTS-XXX/LNBTS-XXX/LNCEL-Y <p name="tddFrameConf">2</p> <p name="tddSpecSubfConf">7</p> Then TDD is enabled, otherwise it will be FDD. And FDD details will be find in class=”LNCEL_FDD”. |
| Operational Frequency bands for FDD | [Optional] | - Only if the value of “redirRat” is equal to “2” , follow the below steps: - Get each “earfcnDL” found under each <managedObject class=“LNCEL”. - Note: If same “earfcnDl” was found, ignore the repeated values and take only one value. - Check to which “Value Range” the obtained value belongs to, then take the value under “Band” column. - The Operation Frequancy bands for FDD will be then “FDD_” concatenated with the value under the “Band”. - E.g.: "earfcnDL">1800</p> Value Range is 1200-1949 , therefore the Band is equal to 3, “hence the Operational Frequency bands for FDD” will be equal to FDD_3 Band; Value Range. 1; 0 – 599. 2; 600 – 1199. 3; 1200 – 1949. 4; 1950 – 2399. 5; 2400 – 2649. 6; 2650 – 2749. 7; 2750 – 3449. 8; 3450 – 3799. 9; 3800 – 4149. 10; 4150 – 4749. 11; 4750 – 4949. 12; 5010 – 5179. 13; 5180 – 5279. 14; 5280 – 5379. 17; 5730 – 5849. 18; 5850 – 5999. 19; 6000 – 6149. 20; 6150 – 6449. 21; 6450 – 6599. 22; 6600 – 7399. 23; 7500 – 7699. 24; 7700 – 8039. 25; 8040 – 8689. 26; 8690 – 9039. 27; 9040 – 9209. 28; 9210 – 9659. 29; 9660 – 9769. 30; 9770 – 9869. 31; 9870 – 9919. - Note: In case of different values of “earfcnDL” were found list them all. - FDD_3 , FDD_5 , FDD_20 |
| Operational Frequency bands for TDD | [Optional] | - Only if “TDD supported” is enabled , follow the below steps : - Get each “earfcn” found under each <managedObject class=“LNCEL”. - Note: If same “earfcn” was found, ignore the repeated values and take only one value. - Check to which “Value Range” the obtained value belongs to, then take the value under “Band” column. - The Operation Frequency bands for TDD will be then a concatenation of “TDD_” with the matched value under “Band” Column. - E.g.: "earfcn ">1800</p> Value Range is 1200-1949, therefore the Band is equal to 3, “hence the Operational Frequency bands for FDD” will be equal to “FDD_3”. Band; Value Range. 33. 36000-36199. 34. 36200-36349. 35. 36350-36949. 36. 36950-37549. 37. 37550-37749. 38. 37750-38249. 39. 38250-38649. 40. 38650-39649. 41. 39650-41589. 42. 41590-43589. 43. 43590-45589. 44. 45590-46589. - Note: In case of different values of “earfcnDL” were found list them all. - TDD_41 , TDD_43 , TDD_44 |
| Supported Bandwidths | [Optional] | - Get the deduced Band value obtained from the Band column present in the “Operational Frequency bands for TDD” table or “Operational Frequency bands for FDD” table, and match it in the below table. Once matched, get the value of corresponding supported bandwidths: Band; Supported Bandwidths. 1; 5, 10, 15, 20. 2; 1.4, 3, 5, 10, 15, 20. 3; 1.4, 3, 5, 10, 15, 20. 4; 1.4, 3, 5, 10, 15, 20. 5; 1.4, 3, 5, 10. 7; 5, 10, 15, 20. 8; 1.4, 3, 5, 10. 9; 5, 10, 15, 20. 10; 5, 10, 15, 20. 11; 5, 10. 12; 1.4, 3, 5, 10. 13; 5, 10. 14; 5, 10. 17; 5, 10. 18; 5, 10, 15. 19; 5, 10, 15. 20; 5, 10, 15, 20. 21; 5, 10, 15. 22; 5, 10, 15, 20. 23; 1.4, 3, 5, 10. 24; 5, 10. 25; 1.4, 3, 5, 10, 15, 20. 26; 1.4, 3, 5, 10, 15. 27; 1.4, 3, 5, 10, 15. 28; 5, 10, 15, 20. 28; 5, 10. 30; 5, 10. 33; 5, 10, 15, 20. 34; 5, 10, 15. 35; 1.4, 3, 5, 10, 15, 20. 36; 1.4, 3, 5, 10, 15, 20. 37; 5, 10, 15, 20. 38; 5, 10, 15, 20. 39; 5, 10, 15, 20. 40; 5, 10, 15, 20. 41; 5, 10, 15, 20. 42; 5, 10, 15, 20. 43; 5, 10, 15, 20. 44; 5, 10, 15, 20. |
| Uplink 64-QAM enabled | [Optional] | (catalog Level) |
| ICIC enabled | [Optional] | (catalog Level) |

### Cell Logical Parameters

| Attributes | Importance | Comment |
| --- | --- | --- |
| Node ID | Mandatory | - = “distName” attribute under the <managedObject Tag with having the “class” attribute = “MRBTS” OR “class” attribute =“SBTS” - E.g.: <managedObject class="MRBTS" version="LN3.0" distName="PLMN-PLMN/MRBTS-65531" id="101161000001114349"> Node ID = PLMN-PLMN/MRBTS-65531 - E.g.: <managedObject class="MRBTS" version="LN3.0" distName="PLMN-PLMN/SBTS-65531" id="101161000001114349"> - Node ID = PLMN-PLMN/SBTS-65531 |
| Parent Site ID | Mandatory | - (NEP tool calculated) |
| Cell ID | Mandatory | - It is equal to the concentration of the Node ID with the local cell ID from the distName where class=“LNCEL” - Example: Separate LNBTS and LNCEL with “-“ Example :LNBTS= 900719, cell ID= 1 cell ID =900719-1 In case of the Node Id contain MRBTS or SBTS remove it from the cell ID Special Case: For Role4G node LNBTS-512011 (Parent Node MRBTS-512011) Whose 4G Cell ID & Name are duplicate with Huawei 4G Cell ID & Name for node BARF011. So, in this case applying below rule for these 6 cells only. To be add “_NSN” in these 6 cell id & name. |
| Cell Name | Optional | - Get the different Cell Name values for each node as follows: Under class="LNCEL" - E.g.: <p name="name">LTE_Galleria_Int_GSDN01_KZN_1</p> Special Case: If duplicate cells will be found with Huawei RAN. Append “_1” for the first cell’s ID and Name and increment +1 subsequently for the next duplicates. |
| External Code | Mandatory | - Is the concatenation of “EnodeBID”, “_” and “Cell ID”. - “eNodeBID” = “distName” attribute under the <managedObject Tag with having the “class” attribute = “MRBTS” OR “SBTS” - = CellId equals class="LNCEL" under "distName" attribute and after the keyword "LNCEL-" - E.g. PLMN-PLMN/MRBTS-65531_14541. |
| Cell Technology | Optional | - = NEP tool calculated, Else =”LTE”. |
| MCC | [Mandatory] | - = “mcc” attribute under the <p Tag associated with the <managedObject Tag having the “class” attribute = “LNCEL” - E.g.: name="mcc">655</p> - The “name” attribute mentioned under same class=”LNCEL” refers the node name. |
| MNC | [Mandatory] | - =”mnc” attribute under the <p Tag associated with the <managedObject Tag having the “class” attribute = “LNCEL” - E.g.: name="mnc">01</p> - The “name” attribute mentioned under same class=”LNCEL” refers the node name. |
| Neighbor Cells 2G | Optional | - Under each class="LNCEL" there exist several class="LNRELG" - Get the different “Neighbor Cells 2G” ID values as follows : It is a concatenation of :”LAC”,”_” and ”Cell_ID” as follow: - The “Cell_ID” and “Lac” are exctracted under each class="LNRELG"pname=“ci” and name="lac" - E.g.: <p name="ci"> 10563 </p> - <p name="lac">307</p> E.g.:307_10563 - The obtained values refer to the Cell_ID obtained under same class="LNCEL" |
| Neighbor Cells 3G | Optional | - Under each class="LNCEL" there exist several class="LNRELW" - Get the different "Neighbor Cells 3G" ID values as follows: - It is a concatenation of “LAC” , “_” and “Cell_ID” as follow: Under each class="LNRELW"pname=“uTargetCid” and name="uTargetLac - E.g.: <p <p name="uTargetCid "> 5531 </p> - <p name="uTargetLac">3021</p> E.g.:3021_5531 - If the class=“uTargetLac” is not found - Neighbor Cell Rnc ID= class=“uTargetRncId" - Neighbor Cell ID= class="uTargetCid" - Match the neighbor Cell Id with Cell ID in the same Rnc ID - The obtained values refer to the Cell_ID obtained under same class=”LNCEL” |
| Neighbor Cells 4G | Optional | - For each Cell there exists more than one Neighbouring Cell_ID. - Under each class="LNCEL" you will find several class="LNREL" this will lead us to know the “Neighbor Cells 4G” as follow : - Get under each class="LNREL" whenever exists, the name="ecgiAdjEnbId" value. - For each of these obtained values match it with name=" ecgiAdjEnbId" under each class="LNADJL" - Once matched get the value of “phyCellId” and “tac” under the same class="LNADJL" - Then search for the same distName="PLMN-PLMN/MRBTS-x” where x represents the “ecgiAdjEnbId” , if there no match search for distName="PLMN-PLMN/SBTS-x” - Once matched lookup for the same "phyCellId" and “tac” value under class="LNCEL" - The Cell that possesses a value under LNCELlistpname=“phyCellId” and “tac” which matches the original retrieved value is the Cell whose ID to be taken from this same class=”LNCEL” line using the rule as aforementioned in the Cell ID section. - This process is repeated for every Neighbouring cell corresponding to a certain eNodeB node. |

Neighbor Cells 4G:

- Specific attributes for 4G cells

| Attributes | Example | Comments |
| --- | --- | --- |
| Cell Duplexing Mode | Optional | - under class=“LNCEL” - If these attributes exist “tddFrameConf” and “tddSpecSubfConf”, then it is TDD - If not exist then check above attributes in class class="LNCEL_TDD”, which would be under the corresponding LNCEL class. E.g class="LNCEL_TDD" version="TL17A_1701_07_1701_06" distName="PLMN-PLMN/MRBTS-801068/LNBTS-801068/LNCEL-2/LNCEL_TDD-0 If attributes exist here, then its TDD, otherwise FDD. |
| PCID | Optional | - Under Class=”LNCEL” name="phyCellId” - "E.g.: <PhyCellId>185</PhyCellId> |
| TAC list | Optional | - Under : Class=”LNCEL” name="tac" - E.g.: name="tac">21000</p> - This value is the same for all the cells under a specific node. |
| Downlink Bandwidths | Optional | - For FDD: Based on the value of “dlChBw”, the “Downlink Bandwidths” will be deduced. - It is found Under: Class=”LNCEL” name="dlChBw" E.g.: name="dlChBw">10 MHz</p> If not found under class=”LNCEL”, then check under class="LNCEL_FDD" which would be under the corresponding LNCEL e.g class="LNCEL_FDD" version="FL17A_1701_07_1701_06" distName="PLMN-PLMN/MRBTS-254/LNBTS-254/LNCEL-1/LNCEL_FDD-0 - For TDD Based on the value of “ChBw”, the “Downlink Bandwidths” will be deduced. - It is found Under: class=”LNCEL” name="ChBw" E.g.: name="ChBw">20 MHz</p> if not found under class=”LNCEL” then check under class="LNCEL_TDD" which would be under the corresponding LNCEL |
| Uplink Bandwidths | Optional | - For FDD: Based on the value of “ulChBw”, the “Uplink Bandwidths” will be deduced. - It is found Under: class=”LNCEL” name="ulChBw" E.g.: name="ulChBw">10 MHz</p> If not found under class=”LNCEL”, then check under class="LNCEL_FDD" which would be under the corresponding LNCEL For TDD: Uplink Bandwidth= Downlink Bandwidth |
| Downlink Earfcn | Optional | - For FDD: Under class=”LNCEL”name="earfcnDL" E.g.: name="earfcnDL">1800</p> If not found under class=”LNCEL”, then check under class="LNCEL_FDD" which would be under the corresponding LNCEL e.g class="LNCEL_FDD" version="FL17A_1701_07_1701_06" distName="PLMN-PLMN/MRBTS-254/LNBTS-254/LNCEL-1/LNCEL_FDD-0 - For TDD Under: class=”LNCEL”name=" earfcn", if not found under class=”LNCEL” then check under class="LNCEL_TDD" which would be under the corresponding LNCEL. For TDD cell Downlink Earfcn= Uplink Earfcn |
| Uplink Earfcn | Optional | - Under: class=”LNCEL”name=" earfcnUL" E.g.: name="earfcnUL">19800</p> If not found under class=”LNCEL”, then check under class="LNCEL_FDD" which would be under the corresponding LNCEL e.g. class="LNCEL_FDD" version="FL17A_1701_07_1701_06" distName="PLMN-PLMN/MRBTS-254/LNBTS-254/LNCEL-1/LNCEL_FDD-0 - For TDD Under: class=”LNCEL”name=" earfcn", if not found under class=”LNCEL” then check under class="LNCEL_TDD" which would be under the corresponding LNCEL. For TDD cell Downlink Earfcn= Uplink Earfcn |
| Frequency Band | Optional | - Based on duplexing mode & downlink EARFCN ranges to be parse from static sheet “FrequencyBand”. |
| MIMO configuration | Optional | - For FDD: Following the value of “dlMimoMode”, the “MIMO configuration” will be deduced. - “dlMimoMode” is found under Class=”LNCEL” name="dlMimoMode" E.g.: name="dlMimoMode">40</p> If not found under class=”LNCEL”, then check under class="LNCEL_FDD" which would be under the corresponding LNCEL e.g class="LNCEL_FDD" version="FL17A_1701_07_1701_06" distName="PLMN-PLMN/MRBTS-254/LNBTS-254/LNCEL-1/LNCEL_FDD-0 - For TDD: Following the value of “dlMimoMode”, the “MIMO configuration” will be deduced. - “dlMimoMode” is found under class=”LNCEL” name="dlMimoMode" E.g.: name="dlMimoMode">40</p> if not found under class=”LNCEL” then check under class="LNCEL_TDD" which would be under the corresponding LNCEL Below mapping to be used to get mimo configuration from dlMimoMode - 0 : SingleTX; 10 : TXDiv; 11 : 4-way TXDiv; 30 : Dynamic Open Loop MIMO; 40 : Closed Loop Mimo; 41 : Closed Loop MIMO (4x2); 43 : Closed Loop MIMO (4x4) 50: MASSIVEMIMO_SINGLE_BEAM_64x64 60: MASSIVEMIMO_DUAL_BEAM_64x64 |
| Cell Status | Optional | - Under each Cell of a enodeB (Class= “LNCEL”) - Take the value from attribute <p name="administrativeState" - If value is equal to 1, then the Cell will be in ACTIVE state. - If value is equal to 0, then the Cell will be in HALT state. |

### Cell Parameters for “MRBTS Version SBTS” for FDD CELLS

| Downlink Earfcn | Optional | - Under:class="LNCEL_FDD"name="earfcnDL" - E.g.: name="earfcnDL">1374</p> So, first we need to search for class=LNCEL_FDD and in same line we can get the value of LNCEL-1. For Eg::- class="LNCEL_FDD" version="xL18_1708_008" distName="PLMN-PLMN/MRBTS-900505/LNBTS-900505/LNCEL-1 |
| --- | --- | --- |
| Uplink Earfcn | Optional | - Under : Class="LNCEL_FDD"name=" earfcnUL" - E.g.: name="earfcnUL">19374</p> |

### Cell Parameters for “MRBTS Version SBTS” for TDD CELLS

| Downlink Earfcn & Uplink Earfcn for TDD Cell | Optional | - Under:class="LNCEL_TDD"name="earfcn" - E.g.: name="earfcn">38800</p> So, first we need to search for class=LNCEL_TDD and in same line we can get the value of LNCEL-1. For Eg::- class="LNCEL_TDD" version="TL17A_1701_07_1701_06" distName="PLMN-PLMN/MRBTS-803949/LNBTS-803949/LNCEL-1 ### Note: - In class="LNCEL_TDD" the value of Downlink Earfcn & Uplink Earfcn remains the same. |
| --- | --- | --- |

# 5. IP Parsing eNodeB

## Physical port

- To be parsed from class=“IVIF”, where the distName="PLMN-PLMN/MRBTS-xxx/LNBTS-xxx where xxx represent the eNodeB ID

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Physical port |
| Node ID | [Mandatory] | - = from the distName |
| Physical Port C/Sh/S/I/P | [Mandatory] | - To be provided later. |
| IP address | [Optional] | - = attribute "localIpAddr" |
| Subnet Mask | [Optional] | - = attribute "netmask" |
| VLAN | [Optional] | - = attribute "vlanId" |
| Gateway | [Optional] | - From class=“IPRT” - Deduce the range of IP from attribute "destIpAddr" and attribute "netmask" - Match the above IP address with the deduce range of IP - When matched take Gateway = attribute "gateway" |

If class=”IVIF” not found then available, then take class=”FSTSCH”

- In the same class take the value under the attribute “ipIfDN” where the “userLabel” equal to “LTEU”, “LTEC” and “MPLANE”

- Match this value of <list name="ipIfDN"> with the distname where the class=”IPADDRESSV4” is available and parse "localIpAddr" as IP address

- For “Vlan” check value of <list name="ipIfDN"> which we got under class=”FSTSCH”  and match with  class= “IPIF” distName available and get attribute of name="interfaceDN"

- Match attribute name="interfaceDN" get from class=”IPIF” under class=”VLANIF” and take vlanId as VLAN.

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Physical port |
| Node ID | [Mandatory] | - = from the distName |
| Physical Port C/Sh/S/I/P | [Mandatory] | - To be provided later. |
| IP address | [Optional] | - = attribute "localIpAddr" under class=”IPADDRESSV4” parse as IP address |
| Subnet Mask | [Optional] | - = attribute "localIpPrefixLength" under class=”IPADDRESS4 |
| VLAN | [Optional] | - = attribute "vlanId" as explained above |
| Gateway | [Optional] | - From class=“IPRT” - Deduce the range of IP from attribute "destIpAddr" and attribute "netmask" - Match the above IP address with the deduce range of IP - When matched take Gateway = attribute "gateway" |

# IP Parsing SRAN

## Physical port “SBTS”

- To be parsed from class=“IPIF”, where the distName="PLMN-PLMN/SBTS-xxx/TNL-y/IPNO-z/IPIF-x where xxx represent the node ID and each x represent an interface

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Physical port |
| Node ID | [Mandatory] | - = from the distName |
| Physical Port C/Sh/S/I/P | [Mandatory] | - To be provided later. |
| IP address | [Optional] | - = attribute "localIpAddr" |
| Subnet Mask | [Optional] | - = attribute "localIpPrefixLength" |
| VLAN | [Optional] | - = attribute "vlanId" under class=“VLANIF” - Matching is described in below snapshot |
| Gateway | [Optional] | - From class=“RTPOL” - Match deduced IP address above with attribute "srcIpAddress" - Take the attribute "routingTablePtr" for the deduced "srcIpAddress" - From class=“IPRT” Put the value of "routingTablePtr" in distName="PLMN-PLMN/SBTS-5672/TNL-1/IPNO-1/Value_OF_routingTablePtr” - Gateway= attribute "gateway" |

## Physical Port “MRBTS” version “SBTS

- If you are not able to get IP Address from class=“IVIF”, then go to  class=“IPADDRESSV4”  where the distName="PLMN-PLMN/MRBTS-xxx/LNBTS-xxx where xxx represent the eNodeB ID.

| Attributes | Importance | Parsing |
| --- | --- | --- |
| Interface Name | [Mandatory] | - =Physical port |
| Description | [Optional] | - Match the section start by mrbts till IPIF in the distname example “MRBTS-905708/TNLSVC-1/TNL-1/IPNO-1/IPIF-7” with value under attribute “ipIfDN” under the class “FSTSCH” where the distname start with “PLMN-PLMN/MRBTS-ID - Example in the below screen shot |
| Node ID | [Mandatory] | - = from the distName |
| Physical Port C/Sh/S/I/P | [Mandatory] | - To be provided later. |
| IP address | [Optional] | - = attribute "localIpAddr" |
| Subnet Mask | [Optional] | - = attribute "localIpPrefixLength" |
| VLAN | [Optional] | - In the line class=“IPADDRESSV4” i.e. class="IPADDRESSV4" version="TNL17A_1708_008" distName="PLMN-PLMN/MRBTS-900968/TNLSVC-1/TNL-1/IPNO-1/IPIF-1/IPADDRESSV4-1" - Take the value of IPIF-1 the value will be 1 - Now go to class="IPIF", which has IPIF-1 in it. - For example:- class="IPIF" version="TNL17A_1708_008" distName="PLMN-PLMN/MRBTS-900968/TNLSVC-1/TNL-1/IPNO-1/IPIF-1" - Now under class=“IPIF”, you will get attribute interfaced take the value of VLANIF i.e. name="interfaceDN">MRBTS-900968/TNLSVC-1/TNL-1/ETHSVC-1/ETHIF-1/VLANIF-1, the value will be 1 - Now go to class="VLANIF", which has VLANIF-1 in it. - Under the attribute name="vlanId", you will get the VLAN. |
| Gateway | [Optional] | - From class=“IPRT” - Deduce the range of IP from attribute "destIpAddr" and attribute "netmask" - Match the above IP address with the deduce range of IP - When matched take Gateway = attribute "gateway" |

# Additional info for eNodeB

- The attribute element to be increment by 1 for each new records

| Category | Value | Note |
| --- | --- | --- |
| Software version | eNodeB ID | In the same line where class=“MRBTS” take the version equal to the value after the version |

## X2 Interface

- This section related to eNodeB MRBTS

- From class=“IPNO”, from the distName="PLMN-PLMN/MRBTS-800024 where after “MRBTS-” parse the eNodeB ID.

- In the same class take the Local_IP from the attribute "cPlaneIpAddress"

- From class=“LNADJ”, where the distName="PLMN-PLMN/MRBTS-eNodeB ID/LNBTS-eNodeB ID/LNADJ-X” (x is an integer), each LANDJ-X represent one additional info on the same Node and the attribute to be increment by 1 for each LANDJ-X

- From the same class take the Remote_IP from the attribute "cPlaneIpAddr"

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | eNodeB ID/LNADJ-X | - CONNECTION_TYPE=X2 - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | eNodeB ID/LNADJ-X | - LOCAL_IP= the value of "cPlaneIpAddr" in class=“IPNO” - REMOTE_IP= the value of "cPlaneIpAddr" in class=“LNADJ” |

## 2 S1-MME

- This section related to MRBTS

- From class=“IPNO”, from the distName="PLMN-PLMN/MRBTS-800024 where after “MRBTS-” parse the eNodeB ID.

- In the same class take the Local_IP from the attribute "mPlaneIpAddress"

- From class=“LNMME”, where the distName="PLMN-PLMN/MRBTS-xxx/LNBTS-xxx/LNMME-Y where xxx is the eNodeB ID, each Y represent an additional info

- From the same class take the Remote_IP from the attribute "ipAddrPrim"

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | eNodeB-ID /LNMME-Y | - CONNECTION_TYPE=S1-MME - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | eNodeB-ID /LNMME-Y | - LOCAL_IP= the value of "mPlaneIpAddress" in class=“IPNO” - REMOTE_IP= the value of "ipAddrPrim" in class=“ LNMME” |

Case1: If class=”IPNO” doesn’t exist or does not have "mPlaneIpAddress" then follow given below rule for MRBTS

- This section related to MRBTS both version

- From class=“FSTSCH”, from the distName="PLMN-PLMN/MRBTS-ID where after “MRBTS-” parse the SBTS/MRBTS ID.

- In the same class take the value under the attribute “ipIfDN” where the “userLabel” equal to “LTEC”

- Match this value with the distname where the class=”IPADDRESSV4”

- Take LOCAL_IP=attribute “localIpAddr” under the above class

- If the IP did not deduce from above logic use this Logic for Local IP.

From class="IPADDRESSV4_R" Where distName="PLMN-PLMN/MRBTS-xxxx/TNLSVC-Y/TNL-Y/IPNO-Y/IPIF-Z/IPADDRESSV4-Y/IPADDRESSV4_R-Y where xxxx is the Node ID and y is integer and IPIF-Z is IPIF-3.

- LOCAL_IP= attribute " localIpAddrAllocated"

EX:-

LOCAL IP will be 10.33.1.90 in this case.

- From class=“LNMME”, where the distName="PLMN-PLMN/MRBTS-xxx/LNBTS-xxx/LNMME-Y where xxx is the SBTS/MRBTS ID, each Y represent an additional info

- From the same class take the Remote_IP from the attribute "ipAddrPrim"

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | MRBTS-ID /LNMME-Y | - CONNECTION_TYPE=S1-MME - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | MRBTS-ID /LNMME-Y | - LOCAL_IP= the value of " localIpAddr" in class=“IPADDRESSV4” as described above - REMOTE_IP= the value of "ipAddrPrim" in class=“ LNMME” |

## 3 S1-U

If class=”IPNO” doesn’t exist or does not have "mPlaneIpAddress" then follow given below rule for MRBTS

- This section related to MRBTS version “SBTS/MRBTS” both

- From class=“FSTSCH”, from the distName="PLMN-PLMN/MRBTS-ID where after “MRBTS-” parse the SBTS/MRBTS ID.

- In the same class take the value under the attribute “ipIfDN” where the “userLabel” equal to “LTEU”

- Match this value with the distname where the class=”IPADDRESSV4”

- Take LOCAL_IP=attribute “localIpAddr” under the above class

- If the IP did not deduce from above logic use this Logic for Local IP.

From class="IPADDRESSV4_R" Where distName="PLMN-PLMN/MRBTS-xxxx/TNLSVC-Y/TNL-Y/IPNO-Y/IPIF-Z/IPADDRESSV4-Y/IPADDRESSV4_R-Y where xxxx is the Node ID and y is integer and IPIF-Z is IPIF-1.

- LOCAL_IP= attribute " localIpAddrAllocated"

Ex:-

Local ip will be 10.33.49.90 in this case.

- From class="GTPU", from the distName="PLMN-PLMN/MRBTS-xxx/LNBTS-xxx/ where xxx is the eNodeB ID

- From the same class, each attribute "sgwIpAddress" represent one remote IP, increase the attribute of the additional info by 1 for each remote IP

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | MRBTS-ID/GTPU-Y | - CONNECTION_TYPE=S1-U - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | MRBTS-ID/GTPU-Y | - LOCAL_IP= the value of " localIpAddr" in class=“IPADDRESSV4” as described above REMOTE_IP= the value of "sgwIpAddress" in class=“ GTPU” |

## 4.Additional attributes for eNodeB or SRAN in case of 4G

| Category | Attribute | Value |
| --- | --- | --- |
| RADIO | actUeBasedAnrIntraFreqLte | Take value from class="LNBTS" as shown below |
| RADIO | actCSFBRedir | Take value from class="LNBTS" as shown below |
| RADIO | actIfHo | Take value from class="LNBTS" as shown below |
| RADIO | actSrvccToGsm | Take value from class="LNBTS" as shown below |
| RADIO | actSrvccToWcdma | Take value from class="LNBTS" as shown below |

# Additional info for SRAN

| Category | Value | Note |
| --- | --- | --- |
| Software version | SRAN ID | In the same line where class=“SBTS” take the version equal to the value after the version |

## X2 Interface

- From class=“BTSSCL”, where the distName="PLMN-PLMN/SBTS-xxx/BTSSCL-Y xxx is the eNodeB ID and y is an integer

- From the same class, LOCAL_IP equal to attribute "ipV4Address1" under the attribute "cPlaneList" Or under class=”FSTSCH” take value of “ipIfDN” find same value of “iPIfDN” in class=”IPADDRESSV4” then take value of “localIpAddr”.

- From class=“LNADJ”, where the distName="PLMN-PLMN/SBTS-eNodeB ID/LNBTS-eNodeB ID/LNADJ-X” (x is an integer), each LANDJ-X represent one additional info on the same Node and the attribute to be increment by 1 for each LANDJ-X

- From the same class take the Remote_IP from the attribute "cPlaneIpAddr"

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | SRAN-ID/LNADJ-X | - CONNECTION_TYPE=X2 - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | SRAN-ID/LNADJ-X | - LOCAL_IP= the value of "ipV4Address1" in class=“BTSSCL” under the attribute "cPlaneList" - REMOTE_IP= the value of "cPlaneIpAddr" in class=“LNADJ” |

## S1-MME

- From class= "MPLANE" where the distName="PLMN-PLMN/SBTS-xxx/MGMNT-Y/MPLANE-Y where xxx is the Node ID and Y is an integer

- LOCAL_IP= attribute "mPlaneIpAddress"
OR

- From class=”MPLANENW” where the distName="MRBTS-512087/MNL-1/MNLENT-1/MPLANENW-1" we need to take the value of attributes <p name="mPlaneIpv4AddressDN"> and search in class=”IPADDRESSV4_R”

- LOCAL_IP= <p name="localIpAddrAllocated">

- If the IP did not deduce from above logic use this Logic for Local IP.

From class="IPADDRESSV4_R" Where distName="PLMN-PLMN/MRBTS-xxxx/TNLSVC-Y/TNL-Y/IPNO-Y/IPIF-Z/IPADDRESSV4-Y/IPADDRESSV4_R-Y where xxxx is the Node ID and y is integer and IPIF-Z is IPIF-3.

- LOCAL_IP= attribute " localIpAddrAllocated"

EX:-

LOCAL IP will be 10.33.1.90 in this case.

- From class=“LNMME”, where the distName="PLMN-PLMN/SBTS-xxx/LNBTS-xxx/LNMME-Y where xxx is the eNodeB ID, each Y represent an additional info

- From the same class take the Remote_IP from the attribute "ipAddrPrim"

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | SRAN-ID /LNMME-Y | - CONNECTION_TYPE=S1-MME - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | SRAN-ID /LNMME-Y | - LOCAL_IP= the value of "mPlaneIpAddress" in class=“IPNO”/ the value of " localIpAddrAllocated " in class=“ IPADDRESSV4_R” - REMOTE_IP= the value of "ipAddrPrim" in class=“ LNMME” |

## S1-U

- From class=“BTSSCL”, where the distName="PLMN-PLMN/SBTS-xxx/BTSSCL-Y xxx is the eNodeB ID and y is an integer

- From the same class, LOCAL_IP equal attribute "ipV4Address1" under the attribute "uPlaneList"

- If the IP did not deduce from above logic use this Logic for Local IP.

From class="IPADDRESSV4_R" Where distName="PLMN-PLMN/MRBTS-xxxx/TNLSVC-Y/TNL-Y/IPNO-Y/IPIF-Z/IPADDRESSV4-Y/IPADDRESSV4_R-Y where xxxx is the Node ID and y is integer and IPIF-Z is IPIF-1.

- LOCAL_IP= attribute " localIpAddrAllocated"

Ex:-

Local ip will be 10.33.49.90 in this case.

- From class="GTPU", from the distName="PLMN-PLMN/SBTS-xxx/LNBTS-xxx/ where xxx is the eNodeB ID

- From the same class, each attribute "sgwIpAddress" represent one remote IP, increase the attribute of the additional info by 1 for each remote IP

| Category | Value | Note |
| --- | --- | --- |
| ADJNODE | SRAN-ID/GTPU-Y | - CONNECTION_TYPE=S1-U - NODE_ID= IP_DERIVED |
| ASSOCIATION // 1st association // | eNodeB-ID/GTPU-Y | - LOCAL_IP= the value of "uPlaneIpAddress" in class=“IPNO”/ the value of " localIpAddrAllocated " in class=“ IPADDRESSV4_R” - REMOTE_IP= the value of "sgwIpAddress" in class=“ GTPU” |

# 9.Additional info for LTE CELL

| Category | Attribute | Value | Note |
| --- | --- | --- | --- |
| RADIO | vswrMajorAlarm | antlId=* | Divide the value by 10. If value is 26 then it is 2.6 |
| RADIO | vswrMinorAlarm | antlId=* | Divide the value by 10. If value is 15 then it is 1.5 |
| RADIO | qRxLevMin | Take value from class="LNCEL" as shown below |  |
| RADIO | dlMimoMode | Take value from class="LNCEL" as shown below |  |
| RADIO | t300 | Take value from class="LNCEL" as shown below |  |
| RADIO | t301 | Take value from class="LNCEL" as shown below |  |
| RADIO | t304IntraLte | Take value from class="LNCEL" as shown below |  |
| RADIO | t311 | Take value from class="LNCEL" as shown below |  |
| RADIO | t320 | Take value from class="LNCEL" as shown below |  |

- For Version:- TL16,TL16A,FL16,FL16A.

1.Take the id <p name="antlId"> from class="LCELL" version="TL16" or “TL16A” or "FL16" "FL16A" distName="PLMN-PLMN/MRBTS or SBTS -XXXX/BTSSCL-Y/LCELL-ZZZ" (where X,Y,Z are integer).

2. Match the class=LCELL with LNCELL to deduce the cell id.

3. Map ANTL-id from class="ANTL" version="TL16" or “TL16A” or "FL16" "FL16A"distName="PLMN-PLMN/MRBTS-XXXX/ANTL-Y"(where X and Y is integer)

3. Take the value from  attribute

<p name="vswrMajorAlarm">xx</p> and <p name="vswrMinorAlarm">yy</p> (where x and y is integer)

- For Version-TL17, TL17A, FL17,FL17A

- Take the ID from “<managedObject class="CHANNEL" version="MNL17A_" distName="PLMN-PLMN/MRBTS or SBTS-XXX/MNL-1/MNLENT-1/CELLMAPPING-1/LCELL-Y/CHANNELGROUP-1/CHANNEL-Z”

- Take the ANTL ID from <p name="antlDN">MRBTS-902232/EQM-1/APEQM-1/RMOD-1/ANTL-5</p>

- Match the class=LCELL with LNCELL to deduce the cell id.

- Take VSWR value from ‘<managedObject class="ANTL" version="EQM17_1701_007" distName="PLMN-PLMN/MRBTS-902232/EQM-1/APEQM-1/RMOD-1/ANTL-5"’

## 9.1 Rach Root Sequence

Take the value of Rach Root Sequence under attribute <p name="rootSeqIndex"> from class <managedObject class="LNCEL".

If not present in class="LNCEL" ,then take the value under attribute <p name="rootSeqIndex"> from class <managedObject class="LNCEL_FDD" & <managedObject class="LNCEL_TDD" respectively.

## 9.2 RET Information

RET parameter will parse as below.

- From class=”LNBTS” take MRBTS for that node.

- check class=”LNCEL” for that MRBTS

- From the above step we got “LNCEL-1” so we match “LCELL-1” in distName under class= “CHANNEL”

Example:

distName="PLMN-PLMN/MRBTS-900174/MNL-1/MNLENT-1/CELLMAPPING-1/LCELL-1

under class”CHANNEL” take attribute name=”antlDN”

example:

<p name="antlDN">MRBTS-900174/EQM-1/APEQM-1/RMOD-1/ANTL-1</p>

If antlDN did not found take attribute “ressourceDN”.

- From class “RETU_R” for same MRBTS, Deduce antlDNList.

SS:-

- Replace “_R” from deduce value of step 4 and match it with deduce value of step 3.

This is how cell will map to RETU_R class.

- Below attributes will taken from class=”RETU_R as per step 7.

| Object Type | Category | Attribute | Value |
| --- | --- | --- | --- |
| LTE CELL | RET | E Tilt |  |
| LTE CELL | RET | Max E Tilt |  |
| LTE CELL | RET | Min E Tilt |  |
| LTE CELL | RET | Operational State |  |
| LTE CELL | RET | Ret Subunit iD |  |
| LTE CELL | RET | Antenna Model |  |
| LTE CELL | RET | Antenna Serial Number |  |
| LTE CELL | RET | Antenna Group |  |

- Below is attribute and dumps mapping:-

| Attribute | Parameters detail |
| --- | --- |
| E Tilt | angle |
| Max E Tilt | maxangle |
| Min E Tilt | minangle |
| Operational State | opertionalstate |
| Ret Subunit iD | Sub unit Number |
| Antenna Model | antmodel |
| Antenna Serial Number | ant serial |
| Antenna Group | antDNlist |

Ex:- E Tilt

# SIWA Parameters

Please parse following parameters for SIWA scope in add info. Where ever value is not mentioned please keep blank. Logic will be provided later.

| Object Type | Object ID | Category | Attribute | Value | Note |
| --- | --- | --- | --- | --- | --- |
| LTE_Site |  | SIWA | MME ID |  |  |
| LTE cell |  | SIWA | Band | <below table > |  |
| LTE cell |  | SIWA | RS Power | <as per below logic> |  |
| EnodeB/SRAN |  | SIWA | eNB Model No |  |  |
| EnodeB/SRAN |  | SIWA | RRU model no |  |  |
| LTE cell |  | SIWA | MIMO Configuration |  |  |
| LTE_Site |  | SIWA | Connectivity Type | IP |  |
| LTE cell |  | SIWA | No of Active users - License supported |  |  |
| EnodeB/SRAN |  | SIWA | Total Aggregate bandwidth provisioned per eNB |  |  |
| EnodeB/SRAN |  | SIWA | Active Technology on SRAN BTS | <as per below logic > |  |
| LTE cell |  | SIWA | Cell range |  |  |
| EnodeB/SRAN |  | SIWA | SRAN | “Yes” if parent node is SRAN Otherwise “NO” |  |  |  |
| EnodeB/SRAN |  | SIWA | BBU manufacturer | NSN |  |  |  |
| EnodeB/SRAN |  | SIWA | RRU manufacturer | NSN |  |  |  |
| LTE cell |  | SIWA | DL Power | Same as attribute power which is already coming in addInfo |  |  |  |
| LTE cell |  | SIWA | RRU Model No |  |  |  |  |
| EnodeB/SRAN |  | SIWA | Latitude |  |  |  |  |
| EnodeB/SRAN |  | SIWA | Longitude |  |  |  |  |
| LTE cell |  | SIWA | Circle | < same as attriute “MNC”,which is already parsing> |  |  |  |
| EnodeB/SRAN |  | SIWA | eNB Model No |  |  |  |  |
| EnodeB/SRAN |  | SIWA | BBU Model No |  |  |  |  |
| EnodeB/SRAN |  | SIVA | No of VLANs per eNB | <below logic> |  |

Band:

The value of Band for LTE cells can be extracted as below below:

| Dlearfcn |  |  |
| --- | --- | --- |
| From | To | Value |
| 0 | 599 | 2100 |
| 600 | 1199 | 1900 |
| 3450 | 3799 | 900 |
| 1200 | 1949 | 1800 |
| 2400 | 2649 | 850 |
| 2750 | 3449 | 2600 |
| 38650 | 39649 | 2300 |
| 39650 | 41589 | 2500 |

Active Technology on SRAN BTS:

If parent node is SRAN then value is supported technologies for that SRAN node.

No of VLANs per Enb : Count of VLANs of the parent node.

RS Power:  the value of RS power can be extracted as per below formula

RS Power = (pMax – dlCellPwrRed -10*LOG10(redBwMaxRbDl*12))+ dlRsBoost.

Extract the value from the attribute <p name="pMax"> under class <managedObject class="LNCEL"

So, pMax= (value extracted from <p name="pMax"> /10).

Extract the value from the attribute <p name="dlCellPwrRed"> under class <managedObject class="LNCEL" if not present in the above class the check in class <managedObject class="LNCEL_FDD" or <managedObject class="LNCEL_TDD"

So, dlCellPwrRed =(value extracted from <p name="dlCellPwrRed">/10).

Take the value for redBwMaxRbDl from the attribute <p name="redBwMaxRbDl"> and the value of dlRsBoost from the attribute <p name="dlRsBoost"> under class <managedObject class="LNCEL".

# 10.Additional New

## 10.1 LNBTS Name

To be added for SRAN or eNodeB deduced from SBTS or MRBTS Classes.

Category= LNBTS_name

Value= name under LNBTS under MRBTS or SBTS.

Attribute name=Category_Value.

| Attribute | Value | Category | Note |
| --- | --- | --- | --- |
| LNBTS_MUMFDLTEGROWELPH2IB | MUMFDLTEGROWELPH2IB | LNBTS_name |  |

## 10.2 LNBTS enbName

To be added for SRAN or eNodeB deduced from SBTS or MRBTS Classes.

Category= LNBTS_enbName

Value= enbName under LNBTS under MRBTS or SBTS.

Attribute name=Category_Value.

| Attribute | Value | Category | Note |
| --- | --- | --- | --- |
| LNBTS_enbName_MUMFDLTEDMARTKHARIO | MUMFDLTEDMARTKHARIO | LNBTS_enbName |  |

## 10.3 SBTS/MRBTS btsName

To be added for SRAN or eNodeB deduced from SBTS or MRBTS Classes.

Category= SBTS_MRBTS_btsName

Value= btsName or sbtsName under class="SBTS" or class =”MRBTS”

Attribute name=Category_Value.

| Attribute | Value | Category | Note |
| --- | --- | --- | --- |
| SBTS_MRBTS_btsName | SBTS-MUMFDLTELODHAVENEZIB_905404 | SBTS_MRBTS_btsName |  |

## 10.4 Lat/Long parsing rule

Under  <managedObject class="GNSSE_R" take value of <p name="gnssAntennaLatitude"> for latitude and <p name="gnssAntennaLongitude"> for Longitude and divide it with 10 million(10^7).

Take the corresponding value of parent node for particular MRBTS.

Example:

<p name="gnssAntennaLatitude">225797836</p>

So Latitude will be  22.5797836

<p name="gnssAntennaLongitude">729699597</p>

So Longitude will be 72.9699597

| Category | Attribute | Value |
| --- | --- | --- |
| enodeb | latitude | Value of latitude |
| enodeb | longitude | Value of longitude |

## 4G Cell to RRU Mapping

- From class=”LNBTS” take MRBTS for that node.

- check class=”LNCEL” for that MRBTS

- From the above step we got “LNCEL-1” so we match “LCELL-1” in distName under class= “CHANNEL”

Example:

distName="PLMN-PLMN/MRBTS-900174/MNL-1/MNLENT-1/CELLMAPPING-1/LCELL-1

under class”CHANNEL” take attribute name=”antlDN”

example:

<p name="antlDN">MRBTS-900174/EQM-1/APEQM-1/RMOD-1/ANTL-1</p>

If antlDN did not found take attribute “ressourceDN”.

- under class”CHANNEL” take attribute name=”antlDN”/resourceDN  till RMOD type, and match it with attribute name=”configDN” under class= ”RMOD_R”

so we match attribute like given below screenshots:

name=”antlDN

<p name="antlDN">MRBTS-900174/EQM-1/APEQM-1/RMOD-1

name=”configDN”

i.e. <p name="configDN">MRBTS-900714/EQM-1/APEQM-1/RMOD-1</p>

For cell to RRU mapping from class="RMOD_R”, parse the value of name="productName" in  RU Model and name="serialNumber" in Serial Number of additionalinfo.

| Object ID | Object Type | Attribute | Value | Category |
| --- | --- | --- | --- | --- |
| CELLID | LTECell | Serial Number | Value of SerialNumber: Value of SerialNumber | RADIO |
| CELLID | LTECell | RU Model | Value of productName: Value of productName | RADIO |

## Physical Site ID

- Get the value of “siteTemplateName” attribute under class=”LNBTS” and map with the node.

So Physical Site ID will be : KLPACL02213IM11

| elementId | elementType | attribute | Value | category | note |
| --- | --- | --- | --- | --- | --- |
| MRBTS-123 | eNodeB/SRAN | Physical Site Id | Value of attribute “siteTemplateName” | Physical Site |  |