# Huawei RAN Connector FRS (Functional Requirement Specifications) Auto-Import connector

Connector Name:

Huawei_RAN_Ooredoo_Tunisia

Change History

| Version | Change Description | Author | Date |
| --- | --- | --- | --- |
| 0.1 | First draft | Indrasan Yadav | 05-05-2025 |
| 0.2 | Added a rule to parse site id in section 1.3 for all node type | Indrasan Yadav | 19-08-2025 |
| 0.3 | Added a rule where duplicate name in antenna under section 1.2 | Indrasan Yadav | 23-09-2025 |
| 0.4 | Added a rule to parse supported technologies for SRAN under section 1 highlighted with color green | Indrasan Yadav | 19-12-2025 |
| 0.5 | Added a rule to parse site id for micro site under section 1.3.7 (Applicable for all node type but currently in SRAN only now) marked with green color | Indrasan Yadav | 24-12-2025 |
| 0.6 | Update the sheet “Node_Matching” which to be consider by AI DEV to parse node type based on “NEType” under section 1 highlighted with green color. | Indrasan Yadav | 20-01-2026 |
| 0.7 | Update the static sheet “HW_Matching” which to be use for RU Index parsing under section 1.1.2 highlighted with color green. | Indrasan Yadav | 21-01-2026 |

Table of Contents

Introduction	3

1	RAN Nodes	5

1.1	RAN Physical configuration	8

1.1.1	Cabinet	8

1.1.2	Shelf	9

1.1.3	Boards	11

1.2 Antenna…………………………………………………………………………………………………………………………13

# Introduction

- This FRS is intended for Huawei RAN Nodes extraction.

- It describes how to parse the nodes details and physical configuration.

- The dump files provided are of xml file per node.

- The folder location is to be précised later.

- The files format is as follow:

# RAN Nodes

We will consider the static sheet to define node type based on the value of attribute “NEType” in dump.

Static file name: “Node_Matching”.

- The following attributes are defined at node level:

| Attribute | Importance | Comment |
| --- | --- | --- |
| NE External Code | [Mandatory] | - NE ID |
| NE ID | [Mandatory] | - =NE name. |
| NE Name | [Mandatory] | NE Name will be extracted from attribute “NEName=”” in individual dump. |
| Site id | [Mandatory] | - Explain later as per requirement |
| Site Name | [Mandatory] | - = Site id |
| Type | [Mandatory] | To be deduced from node matching file |
| Model | [Mandatory] | - The value of “NEType=” in node dump |
| Manufacturer | [Mandatory] | - = Huawei |
| IP + subnet mask | [Optional] | - =NULL |
| Technology Supported | [Optional] | - First, the Technology supported will be deduced as follow through matching the “NEType” within the below table: NEType; Technology Supported. GSMBTS; 2G. BSC6900GSM; 2G. BSC6900UMTS; 3G. BSC6900GU; 2G/3G. BSC6910GSM; 2G. BSC6910UMTS; 3G. BSC6910GU; 2G/3G. NodeB; 3G. MBTS(GL); 2G/4G. MBTS(GU); 2G/3G. MBTS(UL); 3G/4G. MBTS(GUL); 2G/3G/4G. eNodeB; 4G. BTS; 2G. MBTSGU; 2G/3G. BSC6000BTS; 2G. BTS3900/BTS5900/BTS59005G; 2G/3G/4G. Rule: In the physical dump of SRAN, under the class <TABLE attrname="HostVer">, the following attribute values should be checked to determine the corresponding technology: - <ROW HostVerType="GBTSFunctionHOSTVER" with sDesc="GRAT SOFTWARE VERSION" → 2G OR <ROW HostVerType="GBTSApplicationVER" 2G - <ROW HostVerType="NodeBFunctionHOSTVER" with sDesc="URAT SOFTWARE VERSION" → 3G OR <ROW HostVerType="NodeBApplicationVER" 3G - <ROW HostVerType="eNodeBFunctionHOSTVER" with sDesc="LRAT SOFTWARE VERSION" → 4G OR <ROW HostVerType="eNodeBApplicationVER"4G - <ROW HostVerType="gNodeBFunctionHOSTVER" with sDesc="NRAT SOFTWARE VERSION" → 5G OR <ROW HostVerType="gNodeBApplicationVER"5G |
| Parent BSC | [Mandatory] | - The node (BTS) with SiteName= which is available in that BSC dump under attribute <TABLE attrname="BTS"> then that BSC will be parent BSC for all available node (BTS). Note: same rule will be applied also for if parent will be SRAN Controller. |
| Parent RNC | [Mandatory] | - The node (NodeB) with NodeBName= which is available in that RNC dump under attribute <TABLE attrname="UCELL"> then that RNC will be parent RNC for all available node (NodeB). Note: same rule will be applied also for if parent will be SRAN Controller. |
| Software version | [Optional] | - =Null |
| Additional Information | [Optional] | - =Null |

## 1.1 RAN Physical configuration

### 1.1.1 Cabinet

- Each XML file of each node contains information about cabinets. From this file we will be taking the data for the cabinet.

- each type has one attribute <TABLE attrname="Cabinet"> for the cabinet’s info.

- The cabinet attributes are extracted as follow:

| Attribute | Importance | Comments |
| --- | --- | --- |
| NE External code | [Mandatory] | - Same as NE ID |
| Cabinet index | [Mandatory] | - The value of “CabinetNo” under attribute <TABLE attrname="Cabinet"> - Eg: CabinetNo="0" Cabinet index=0 - If “CabinetNo” is empty, then ignore the line. - Sometimes we can get more than one cabinet. Parse all of them. Note: If any node which contains blank physical (cabinet, shelf, board) information, then in that case only node will be parse & physical data (cabinet, shelf & boards & antenna) will not be parse. |
| Cabinet id | [Mandatory] | - = NE External code _cabinet index - E.g.: BARF011_0 (For above example) |
| Model | [Mandatory] | - The value of “RackType” attribute - Eg: RackType="VIRTUAL" VIRTUAL |
| Part Number | [optional] | - =Value of “BOMCode” attribute - Eg: BOMCode="01071087” |
| Date Of Manufacturer | [optional] | - =Value of “DateOfManufacture” attribute - Eg: DateOfManufacture="2011-03-10" DateOfManufacture=2011-03-10 |
| Description | [optional] | - =Value of “ManufacturerData” attribute - If empty = null |
| Serial Number | [optional] | - =Value of “SerialNumber” attribute - E.g.: SerialNumber="21010710876TB3000615 |
| Manufacturer | [Mandatory] | - =Huawei |
| Power Consumption | [optional] | - =null |
| Additional Information | [optional] | - =null |
| Dimensions and Weight | [optional] | - =null |

### 1.1.2 Shelf

- Each row under attribute <TABLE attrname="Subrack"> shows shelf information.

- The below table describes how to parse the shelf attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Cabinet Id | [Mandatory] | - = NE External code _cabinet index - E.g.: BARF011_0 |
| Shelf Id | [Mandatory] | - Concatenation between NEName, “_” and CabinetNo=, “_” and SubrackNo in related row under attribute <TABLE attrname="Subrack"> |
| Shelf index | [Mandatory] | - The value of “SubrackNo=” under attribute <TABLE attrname="Subrack"> - E.g.: SubrackNo="0" Shelf index=0 - If “SubrackNo” is empty, then ignore the line. - Sometimes we can get more than one subrack. Parse all of them. |
| RU Index | [Mandatory] | - To be followed static sheet “HW_MatchingFile”. |
| Part Number | [Optional] | - =Value of “BOMCode” attribute - Eg: BOMCode="01071087” |
| Model | [Mandatory] | - =”FrameType” attribute, e.g.: MPS or EPS. |
| Date Of Manufacture | [Optional] | - =Value of “DateOfManufacture” attribute Eg: DateOfManufacture="2011-03-10" DateOfManufacture=2011-03-10 |
| Issue Number | [Optional] | - Take the Value of “IssueNumber=” under same attribute <TABLE attrname="Subrack"> for related shelf. - =Value of “IssueNumber” attribute Eg: IssueNumber="00” |
| Description | [Optional] | - =Value of “ManufacturerData” attribute If empty = null |
| Serial Number | [Optional] | - =Value of “SerialNumber” attribute E.g.: SerialNumber="21010710876TB3000615 |
| Power Consumption | [Optional] | - Null |
| Additional Information | [Optional] | - Null |
| Dimensions and Weight | [Optional] | - Null |

### 1.1.3 Boards (Need to cross check again for slot no. & index on slot once full dump received & SFP will be parse as board or transceiver?)

- Each row under attribute <TABLE attrname="Board"> shows boards information.

- The below table describes how to parse the boards attributes:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Shelf Id | [Mandatory] | - Concatenation between NEName, “_”, CabinetNo,” _” and SubrackNo |
| Shelf Index | [Mandatory] | - The value of “SubrackNo=” under attribute <TABLE attrname="Subrack"> - E.g.: SubrackNo="0" Shelf index=0 - If “SubrackNo” is empty, then ignore the line. - Sometimes we can get more than one cabinet. Parse all of them. |
| Index on slot | [Mandatory] | =0 By default |
| Board Id | [Mandatory] | - Concatenation between NEName, “_”, “CabinetNo”, “_”, SubrackNo,” _”, “SlotNo” and Index on slot. - E.g.: BARF011_0_0_6_0 |
| Slot Index | [Mandatory] | - =Value of “SlotNo” attribute in each Row. |
| Board Type | [Mandatory] | Take the value of “BoardName=” under attribute “<TABLE attrname="Board">” Note: The existing rule can be retained with the following data also: - We need to add BoardName="FModule" in the ‘npsboards’ section under the <TABLE attrname="Board"> class. - The “FModule” board will be parse as described below, it should be parsed as boards with the required cabinet, shelf, slot, and board index as specified in the dump. Sample Case: The snippet below illustrates an example: In this case, the “FModule” boards (count: 6) are present under cabinet-0 and are distributed across shelf-0, shelf-60, shelf-61 & shelf-62 occupying slots 0 and 6. All instances of “FModule” should be parsed in the XML as per the dump details. |
| Part Number | [Optional] | - =Value of “BOMCode” attribute E.g.: BOMCode="03051122" Part Number= 03051122 |
| Description | [Optional] | - =Value of “ManufacturerData” attribute E.g.: ManufacturerData="PARC WP11OIUa 1-port channelized Optical Interface Unit REV:a" then description will be = "PARC WP11OIUa 1-port channelized Optical Interface Unit REV:a" |
| Date Of Manufacture | [Optional] | - =Value of “DateOfManufacture” attribute Eg: DateOfManufacture="2011-05-07" DateOfManufacture=2011-05-07 |
| Serial number | [Optional] | - =Value of “SerialNumber” attribute E.g.: SerialNumber="210305112210B5000522" SerialNumber=210305112210B5000522 |
| Issue Number | [Optional] | - =Value of “IssueNumber” attribute Eg: IssueNumber="00" Issue Number=00 |
| Power Consumption | [Optional] | - =null |
| Additional Information | [Optional] | - =null |
| Dimensions and Weight | [Optional] | - =null |

## 1.2 Antenna Parsing

Antenna details will be parse under table of “radioantenna” in XML.

Antenna Parsing rule described below:

| Attribute | Importance | Comments |
| --- | --- | --- |
| Antenna Id/Object ID | [Mandatory] | - =Concatenate of “NEName=” and the value of “InventoryUnitId” (“InventoryUnitId” will be available under attribute <TABLE attrname="Antenna"> |
| Name | [Mandatory] | - =Attribute “UnitPosition” take the value before the “,” indicator. - Example: UnitPosition="AntennaDeviceName=RET_UMTS-A,AntennaDeviceNo=1" Name = RIY2657-P1-HUB_SRAN-FON_1,RET_UMTS-A. Note: 1- In cases where duplicate names are found, each subsequent duplicate must be made unique after the first data is parsed. 2- To achieve this, a special character \1 should be appended to the duplicate name, with the number incremented by +1 for each occurrence. Example: As per given rule 2nd one should be BAR1176S1TLL1821_HWAIM_P_02\1. |
| RadioantennaType | [Mandatory] | - =attribute “AntennaDeviceType=” - Example: AntennaDeviceType="RCU136" type =”RCU136” |
| VersionID | [Optional] | - Value of HardwareVersion, e.g if HardwareVersion="RF11CLPT_VER.D" then VersionId is RF11CLPT_VER.D |
| Band | [Optional] | - =attribute “ManufacturerData” |
| Sector | [Optional] |  |

Additional information: Serial number will be parse under addinfo in XML under attribute “Value”.

| Attribute | Importance | Comments |
| --- | --- | --- |
| SerialNumber_<serialnumber> | [Optional] | - =Value of “SerialNumberEx” attribute - Eg:SerialNumber="030LPM4MB5618639" SerialNumber=030LPM4MB5618639 |
| SerialNumber_<serialnumber> |  | - Value of “SerialNumber” attribute Eg:SerialNumber="030LPM4MB5618639" SerialNumber=030LPM4MB5618639 |

Note:

Current Implementation:

The system currently parses SerialNumberEx as the primary source for the antenna serial number.

Issue:

SerialNumberEx is often found to be empty in the data dump, which causes it to come blank for some Antenna nodes in the NEP report.

1. Objective

Ericsson antennas are connected to Huawei RAN. To correctly identify and import serial numbers (SN) without duplications in this unique scenario, the active assets include:

- Ericsson antennas connected to Huawei RAN.

- Serial Numbers are in either SerialNumberEx or AntennaSerialNumber attribute whereby SerialNumberEx is for Huawei and AntennaSerialNumber is for Ericsson antenna

3. Data Import Logic

To correctly import the active assets without duplications, we propose the following logic:

- Step 1: Check Column U (SerialNumberEx)) for SN

  - If Column U has a value, use it as the SN for Huawei antennas and proceed with the import. If it is empty Check AntennaSerialNumber

  - The value from Column U is imported only if it is unique

- Step 2: Check AntennaSerialNumber for Ericsson Antennas

  - If it is empty, use the SN from SerialNumberEx and proceed with the import, ensuring no duplicates

## 1.3 Site Id Parsing Rule

The Site ID will be parsed from the node name according to the rules defined for each node type.

Priority Order:

- Priority 1: Site Matching Sheet (to be provided separately).

- Priority 2: Naming Convention Rules (as described below, based on node type).

### 1.3.1 BTS

Site ID Parsing from BTS Node Name

The Site ID will be derived from the BTS node name based on the following rules:

Case 1: If the node name contains the special character “_” (underscore), the Site ID will be the value appearing after the last underscore.

Case 2: If the node name does not contain an underscore, the Site ID will be the same as the node name.

Case 3: If any other case occurs, the site will be ignored and parsed as the default site.

Examples:

Example 1:

Node Name: N_TIERET_TAT1065

Rule Applied: Case 1

Site ID = TAT1065

Example 2:

Node Name: SLT1079

Rule Applied: Case 2

Site ID = SLT1079

### 1.3.2 NodeB

Site ID Parsing from NodeB Node Name

The Site ID will be derived from the NodeB node name based on the following rules:

- Case 1: Replace the 4th numeric digit with 1 except 2 (after the first three alphabets). The remaining characters stay unchanged.

- Case 2: If 4th numeric digit is 2 (after the first three alphabets) and after 2 next 3 numeric digit is >600 then to be subtract 600 from last 3 digit and find value will be site id.

- Case 3: If above Case 1 & Case 2 is not applicable, the Site ID will be parsed as the default site.

Example:

Example 1: For Case 1

Node Name: NAB4111

Rule Applied: Case 1 (4 → 1)

Site ID = NAB1111

### 1.3.3 eNodeB

Site ID Parsing from eNodeB Node Name

The Site ID will be derived from the eNodeB node name based on the following rules:

- Case 1: Replace the 4th alphabetic character (commonly F) with 1.

- Case 2: If Case 1 is not applicable, the Site ID will be parsed as the default site.

Example:

Example 1:

- Node Name: BARF026

- Rule Applied: Case 1 (F → 1)

- Site ID = BAR1026

### 1.3.4 SRAN

Note: For Role2G, Role3G, and Role4, the Site ID will remain the same as the parent SRAN Node ID.

Site ID Parsing from SRAN Node Name

The Site ID will be derived from the SRAN node name based on the following rules:

- Case 1: If the node name contains “_” (underscore) or “-” (hyphen), take the value after the last special character and replace the 4th digit with 1.

- Case 2: If the node name does not contain any special characters, replace the 4th digit with 1 directly.

- Case 3: In all cases, if the value extracted after the special character has the 4th digit as 2 and the next three digits are less than 600, the extracted value itself should be considered as the Site ID.

- Case 4: If any other case occurs, the site will be ignored and parsed as the default site.

Examples:

Example 1:

Node Name: N_BEJ4012

Rule Applied: Case 1 (4 → 1)

Site ID = BEJ1012

Example 2:

Node Name: BAR4015

Rule Applied: Case 2 (4 → 1)

Site ID = BAR1015

Example 3:

Node Name: MBTS-Ste_SOPAT_MAH2010

Rule Applied: Case 3 (Extracted value after special character is MAH2010 where 4th digit is 2 & next three digits are less than 600)

Site ID = MAH2010

### 1.3.5 BSC

Site Id will be same as node name of BSC.

### 1.3.6 RNC

Site Id will be same as node name of RNC.

### 1.3.7 Site ID for micro site

#### 1.3.7.1 Case 1: 4th digit is 2 & next three digits are ‘=’ or ‘>’ 600.

If extracted value from Node name, the 4th character is 2 & after 2 next three digit is = or > 600 then subtract 600 from last three digit. (It will be applicable for all node type but currently this condition is finding in SRAN only)

Example: For Case 1

Node Name: SFX2612

Rule Applied: Case 3 (4th is → 2 and next three digit is > 600 then subtract 600 from last three digit)

Site ID = SFX2012

#### 1.3.7.2 Case 2: 4th digit is 2 and next three digit are ‘=’ or ‘>’ 500

If extracted value from Node name before & after special character ‘_’ or ‘– ‘, the 4th character is 2 & after 2 next three digit is = or > 500 then subtract 50 from last three digit. (It will be applicable for only NodeB type)