# Enclosure:

- A new network element can be added to the site.

- The Enclosure has the following attributes:

  - Site

  - Name

  - Object ID: unique

  - Status: standard NEP status

  - Additional information: same as other elements

  - Model Type: Enum - Dome, Inline, or Wall-mount.

  - Material: String (e.g. High-density polyethylene (HDPE), Metal, etc.)

  - IP Rating: String (e.g. Resistance to water/dust (e.g., IP68 for underground).)

  - Max Tray Capacity: Integer (Total number of trays the enclosure can hold.)

  - Entry Ports: Integer (Number of holes for cables to enter (Alvéoles/Ducts).)

  - Mounting Type: Enum - Aerial (Pole), Manhole (Vault), or Pedestal.

- Import CSV and XML: Panel, report

# Splice Module: Update the existing model

- The splice module can be linked to the enclosure

- The following attributes to be added:

  - Status: standard NEP status (make sure it’s available)

  - Additional information: same as other elements (make sure is available)

  - Parent Enclosure: Links the tray to its specific physical box.

  - Sequence Number: Integer - The position of the tray in the stack (Tray 1, Tray 2, etc.).

  - Capacity: Integer - Number of splice "slots" available (usually 12, 24, or 48).

  - Splice Type: Enum Fusion (heat) or Mechanical.

  - Fiber Type: Enum - Single-mode (SM) or Multi-mode (MM).

  - Occupied Slots: Integer - Current number of splices already performed.

  - Color Code: String - Often the tray itself is color-coded to match a specific buffer tube.

- Consider the new parameters in import, report and panel

- Enclosure → Contains → Splice Trays → Contain → Splices (the actual joins).

# Ability to add additional information & status

Add the ability to add additional information & status to:

- ODF Cabinet

- ODF Magazine

- ODF Port

- MDF Cabinet

- MDF Magazine

- MDF Port

# Patch Cord (new entity):

- A new network element can be added to the site.

- The Patch Cord has the following attributes:

  - Site

  - Name

  - Object ID: unique

  - Status: standard NEP status

  - Additional information: same as other elements

  - Connector_Type_A: Enum: LC, SC, ST, FC, etc.

  - Connector_Type_B: Enum: LC, SC, ST, FC, etc.

  - Fiber_Type: EnumOS2 (Singlemode), OM3/OM4 (Multimode)

  - Length: Float Length in meters (e.g., 1m, 2m, 3m, 5m).

  - Color: String - Standard coding (e.g., Yellow for SM, Aqua for OM3).

  - Polarity: Enum: A-to-A or A-to-B (for duplex cords).

- Consider the new parameters in import, report and panel

- The patch cord will be used in OSP connections (same as ODF port, MDF port, Strand, Node Interface, etc.)

# OSP Pathway Inner duct

Check current data mode to agree on the implementation